SCIPI Data Pull

Unplug wet connect or on SCIPI

Plug in test cable

Plug in USB connector (behind 5/8 inch SAE port)

Connect to serial port on laptop

With hyperterm using 9600 N 1 N

Provide 18vdc to power pins on test cable

SCIPI should come up with RDY

Pwr On to turn on device

Help all for low level commands

Cam on to turn on camera

Plug USB into computer.

When recognized look for files in DCIM directory