- Power

  - 36 to 12 volts at 8amps (100+ watts)  for strobes; with a switched enable line

  - 36 to 8.4 volts at 10 watts for the camera

  - 36 to 5 volts for the computer ~ 10 watts?

  - 5 volts for the lasers – 1 watt; switchable

- Comms

  - 10/100 ethernet between camera computer and MVC

  - 10/100/1000 ethernet out of camera computer, through pressure can to a flying lead accessible for high speed download deck cable

  - USB between the camera and the camera computer

    - Access camera from off-board to make setting changes

    - Pull image files off camera and store on a local hard-drive

    - Trigger camera when MVC wants it to fire

  - RS-232 Serial between GPS adaptor on camera and camera computer (for time stamp below, though this could be done over USB)

- Switching

  - Trigger camera from software command from MVC

    - Either a switch closure

    - or direct software command over USB

  - Turn strobes on/off

  - Turn lasers on/off

- Data

  - Embed AUV time into each image

    - Use RS-232 input GPS adaptor on camera

    - Direct software through USB when trigger command is sent

  - Measure temp and humidity inside housing and log data and send an alarm when a setpoint is reached

- PV penetrations

  - Bus cable: MIN K 10L FCR/BCR for bus cable to MVC

  - Strobe: one or two, two pin connectors MCBH(RA) 6F (7/16 dia.)

  - Lasers: one or two MCBH(RA) 2F (7/16 dia.)

  - Fast data offload DBH8F (7/16)