# Port Assignments for Dec 1, 2010 BRS Deployment

## System Controller

### Port 0 – User I/F, RS-232 at 38400 baud

### Port 1 – Modbus, RS-485 at 57600 baud

### Port 2 – Modbus, RS-485 at 57600 baud

| Chamber controller | Bus | ID | Stir bit/conn | Inject bit/conn | Flush bit/conn | Extract bit/conn | Optrode serPort/conn | Connector J3 (serial port 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 2 | 1 | 2 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 3 | 1 | 3 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 4 | 1 | 4 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 | Ambient Optrode |
| 5 | 2 | 5 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 6 | 2 | 6 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 7 | 2 | 7 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 |  |
| 8 | 2 | 8 | 0 / J4 | 2 / J5 | 4 / J6 | 6 / J7 | 1 / J2 | Infrared comm |

## Chamber Controller 1

Bus 1, ID 1, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 2

Bus 1, ID 2, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 3

Bus 1, ID 3, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 4

Bus 1, ID 4, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Chamber Optrode on serial port 1, connector J2

Ambient Optrode on serial port 2, connector J3

## Chamber Controller 5

Bus 2, ID 5, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 6

Bus 2, ID 6, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 7

Bus 2, ID 7, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

## Chamber Controller 8

Bus 2, ID 8, RS-485 at 57600 baud

Stir on bit 0, connector J4

Inject on bit 2, connector J5

Flush on bit 4, connector J6

Extract on bit 6, connector J7

Optrode on serial port 1, connector J2

Infrared User I/F on serial port 2, connector J3