# Configuring the Benthos Acoustic Modem for BEDS

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.0
Date: 2/6/2013

This document describes how to configure the acoustic modem that is deployed with BEDS (i.e., the bottom-side modem, not the top-side modem).

The Benthos Acoustic modem is configured via its S-Registers.  This modem is Hayes compatible.  This means that in order to get into command mode, you need to type “+++” with at least a one-second pause before and after this string.  At that point, you can talk directly to the modem, which will prompt with “>”.  You can set the S Registers with, for example, ATS18=1.  You can get the state of all S Registers with ATS?

This is a list of all the S Registers, with the state as we need it for the BEDS deployed side.  When you get done setting the registers, save them with AT&W

| Register | Name | Value | Comments |
| --- | --- | --- | --- |
| S0 | Firmware Version |  | Not settable by user |
| S1 | Doppler Tracking | 0 | Bottom side is stationary. Not needed if top is also. |
| S2 | Packet Retry | 0 |  |
| S3 | Serial Baud Rate | 2 | 9600 baud RS-232 no echo |
| S4 | Acoustic Bit Rate | 5 | 800 baud for first deployment. We may adjust later |
| S6 | Transmit Power Level | 8 | Max for deployment. Lower for lab testing. |
| S7 | Response Timeout | 15 | 7.5 seconds. Good for slant range up to 3000m |
| S8 | Forwarding Delay | 10 | 0.5 seconds |
| S10 | Idle Time | 36 | 180 seconds idle to go to sleep |
| S11 | RS-232 Flow Control | 0 | No handshaking (may need RTS/CTS?) |
| S12 | Test Message Length |  | Don’t care |
| S13 | Verbose Level | 1 | Data and minimal diagnostics |
| S14 | Remote Addr | 0 | Gets set when topside connects |
| S15 | Operating Mode | 1 | Go online when powered |
| S16 | Error Display | 0 | Display all packets, even if error |
| S17 | Transmit Wakeup | 0 | Don’t need to send wakeup to top side |
| S18 | Modem Address | 1 | Set unique address, starting at 1. 0 is for top side |