# BEDS Operating Variables

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.0
Date: 8/28/2012

This is a list of variables that affect the operation of BEDS.  The intent is that these variables can be set by either a configuration file (BEDS.CFG) or via a user interface command.

| Variable Name | Default | Use |
| --- | --- | --- |
| EvtFreq | 50 | FIFO rate during event, = freq of INU records during event |
| WatchFreq | 10 | FIFO rate durint watch mode |
| LowBattNoSample |  | Low battery voltage at which we stop getting INU data |
| LowBattOff |  | Low battery voltage at which we turn off entire system |
| ModemPeriod | 3600 | Period (secs) for turning on acoustic modem |
| ModemOnTime | 60 | Period we leave modem on |
| AccelRange | 16 | Accelerometer range in g’s |
| GyroRange | 2000 | Gyro range in degrees/sec |
| DigitalFilterBW | 42 | Bandwidth of digital filter on MPU6000. Allowable values are 5, 10, 20, 42, 98, 188 or 256 Hz. Will set to next lower value. |
| NoMotionThresh |  | MPU no motion threshold |
| NoMotionTime |  | MPU no motion time |
| BEDSLogLevel |  | Log level for the BEDS software |
| InvLogLevel |  | Log level for the Invensense stack logging routines |