# BEDS Operating Variables

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.2
Date: 9/6/2012

This is a list of variables that affect the operation of BEDS.  The intent is that these variables can be set by either a configuration file (BEDS.CFG) or via a user interface command.

| Variable Name | Units | Default | Type | Use |
| --- | --- | --- | --- | --- |
| EvtFreq | Hz | 50 | I | FIFO rate during event, = freq of INU records during event |
| WatchFreq | Hz | 10 | I | FIFO rate during watch mode |
| LowBattNoSample | Volts | 6.5 | F | Low battery voltage at which we stop getting INU data |
| LowBattOff | Volts | 0.0 | F | Low battery voltage at which we turn off entire system |
| ModemPeriod | Secs | 3600 | I | Period (secs) for turning on acoustic modem |
| ModemOnOffset | Secs | 0 | I | Note 1 |
| ModemOnTime | Secs | 60 | I | Period we leave modem on |
| AccelRange | g | 16 | I | Accelerometer range in g’s |
| GyroRange | Deg/s | 2000 | I | Gyro range in degrees/sec |
| DigitalFilterBW | Hz | 42 | I | Bandwidth of digital filter on MPU6000. Allowable values are 5, 10, 20, 42, 98, 188 or 256 Hz. Will set to next lower value. |
| Timeout | Secs | 60 | I | Timeout on user I/F, either through modem or debug |
| MaxEvent | Secs | 7200 | I | Max length of recorded event, seconds |
| StartTime | String | Immed | S | Time/date to start running. Note 2. |
| NoMotionThresh | ? |  | I | MPU no motion threshold |
| NoMotionTime |  |  | I | MPU no motion time |
| BEDSLogLevel |  |  | S | Log level for the BEDS software. Note 3 |
| InvLogLevel |  |  | S | Log level for the Invensense logging routines. Note 3 |

Type Field consists of one or more of the following:
I – Integer (32 bit)
F – Float (32 bit)
S – String variable (32 chars max)
P – Protected.  Cannot change after system initialization

Note 1 – Normally, we will power the modem on an even boundary of ModemPeriod.  E.g., for ModemPeriod=3600, we will start on the hour.  ModemOnOffset allows you to offset that convention by a given number of seconds.  In the above example, if ModemOnOffset is 300 seconds, the modem will be powered starting at 5 minutes after the hour.

Note 2 – String to specify the time to start running.  If none or empty string, system starts immediately.  Otherwise, specify either a number, signifying delay after power-up in seconds, or a string of the form mm/dd/yyThh:mm:ss.  The system will simply go to sleep until the specified time.  This is useful to ensure that you don’t record the motion during deployment as an ‘event’.  Note the ‘T’ in the date/time specifier – this is to avoid a blank space, which may cause problems.

Note 3 – Permissible logging levels are: DEBUG, INFO, WARN, ERROR, NONE, SILENT (same as NONE)