# BEDS Operating Variables

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.3
Date: 1/23/2013

This is a list of the BEDS operating variables.  All variables on this list can be changed in two ways:

- Via the user interface.  The following commands are equivalent.  Commands are case-insensitive (capitalization doesn’t matter).
accelRange  =  16
var accelRange 16

- At startup via the BEDS.CFG configuration file.  Only the name = value form works:
accelRange = 16

| Variable Name | Units | Default | Type | Use |
| --- | --- | --- | --- | --- |
| PlatformID |  | 0 | I | Platform ID that is put into data files |
| EvtFreq | Hz | 50 | I | Sample rate during event, = freq of INU records during event |
| WatchFreq | Hz | 10 | I | Sample rate during watch mode |
| ModemPeriod | Secs | 3600 | I | Period (secs) for turning on acoustic modem |
| ModemOffset | Secs | 0 | I | See note (2) |
| ModemOnTime | Secs | 60 | I | Period we leave modem on, seconds |
| ModemWarmup | Secs | 10 | I | Time after modem powerup to wait for signon message, before watching for actual characters from topside |
| ModemWait | Secs | 5 | I | Time to wait before turning off modem, to allow all serial traffic to reach the topside modem |
| AccelRange | g | 16 | I | Accelerometer range in g’s. Valid values: 2, 4, 8, or 16 |
| GyroRange | deg/s | 2000 | I | Gyro range in deg/sec. Valid values: 250, 500, 1000, 2000 |
| DigitalFilterBW | Hz | 42 | I | Bandwidth of digital filter on MPU6000. Allowable values are 5, 10, 20, 42, 98, 188 or 256 Hz. Will set to next lower value. |
| Timeout | Secs | 60 | I | Timeout on user I/F, seconds |
| MaxEvent | Secs | 7200 | I | Max length of recorded event, seconds |
| EventPressureSecs | Secs | 1 | I | Period for sampling pressure during an event, seconds |
| SysRecSecs | Secs | 600 | I | Period for sampling system parameters during watch cycle |
| NoMotionThresh | Deg/s | 2.0 | F | MPU no motion threshold in gyro degrees/sec |
| NoMotionTime | Secs | 2.0 | F | MPU no motion time, seconds |
| SensorWarmup | Secs | 5 | I | Power-on warmup before sampling system sensors, secs |
| ModemBaud | Bits/s | 9600 | I | Baud rate to talk to acoustic modem |
| DebugBaud | Bits/s | 9600 | I | Baud rate to talk to aux serial port (debug port) |
| logToConsole |  | 0 | I | See note (4) |
| LowBattNoSample | Volts | 6.5 | F | Low battery voltage at which we stop getting INU data (1) |
| LowBattOff | Volts | 0.0 | F | Low battery voltage at which we turn off entire system (1) |

Type Field consists of one or more of the following:
I – Integer (32 bit)
F – Float (32 bit)
S – String variable (32 chars max)

### Notes

(1) Currently not used.

(2) – Normally, we will power the modem on an even boundary of ModemPeriod.  E.g., for ModemPeriod=3600, we will start on the hour.  ModemOnOffset allows you to offset that convention by a given number of seconds.  In the above example, if ModemOnOffset is 300 seconds, the modem will be powered starting at 5 minutes after the hour.

(3) – String to specify the time to start running.  If none or empty string, system starts immediately.  Otherwise, specify either a number, optionally followed by s/m/h/d, signifying delay after power-up in seconds/minutes/hours/days, or a string of the form mm/dd/yyThh:mm:ss.  The system will simply go to sleep until the specified time.  This is useful to ensure that you don’t record the motion during deployment as an ‘event’.  Note the ‘T’ in the date/time specifier – this is to avoid a blank space, which may cause problems.

(4) Normally, significant events are logged to the system log file SYSLOG.TXT.  This variable will also output those messages to one of the serial ports, as follows:
  0 – Normal operation, don’t echo to serial port
  1 – Also echo to the main system console serial port
  2 – Echo to the aux (debug) serial port