# BEDS Operating Variables

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.5
Date: 10/9/2013

This is a list of the BEDS operating variables.  All variables on this list can be changed in two ways:

- Via the user interface.  The following commands are equivalent.  Commands are case-insensitive (capitalization doesn’t matter), but you must have a space before and after the = sign.
accelRange  =  16
var accelRange 16

- At startup via the BEDS.CFG configuration file.  Only the name = value form works:
accelRange = 16

| Variable Name | Units | Default | Type | Use |
| --- | --- | --- | --- | --- |
| PlatformID |  | 0 | I | Platform ID that is put into data files |
| DeploymentID |  | 0 | I | Unique ID for deployment, starting at 0 |
| ModemAddr |  | 1 | I | Address to initialize modem with (S18) Note this may be different from PlatformID |
| EvtFreq | Hz | 50 | I | Sample rate during event, = freq of INU records during event |
| WatchFreq | Hz | 10 | I | Sample rate during watch mode |
| ModemPeriod | Secs | 3600 | I | Period (secs) for turning on acoustic modem |
| DeployModemPeriod | Secs | 600 | I | Modem period during deployment mode |
| DeployModeSecs | Secs | 86400 | I | Amount of time to stay in deployment mode |
| ModemOffset | Secs | 0 | I | See note (2) |
| ModemOnTime | Secs | 60 | I | Period we leave modem on, seconds |
| ModemWarmup | Secs | 10 | I | Time after modem powerup to wait for signon message, before watching for actual characters from topside |
| ModemWait | Secs | 5 | I | Time to wait before turning off modem, to allow all serial traffic to reach the topside modem |
| ModemPower | Level | 6 | I | S6 setting (Tx power) for modem |
| AccelRange | g | 16 | I | Accelerometer range in g’s. Valid values: 2, 4, 8, or 16 |
| GyroRange | deg/s | 2000 | I | Gyro range in deg/sec. Valid values: 250, 500, 1000, 2000 |
| DigitalFilterBW | Hz | 42 | I | Bandwidth of digital filter on MPU6000. Allowable values are 5, 10, 20, 42, 98, 188 or 256 Hz. Will set to next lower value. |
| Timeout | Secs | 120 | I | Timeout on user I/F, seconds |
| MaxEvent | Secs | 7200 | I | Max length of recorded event, seconds |
| EventPressureSecs | Secs | 1 | I | Period for sampling pressure during an event, seconds |
| SysRecSecs | Secs | 600 | I | Period for sampling system parameters during watch cycle |
| NoMotionThresh | Deg/s | 2.0 | F | MPU no motion threshold in gyro degrees/sec |
| NoMotionTime | Secs | 2.0 | F | MPU no motion time, seconds |
| SensorWarmup | Secs | 5 | I | Power-on warmup before sampling system sensors, secs |
| SensorMinSecs | Secs | 5 | I | If SysRecSecs < this, leave sensors on |
| ModemBaud | Bits/s | 9600 | I | Baud rate to talk to acoustic modem |
| DebugBaud | Bits/s | 9600 | I | Baud rate to talk to aux serial port (debug port) |
| Echo | Bool | TRUE | B | Echo user commands when using acoustic modem |
| InitModem | Bool | TRUE | B | TRUE to send init strings to modem at boot |
| InitPicoClock | Bool | TRUE | B | TRUE to init PicoDOS clock from RTC at boot |
| logToConsole |  | 0 | I | See note (3) |

Type Field consists of one or more of the following:
I – Integer (32 bit)
F – Float (32 bit)
S – String variable (32 chars max)
B – Boolean (TRUE or FALSE)

### Notes

(1) Currently not used.

(2) – Normally, we will power the modem on an even boundary of ModemPeriod.  E.g., for ModemPeriod=3600, we will start on the hour.  ModemOffset allows you to offset that convention by a given number of seconds.  In the above example, if ModeOffset is 300 seconds, the modem will be powered starting at 5 minutes after the hour.

(3) Normally, significant events are logged to the system log file SYSLOG.TXT.  This variable will also output those messages to one of the serial ports, as follows:
  0 – Normal operation, don’t echo to serial port
  1 – Also echo to the main system console serial port
  2 – Echo to the aux (debug) serial port

### Startup

There is one additional variable that can be set in BEDS.CFG, though not in the user interface.  This is because by the time the user interface runs, it’s too late.  That variable is

START – set system to start after a delay from initial power-up, or at a specified time.  There are two forms:

START AFTER <n>[s|m|h|d]

This will start the system with a fixed delay after the power-plug is inserted.  With no trailing character, it’s specified in seconds.  The trailing character modifies this to be minutes (m), hours (h), or days (d).  For example START AFTER 6h will start the system with a delay of 6 hours.

START [mm/dd/yyyyThh:mm:ss]

This starts the system at the absolute time specified.  Note the ‘T’ that separates the date from the time.  This can be any character, but must be non-blank.  Other notes on this form:

- If the date is specified without the time, it will start at 00:00:00 on the specified date

- If the time is specified without the date, it will start at time hh:mm:ss of the current day.  Be careful – if the time has passed, the system starts immediately.

- The date can be specified as simply mm/dd.  The system assumes the current year.  Be careful – if the date has passed, the system starts immediately.

- The time can be specified as simply hh:mm.  This is equivalent to hh:mm:00