# Wish List for BEDS Firmware

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 1.4
Date: 9/10/2013

- Handle modem errors due to low battery.  Critical.  Modem resets due to low battery may cause an infinite loop in the user interface.

- New file naming convention.  We agreed on
UDDnnnnn.{WAT,EVT}
where U is unit number (0-9, A-Z), DD is deployment ID, and nnnnn is file number.
New convention is needed to make file names unique across units and deployments, so they can’t get mixed up when archived onshore.  However, this may need to change due to point 3:

- File splitting.  Both watch and event files can grow to be huge.   Huge files are hard to download, and subject to needing a restart due to download errors.  So we need a utility to split them.
Propose to denote split files with the file type field.  E.g.
UDDnnnnn.{Wnn,Enn}
So the base file name is the same, and the split files are W00-W99 and E00-E99.  Allows you to split 100 ways.  Or should I use hex, allowing 256-way splits?  That would yield W00-WFF and E00-EFF.

- Change the representation for acceleration to 16 bits from the current 24.  Potentially reduces the size of recovered data by 16%.  Not desirable, gives up too much resolution.

- Clock calibration.

- Change behavior of EVENT  command, to show events to user.  Sort by magnitude?  Show newest first, and only show 5 at a time, a la ‘more’?

- Selective download of information in the .WAT files.  Decimated inertial data from .WAT files

- Addition of a watchdog timer to reset the unit upon software failure

- Check modem scheduling code.  Right now, we may be turning it on after every event?  Just want an additional modem cycle after a max event.

- Add a deploy mode.  Events not recorded, modemPeriod is faster.  Deploy mode is in effect for 24 hours after boot.  You can also get there via a ‘deploy’ command.

- Need a recovery mode?  Flush CFM, stop logging, …

- Resumable file transfer – Zmodem?

- Decimated event data.  Need to validate data points before averaging them, toss out bad ones.

- Save old versions of BEDS.CLK, a la SYSLOG.TXT.