# BEDS Operating Variables

### Project: Benthic Event Detection System (BEDS) – Project 901006
Revision – 2.03
Date:  12/01/2021

This is a list of the BEDS operating variables.  All variables on this list can be changed in two ways:

- Via the BEDS.CFG configuration file at startup.

- Via the user interface. Commands are case-insensitive (capitalization doesn’t matter)

var accelRange 16

Variables modified at the command line are not saved to the BEDS.CFG file

Variables set to unsuitable values will be modified to be the closest possible value. At the command line, it is repeated back.

# BEDS.CFG

At startup via the BEDS.CFG configuration file, with the variables set in name = value form:
accelRange = 16

The # character is used as a comment indicator, everything after it will not be parsed. Blank lines are also be ignored.

Here are the contents of a typical BEDS.CFG file.

# BEDS.CFG File

DeploymentID = 2

PlatformID = 2

ModemAddr = 2

ModemPeriod = 600

#SysRecSecs = 5

SysRecSecs = 600

# Variables

| Variable Name | Units | Default | Type | Min | Max | Use |
| --- | --- | --- | --- | --- | --- | --- |
| PlatformID |  | 0 | I | 1 | 35 | Platform ID that is put into data files |
| DeploymentID |  | 0 | I | 1 | 99 | Unique ID for deployment, starting at 0 |
| ModemAddr |  | 1 | I | 1 | 99 | Address to initialize modem with (S18) Note this may be different from PlatformID |
| EvtFreq | Hz | 50 | I | 10 | 200 | Sample rate during event, frequency of IMU records during event (can be downsampled from ImuOutputRate) |
| WatchFreq | Hz | 1 | I | 1 | 100 | Sample rate during watch mode |
| ModemPeriod | Secs | 3600 | I | 300 | 7200 | Period (secs) for turning on acoustic modem |
| DeployModemPeriod | Secs | 600 | I | 120 | 1800 | Modem period during deployment mode |
| DeployModeSecs | Secs | 86400 | I | 3600 | 1 week | Amount of time to stay in deployment mode |
| ModemOffset | Secs | 0 | I | 0 | 3600 | See note (2) |
| ModemOnTime | Secs | 60 | I | 60 | 3600 | Period we leave modem on, seconds |
| ModemWarmup | Secs | 15 | I | 5 | 600 | Time after modem powerup to wait for signon message, before watching for actual characters from topside |
| ModemWait | Secs | 5 | I | 1 | 120 | Time to wait before turning off modem, to allow all serial traffic to reach the topside modem |
| ModemPower | Level | 6 | I | 2 | 8 | S6 setting (Tx power) for modem |
| AccelRange | g | 16 | I | 2 | 16 | Accelerometer range in g’s. Valid values: 2, 4, 8, or 16 |
| GyroRange | deg/s | 2000 | I | 250 | 2000 | Gyro range in deg/sec. Valid values: 250, 500, 1000, 2000 |
| imuOutputRate | Hz | 50 | I | 50 | 200 | Output frequency of IMU, probably the max of eventFreq and watchFreq |
| imuOutputBitMask | MAGIC | 0x40403 | I | 0x0000 | 0xFFFF | Describes IMU configuration. (4) |
| Timeout | Secs | 120 | I | 60 | 3600 | Timeout on user I/F, seconds |
| MaxEvent | Secs | 7200 | I | 1800 | 43200 | Max length of recorded event, seconds |
| EvtPressureSecs | Secs | 1 | I | 1 | 600 | Period for sampling pressure during an event, seconds |
| SysRecSecs | Secs | 600 | I | 1 | 3600 | Period for sampling sensors during watch cycle |
| NoMotionThresh | Deg/s | 2.0 | F | 1 | 360 | Motion threshold on gyro degrees/sec used to return from event cycle |
| NoMotionTime | Secs | 2.0 | F | 1 | 60 | Amount of time with no motion before returning from event cycle |
| SensorMinSecs | Secs | 5 | I | 2 | 600 | If SysRecSecs < this, leave sensors on |
| SensorWarmupMs | Secs | 2000 | I | 5 | 60000 | Warmup time for sensors when turning on the system |
| ModemBaud | Bits/s | 9600 | I | 300 | 115200 | Baud rate to talk to acoustic modem |
| DebugBaud | Bits/s | 115200 | I | 300 | 1000000 | Baud rate to talk to aux serial port (debug port) |
| InitModem | Bool | FALSE | B | 0 FALSE | 1 TRUE | Send init to modem on every power up. |
| ModemPort |  | 1 | I | 1 | 2 | Which serial port the acoustic modem is on. Other is used for debug serial. |
| sysLogToConsole |  | 0 | I | 0 | 3 | See note (3) |
| adcAverageNSamples | Count | 10 | I | 1 | 100 | Number of ADC samples to average for one reading |
| adcDelayBetweenSamplesUs | uS | 1000 | I | 100 | 10000 | Delay between individual ADC samples |

Type Field consists of one or more of the following:
I – Integer (32 bit)
F – Float (32 bit)
S – String variable (32 chars max)
B – Boolean (TRUE or FALSE)

### Notes

(*)  Min/max only enforced for integer (32 bit) variables

(1) Currently not used.

(2) – Normally, we will power the modem on an even boundary of ModemPeriod.  E.g., for ModemPeriod=3600, we will start on the hour.  ModemOffset allows you to offset that convention by a given number of seconds.  In the above example, if ModeOffset is 300 seconds, the modem will be powered starting at 5 minutes after the hour.

(3) Normally, significant events are logged to the system log file SYSLOG.TXT.  This variable will also output those messages to one of the serial ports, as follows:
  0 – Normal operation, don’t echo to serial port
  1 – Also echo to the main system console serial port
  2 – Echo to the aux (debug) serial port

(4) The IMU output bit mask controls which sensors are used from the IMU. The possible sensors are described in git\beds\motion\Invn\Devices\Drivers\Icm20948\Icm20948Setup.h. The default set up is:

- Sent to user: INV_ICM20948_SENSOR_ACCELEROMETER (0x0 0001) – calibrated accelerometer

- Sent to user: INV_ICM20948_SENSOR_ROTATION_VECTOR (0x0 0400) – quaternion generated by integration of accelerometer, gyroscope, and magnetometer

- Debug only: INV_ICM20948_SENSOR_ORIENTATION (0x40 0000) – the Euler angles

- Debug only: INV_ICM20948_SENSOR_GYROSCOPE (0x0 0002) – calibrated rate sensor

Note that INV_ICM20948_SENSOR_LINEAR_ACCELERATION (0x20 0000) can be used in place of the accelerometer, it is supposed to take out the gravity vector. However, it has not been fully tested and may require additional calibration of uncertain type.

### Startup

There is one additional variable that can be set in BEDS.CFG, though not in the user interface.  This is because by the time the user interface runs, it’s too late.  That variable is

START – set system to start after a delay from initial power-up, or at a specified time.  There are two forms:

START AFTER <n>[s|m|h|d]

This will start the system with a fixed delay after the power-plug is inserted.  With no trailing character, it’s specified in seconds.  The trailing character modifies this to be minutes (m), hours (h), or days (d).  For example START AFTER 6h will start the system with a delay of 6 hours.

START [mm/dd/yyyyThh:mm:ss]

This starts the system at the absolute time specified.  Note the ‘T’ that separates the date from the time.  This can be any character, but must be non-blank.  Other notes on this form:

- If the date is specified without the time, it will start at 00:00:00 on the specified date

- If the time is specified without the date, it will start at time hh:mm:ss of the current day.  Be careful – if the time has passed, the system starts immediately.

- The date can be specified as simply mm/dd.  The system assumes the current year.  Be careful – if the date has passed, the system starts immediately.

- The time can be specified as simply hh:mm.  This is equivalent to hh:mm:00