Part number: 10020895-MOB1 and 10020895-MOB2

Serial Number:    ___________   Revision: ___________ Date: _______________ Name: _________

Test conditions and tools:

- Fully assembled Mother Board, no Persistor Board, no MPU-6050 board.

- Power up cable (banana plugs to 3 pin Molex), lab power supply; multimeter.

- Test load, 33 Ohm and 16 Ohm with connector pins pluggable to U14C connector where Persistor board is normally plugged.

Test controls and indicators:

- Orange LEDs D5, D11: Power

- Jumpers J2, J4: Cap Discharge

- Core operation. Set power supply to 12V. Plug cable to J1 (Pack 1) or J3 (Pack 2)

  - Verify operation on J1, Pack 1 (Check): __________

  - Verify operation on J3, Pack 2 (Check): __________

- Connect jumpers to J2 and J4, verify LEDs light up

  - Record PS current with LEDs running: ________

  - Record PS current without LEDs running: ________

- Connect test load to 3.3V, switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3V at TP2: at 100mA:______V ; at 200mA: ______V

  - Record noise level at 3.3V at TP2: at 100mA:______V ; at 200mA: ______V

- Test load still at 3.3V, switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3VDQ at TP3: at 100mA:______V ; at 200mA: ______V

  - Record noise level at 3.3VDQ at TP3: at 100mA:______V ; at 200mA: ______V

- Check operation of Modem_Enable with 5 Ohm 50W test load in J5 (2.3A).

  - Measure V-CAP-MODEM at TP6: ____________

  - Measure voltage at “Modem power” connector J5.2: ____________

  - And difference between them in mV (<50mV): ____________

- Check V-SYS (TP8) and V-MODEM-MIN (TP9) trackers with Modem-ENBL cycling and 5 Ohm 50W test load in J5

- Check operation of SENSOR-ENBL and measure 5V (TP7) value and noise.

- Check operation of TPU6 (U14.C27) and voltage on U18.A10

Sensors

- External pressure sensor. Check voltages on J6.3, J6.2, and TP10

- Internal Pressure sensor. Check voltages on U6.1, U6.2, TP11

- Temperature sensor. Check voltage on U10.1 (3.3V) and TP15

- Humidity sensor. Check voltage on U11.3 (3.3V) and TP14.

Plug test Persistor board and run AD Test.

Record values on channels:

0:

1:

2:

3:

4:

5: