Applicable to boards: 10020895-MOB1 and 10020895-MOB2

Part Number: ____-MOB2__   Revision: _Rev1______ Date: ___9/24/2015____ Name: ____Klimov___

Test conditions and tools:

- Fully assembled Mother Board, no Persistor Board, no MPU-6050 board.

- Power up cable (banana plugs to 3 pin Molex), lab power supply; multimeter.

- Test load, 33 Ohm and 16 Ohm with connector pins pluggable to U14C connector where Persistor board is normally plugged.

Test controls and indicators:

- Orange LEDs D5, D11: Power

- Jumpers J2, J4: Cap Discharge

- Core operation. Set power supply to 12V. Plug cable to J1 (Pack 1) or J3 (Pack 2). Connect jumpers to J2, J4 one at the time and both.

  - Verify LED D5 lights up (check): _____Y___________

  - Verify LED D11 Lights up (check): _____Y___________

  - Verify operation on J1, Pack 1 (Check): ____Y________

  - Verify operation on J3, Pack 2 (Check): ______Y_______

- With power supply at 12V, record PS current with and without LEDs running

  - PS current with LEDs running (~ 50 mA): ______43mA____

  - PS current without LEDs running (0 mA): _____0 mA______

- Connect test load to 3.3V (U14C pin 3 is GND, pin 9 is 3.3V), switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3V at TP2:

    - at 100mA: _______3.321____V

    - at 200mA: ______3.318_____V

  - Record noise level at 3.3V at TP2:

    - at 100mA: ______+/-25mV____V ;

    - at 200mA: _____+/-30mV_____V

- Test load still at 3.3V, switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3VDQ at TP3:

    - at 100mA (3.3V): _____3.319____V ;

    - at 200mA (3.3V): _____3.316____V

  - Record noise level at 3.3VDQ at TP3:

    - at 100mA (<15mV): ______+/-12mV____V ;

    - at 200mA (<20mV): ______+/-15mV____V

- Check operation of Modem_Enable (pins 9, 25 of U14C) with 5 Ohm 50W test load in J5 (2.3A).

  - Record V-CAP-MODEM at TP6: ____11.562________

  - Record voltage at “Modem power”  J5.2: __11.534__________

  - Record difference between them in mV (<50mV): ___27.5mV_________

- Check V-SYS (TP8) and V-MODEM-MIN (TP9) trackers with Modem-ENBL cycling and 5 Ohm 50W test load in J5:

  - V-SYS at static 12V power (1V): ______0.983______

  - V-Modem-Min at static 12V power (1V): ______0.984____

  - V-Modem raises slow (40 sec) and drops fast (Check): ___Y_____________

- Check operation of TPU2 (U14.C pin23)

  - Voltage on U18.A pin 10: _____3.322 V____

Sensors

- Check operation of SENSOR-ENBL power (pins 9, 24 of U14C)

  - Measure voltage at TP7 (5V): _____5.033_____

  - Measure noise level at TP7 (<10mV): ___+/-8mV_______

  - Measure reference voltage at TP19 (2.5V): ____2.516 ___________

- External pressure sensor. Check supply voltage, sensor output, and test point voltage

  - Check voltage on J6.3 (5V): ____5.032_________

  - Voltage on pin J6.2

    - (557mV@1bar for 70 bar sensor): __.497 diff sensor_____________

    - (540mV@1bar for 100 bar sensor): ________________

  - Voltage on test point TP10

    - (278mV@1bar for 70 bar sensor): ____0.248 diff sensor___________

    - (270mV@1bar for 100 bar sensor): ________________

- Internal Pressure sensor. Check voltages on U6.1, U6.2, TP11:

  - Voltage on pin U6.1 (5V): ____5.032_____

  - Voltage on pin U6.2 (1.1V): ______1.097V____

  - Voltage on test point TP11(550mV): _____548mV______

- Temperature sensor. Check supply voltage on and sensor output voltage.

  - Voltage on pin U10.1 (3.3V): ____3.343V____

  - Voltage on TP15 (500mV+10mV/C or 730mV@23C): ______729mV____

- Humidity sensor. Check supply voltage on and sensor output voltage.

  - Voltage on pin U11.3 (3.3V): ____3.327__________

  - Voltage on TP14 (0.5-2.5V output):_____1.6V,  2.6V finger test______

Test with Persistor board, AD Test.

- Install coin cell. Check voltage

  - Record backup voltage at TP20 (3V): ___3.215 _________________

- Run ADTEST

  - Read “all A/D channels” (command 8) several times.

  - Make sure values on channels 0-5 are good (Check): _________________

- Install test Persistor board, connect serial cable, configure HyperTerminal for 9600 bps, 8 bits, no parity, 1 stop bit, no flow control.

  - Make sure prompt shows up (check): ___Y____________

  - Read RTC time: ___1939______________

  - Set RTC time, read RTC time: ____1970____________

  - Remove power.

  - Restore power, read RTC time again: ___1970________________

- Save window info to “YYYY-MM-DD-adtest-MBSNxxx.txt” file: __2015-09-25-adtest-MB-SN005.txt

Attach label:

Part number and rev level (MOBx RevX):  _____MOB2 Rev1______

Serial number and date: (SNxxx  YYYY/MM/DD): ___SN005  2015/09/25_____