Applicable to boards: 10020895-MOB1 and 10020895-MOB2

Part Number: ____-MOB2__   Revision: _Rev1______ Date: ___03/24/2016____ Name: ____Klimov___

Test conditions and tools:

- Fully assembled Mother Board, no Persistor Board, no MPU-6050 board.

- Power up cable (banana plugs to 3 pin Molex), lab power supply; multimeter.

- Test load, 33 Ohm and 16 Ohm with connector pins pluggable to U14C connector where Persistor board is normally plugged.

Test controls and indicators:

- Orange LEDs D5, D11: Power

- Jumpers J2, J4: Cap Discharge

- Core operation. Set power supply to 12V. Plug cable to J1 (Pack 1) or J3 (Pack 2). Connect jumpers to J2, J4 one at the time and both.

  - Verify LED D5 lights up (check): _____Y___________

  - Verify LED D11 Lights up (check): _____Y___________

  - Verify operation on J1, Pack 1 (Check): ____Y________

  - Verify operation on J3, Pack 2 (Check): ______Y_______

- With power supply at 12V, record PS current with and without LEDs running

  - PS current with LEDs running (~ 50 mA): ______43mA____

  - PS current without LEDs running (0 mA): _____0 mA______

- Connect test load to 3.3V (U14C pin 3 is GND, pin 9 is 3.3V), switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3V at TP2: at 0A (3.3V): ___3.306____________

    - at 100mA: _______3.302____V

    - at 200mA: ______3.301_____V

  - Record noise level at 3.3V at TP2:

    - at 100mA: not tested____V ;

    - at 200mA: _____not tested_____V

- Test load still at 3.3V, switching between 100mA (33 Ohm) and 200mA (16 Ohm) load

  - Record voltage level at 3.3VDQ at TP3:

    - at 100mA (3.3V): _____3.300____V ;

    - at 200mA (3.3V): _____3.299____V

  - Record noise level at 3.3VDQ at TP3:

    - at 100mA (<15mV): not tested____V ;

    - at 200mA (<20mV): not tested ____V

- Check operation of Modem_Enable (pins 9, 25 of U14C) with 5 Ohm 50W test load in J5 (2.3A).

  - Record PS current (2.3A): ____2.322___________

  - Record V-CAP-MODEM at TP6: ____11.462________

  - Record voltage at “Modem power”  J5.2: __11.435__________

  - Record difference between them in mV (<50mV): ___29 mV_________

- Check V-SYS (TP8) and V-MODEM-MIN (TP9) trackers with Modem-ENBL cycling and 5 Ohm 50W test load in J5:

  - V-SYS at static 12V power (1V) before enabling modem: ______0.983______

  - V-Modem-Min at static 12V power (1V) before enabling modem: ______0.985____

  - V-Modem raises slow (40 sec) and drops fast (Check): ___Y_____________

- Check operation of TPU2 (U14.C pin23)

  - Voltage on U18.A pin 10: _____3.308 V____

Sensors

- Check operation of SENSOR-ENBL power (pins 9, 24 of U14C). Before enabling (0v): __0 v_________

  - Measure voltage at TP7 (5V): _____5.018_____

  - Measure noise level at TP7 (<10mV): not tested_______

  - Measure reference voltage at TP19 (2.5V): ____2.510__________

- External pressure sensor. Check supply voltage, sensor output, and test point voltage

  - Check voltage on J6.3 (5V): ____5.018_________

  - Voltage on pin J6.2

    - (557mV@1bar for 70 bar sensor):  ______________

    - (540mV@1bar for 100 bar sensor): ____.492 mV_gauge sensor _________

  - Voltage on test point TP10

    - (278mV@1bar for 70 bar sensor): ____0.2467 mV___________

    - (270mV@1bar for 100 bar sensor): ________________

- Internal Pressure sensor. Check voltages on U6.1, U6.2, TP11:

  - Voltage on pin U6.1 (5V): ____5.017 V_____

  - Voltage on pin U6.2 (1.1V): ______1.096 V____

  - Voltage on test point TP11(550mV): _____548mV______

- Temperature sensor. Check supply voltage on and sensor output voltage.

  - Voltage on pin U10.1 (3.3V): ____3.343 V____

  - Voltage on TP15 (500mV+10mV/C or 730mV@23C): ______723 mV____

- Humidity sensor. Check supply voltage on and sensor output voltage.

  - Voltage on pin U11.3 (3.3V): ____3.326_V_________

  - Voltage on TP14 (0.5-2.5V output): _____1.550 V,  2.470V finger test______

Test with Persistor board, AD Test.

- Install coin cell. Record date code (on packaging typically)

  - Record and Write date code on a cell: __3  2016_________

  - Record backup voltage at TP20 (3V): ___3.221 ___________

- Install test Persistor board, connect serial cable, configure HyperTerminal for 9600 bps, 8 bits, no parity, 1 stop bit, no flow control.

  - Make sure prompt shows up (check): ___Y____________

  - Check operation of reset button (check): ___Y___________

  - Read RTC time: ___1936______________

  - Set RTC time, read RTC time: ____2000____________

  - Remove power.

  - Restore power, read RTC time again: ___2000________________

- Run ADTEST

  - Read “all A/D channels” (command 8) several times.

  - Make sure values on channels 0-5 are good (Check): ______Y___________

- Save window info to “YYYY-MM-DD-adtest-MBSNxxx.txt” file: __2016-03-24-adtest-MB-SN012.txt

Attach label:

Part number and rev level (MOBx RevX):  _____MOB2 Rev1______

Serial number and date: (SNxxx  YYYY/MM/DD): ___SN12  2016-03-24_____