# Notes on Designing the Persistor CF2 Prototype for BEDS

Rev 1.2   7/26/2012

These are some notes on how to interface the various BEDS components to a Persistor CF2.  For the tables below, TPUx means you can use any of the TPU lines, TPU1 through TPU15.

## Power

There are two ways to develop power for the Persistor board.  The easiest is to use the on-board linear regulator.  This wastes a lot of battery power, but is very quiet.  A DC-DC converter is more efficient, but may generate too much noise on the Vcc and ground lines for the analog signals.  You just need to try out both.  Talk to Craig Okuda about this (or about anything having to do with the Persistor).  He’s the in-house expert.

If you want to use the on-board linear regulator, just bring the raw battery into CF2 pin 13.  You should add about 10uf of decoupling.  Then tie together CF2 pins 9 and 11, and use that for any 3.3v logic you need.  Again, you should provide about 10uf of decoupling.

If you want to provide a DC-DC converter, leave open pins 11 and 13 on the CF2, and provide 3.3v to CF2 pin 9.  The traditional converter chip to use is the MAX1837EUT33.  But it’s noisy, and possibly not very available any more.  I once did a design with the TPS62125 from TI, but it’s a tiny chip that’s very hard to use. Very efficient, though; and low noise, since it switches at 1 MHz.

Again, talk to Craig Okuda about power for the CF2.  You need to design for a peak current of 200 mA, though the average will be more like 50 or 60 mA.  To that, you need to add whatever current will be used by devices other than the Persistor CF2.

## Invensense Chip or Board

This documents the pins on the UC3 proto board from Invensense.  If you want to design directly to the Invensense MPU-6050 chip, just substitute the correct chip pin numbers for the UC3 board pin numbers.

| Persistor CF2 |  | Invensense UC3 Board |  |
| --- | --- | --- | --- |
| Pin(s) | Signal | Pin(s) | Signal |
| 9 | VReg | 10 | 3V3_X |
| 3 | Gnd | 9 | Gnd |
| 19 | /PCS1 | 1 | SDA |
| 21 | /PCS0 | 2 | SCL |
|  | TPUx | 8 | PWR_EN_X |
|  | TPUx |  | See note |
| 40 | /IRQ7 |  | See note |

Note – the interrupt signal (MPU_IRQ_X on the UC3 board pin 4) needs to be conditioned and sent to /IRQ7 on the CF2.  But it needs to be NAND’ed with a TPU bit.  Bring the MPU_IRQ_X and any TPU line into a NAND gate, and send the output to /IRQ7.  The TPU line needs a weak (1M) pull-down to ground.

## Clock Chip – Maxim DS3234

| Persistor CF2 |  | Maxim 3234 |  |
| --- | --- | --- | --- |
| Pin(s) | Signal | Pin(s) | Signal |
| 15 | /PCS2 | 1 | /CS |
| 16 | SCK | 20 | SCK |
| 18 | MOSI | 17 | DIN |
| 20 | MISO | 19 | DOUT |
| 14 | Vbbk | 16 | Vbat |
| 41 | /IRQ2 | 5 | /Int |
|  |  | 3 | 32kHz - note |

Note – Bring pin 3 to a test point on the board

## ADS8344 Analog-Digital Converter

| Persistor CF2 |  | ADS8344 |  |
| --- | --- | --- | --- |
| Pin(s) | Signal | Pin(s) | Signal |
| 17 | /PCS3 | 18 | /CS |
| 16 | SCK | 19 | SCK |
| 18 | MOSI | 17 | DIN |
| 20 | MISO | 15 | DOUT |
|  | TPUx | 10 | /SHDN |

Note – Pin 11, Vref, should be connected to a LT1461 reference.  The TPU line driving /SHDN should also drive the Shutdown pin of the LT1461.  Just copy the design on page 13 of the ReceipeCards data sheet.

## Serial Ports

The main serial console port should be pinned out as shown on page 13 of the RecipeCards data sheet.  In addition, we’ll need a serial port for the acoustic modem.  Pick two TPU lines for Rx and Tx.  Traditionally, it would be TPU15 and 14, or TPU13 and 12.  But any two will do.  Wire them into an RS-232 driver chip, e.g. the MAX3222, as shown on the RecipeCards data sheet.  Bring the EIA signals out to a connector you deem appropriate for wiring to the modem.

## Miscellaneous

Please also wire the WAKE, PBM, and RESet pushbuttons as shown on the RecipeCards data sheet.  Also the Lithium backup battery, which is critical to proper operation.  We don’t need the LEDs.