BRS Deploy Check List

12/14/2017

Set up

- Install bags of clean water and purge inject lines.

- Fill chambers with 2” of clean water, vac out chambers first if necessary

- Plug the Shore cable into the com connector, laptop and power supply (Set to15- 16 VDC)

- Turn on the run switch on the Shore Cable

- Type cartridge return

- Type “reset pico”

- Copy oasis.cfg file to system (use y modem send/receive)

- Load script files xxx.scp (must this be resp.scp, stir.scp, modlog.scp )

- Delete old data files (oxy.txt, env.txt etc)

- Check DOS time and date

- Power cycle the system with the switch (switch on connector cable)

- Watch the system run at least 1 cycle

- Test stir, flush and inject pumps.

- Type the oxy.txt files to verify optodes are working

- Reset pico and delete files again

- Check vent port.

- Power the system off and disconnect the power supply

- Install dummy on console port

- Charge battery, oil bottle installed

Pre deploy

- Remove battery charger and plug in power to system

- Purge gas from batteries, top off oil

- All inject valves open. Green valves under the chambers

- Plugs on the bottom of chambers installed

- Load O2/co2 bag(s)

- Top off pump compensation

- Check that run plug is tethered to console port connector

Deploy

- Log Homer number?

- Log Sondardyne beacon number?

- Turn Strobe and VHF Beacon on

- Anchors installed

- Remove release safety pins

- Inject fluid bag connected and valves on (if installed)

- Optode covers off 9 of them

- Batteries plugged in (grease connector)

- Remove battery oil bottles. Clean up oil drips off the couplers

- Fill chambers with seawater

- Install run plug (grease connector)

- Verify the lights on the chamber controllers are on and blinking

Recovery

- Remove run plug and install dummy (grease connector)

- Strobe and VHF Beacon off

- Inject fluid bag lure valves off

- Optode covers on 9 of them

- Install battery oil bottles

- Charge batteries, dummy off system power connections

- Wash out chambers and flush lines

- Wash down elevator