BRS/OASIS software cheat sheet

Draft 12/18/2017

Notes:

- Console port settings: 38400, 8 data bits, No parity, 1stop bit

- To get to Picodos from BRS prompt, Type “reset pico”

- To restart OASIS/ BRS application, type “app” or power cycle for 15 seconds.

Picodos commands used in BRS set up (just the common ones)

- App	launches BRS application from Picodos.

- Copy

- Date

- Dir

- Erase

- Rename

- Reset	Resets the processor, after reboot system will start BRS app after reboot.

- Time

- Type	Displays contents of a text file to screen. At 38400 a big file can take a long time.

- YS	Y-modem Send, used to transfer files from Picodos to PC.

- YR	Y-modem Receive, used to transfer files from PC to Picodos.

BRS commands, common ones. Some dos commands pass through.

- Scripts 	Shows the scripts that are currently running.

- Scripts stop	Stops the current scripts.

- Rempulse	Used to activate an output point for testing like pumps.

- Analog		Displays current a/d input values, batt volts, can temp, can pressure

- DP		Driver Parameter, shows what drivers are doing.

- Type		Displays contents of a text file to screen. At 38400 a big file can take a long time.

- Time

- Date

- Dir

- Resetpico	kills BRS app and goes back to picodos.

Stir, Flush and Inject Testing

Use rempulse (remote pulse) command from the BRS prompt.

Rempulse can be used on:

- O2_inject(number)

- Stir(number)

- Flush(number)

- Any other DO defined in the OASIS config

Examples

- Rempulse “o2_inject1 30” = O2 inject pump 1 on for 30 seconds.

- Rempulse “stir2 10” = stir 2 on for 10 seconds.

Transfer files with Y-modem examples

Move oxy.txt to PC. (from BRS prompt)

- Reset Pico to get to picodos.

- Type “YS oxy.txt”

- At Hyperterminal, receive file, y-modem

Move resp.scp from PC to BRS. (from BRS prompt)

- Reset Pico to get to picodos.

- Rename existing resp.scp (optional). Type “rename resp.scp resp1.scp”

- Type “erase resp.scp” if you did not rename existing resp file.

- Type “YR” to start Y-modem receive

- At Hyperterminal, send file, y-modem, select the new resp.scp.