7700 Sandholdt Road

Moss Landing, CA 95039

# WiSSL Gen 2 FAT

31st March 2025

## Purpose

The FAT is set to take place March 31st, 2025 in Longmont, Colorado at 3D@Depth’s research and development branch.

## Review of Requirements

## Pre-FAT Issues

- The laser limited at 60% output power, to prevent damaging the attenuator which is impacting output power.

- Conversely, when the laser is cold, adjustments have to be made to the SHG TEC control to adjust the laser output to a level to see a trigger signal to the DAQ

- Performance across the full swath is not consistent, due potentially to distortion of the beam in the optical bench and distortion through the curved window.

- Optical Power output of the laser is low, and the sensor does not reach the 10m range requirement.

- I2C issues on boot are leading to long boot times

- 200kHz is stable, 350kHz logging cannot keep up.

- Low Power mode is only leading to a 5% improvement in consumption.

- RedA Temp sensor not responding on I2C.

- Spot size is larger than expected (3cm @ 8m)

- System not tested past 4,000 working depth.

- Outputs on red channels are very weak.

- Accelerometer feature not complete.

## FAT Identified Issues

- 2 Mechanical

  - No manual presented for laser safety.

  - No spare items presented.

  - Anodes are missing on sensor.

  - Safety cover is missing mounting holes.

- 3 Electrical and Thermal

  - Cannot scan at 36V input due to calibration error on VDC measurement.

  - Low power mode at 24VDC is 1.6% lower.

  - Need to test pin isolation

  - Initial scans fail after power up.

- 4 Software

  - Remote protocol was not able to be tested.

  - CCI Needs update, missing power and attenuation controls.

  - Pan & Tilt vestigial controls and script information remains.

  - Angles on visualizer has consistent orientation with software.

  - Will the manual be available April 4th?

- 5 Performance

  - Red channels were difficult to see significant signal, thermal disturbance had little impact.

  - No accelerometer

  - Pulling the ethernet does not succeed at this time, leaving laser on with no recovery mechanism.

## Action Items

### Last Day of FAT

- Finish all FAT Tests

- Characterize beam size at 3m (NTH)