System/Prescription Data

File : C:\Users\dick\Documents\ZEMAX\my zemax files 120609\HOBSON MONTEREY 040211\Hobson dome 5mm DURABLE REV  082912a.ZMX

Title: dome corrector revised 082912

Date : 8/31/2012

LENS NOTES:

with TORC testplates.

WITH OVERSIZED CLEAR APERTURES TO COMPENSATE ANTICIPATED WORST CASE COSINE ELONGATION

OF APERTURE STOP AT CORNER OF FIELD OF VIEW.

GENERAL LENS DATA:

Surfaces                :               14

Stop                    :               10

System Aperture         : Float By Stop Size = 1.4

Glass Catalogs          : OHARA MISC-KRC SCHOTT

Ray Aiming              : Real Reference, Cache On

X Pupil Shift           :               0

Y Pupil Shift           :               0

Z Pupil Shift           :               0

X Pupil Compress        :               0

Y Pupil Compress        :               0

Apodization             : Uniform, factor =   0.00000E+000

Temperature (C)         :    2.00000E+001

Pressure (ATM)          :    1.00000E+000

Adjust Index Data To Environment  : Off

Effective Focal Length  :        4.039056 (in air at system temperature and pressure)

Effective Focal Length  :        4.039056 (in image space)

Back Focal Length       :        19.98573

Total Track             :          71.518

Image Space F/#         :        1.780618

Paraxial Working F/#    :        1.783174

Working F/#             :        1.851562

Image Space NA          :       0.2699861

Object Space NA         :    0.0004968087

Stop Radius             :             1.4

Paraxial Image Height   :        5.483699

Paraxial Magnification  :    -0.001771793

Entrance Pupil Diameter :        2.268345

Entrance Pupil Position :        58.03209

Exit Pupil Diameter     :        2.799974

Exit Pupil Position     :              -5

Field Type              : Object height in Millimeters

Maximum Radial Field    :            3095

Primary Wavelength      :       0.5875618 µm

Lens Units              :   Millimeters

Angular Magnification   :        1.085184

Fields          : 4

Field Type              : Object height in Millimeters

#        X-Value        Y-Value         Weight

1       0.000000       0.000000       6.000000

2       0.000000    1548.000000       1.000000

3       0.000000    2167.000000       1.000000

4       0.000000    3095.000000       1.000000

Vignetting Factors

#       VDX       VDY       VCX       VCY       VAN

1  0.000000  0.000000  0.000000  0.000000  0.000000

2  0.000000  0.000000  0.000000  0.000000  0.000000

3  0.000000  0.000000  0.000000  0.000000  0.000000

4  0.000000  0.000000  0.000000  0.000000  0.000000

Wavelengths     : 3

Units: µm

#          Value         Weight

1       0.486133       1.000000

2       0.587562       1.000000

3       0.656273       1.000000

SURFACE DATA SUMMARY:

Surf     Type         Radius      Thickness                Glass      Diameter          Conic   Comment

OBJ STANDARD       Infinity           3000             SEAWATER          6190              0

1 STANDARD       Infinity              0             SEAWATER      115.6873              0

2 STANDARD          40.64          8.128               S-BSL7            77              0 dome

3 STANDARD         32.512          25.39                              61.78              0 nominal focus

4 STANDARD        -152.56              4                  SF4            58              0 NEW ELEMENT A

5 STANDARD       Infinity              1                                 60              0

6 STANDARD          25.76             12                  BK7            41              0 NEW ELEMENT B

7 STANDARD         77.384              1                                 37              0

8 STANDARD             15              0               S-BSL7            26              0 dummy front

9 STANDARD             15             15                                 26              0

STO STANDARD       Infinity              0                                2.8              0

11 PARAXIAL              -              0                                2.8              - perfect lens

12 STANDARD       Infinity              5                                2.8              0

13 STANDARD       Infinity              0                           11.00447              0

IMA STANDARD       Infinity                                          11.00447              0