System/Prescription Data

File : C:\Users\dick\Documents\ZEMAX\my zemax files 120609\HOBSON MONTEREY 040211\Hobson dome 5mm revised dome091512B.ZMX

Title: dome corrector revised 091512

Date : 9/15/2012

LENS NOTES:

with TORC testplates.

Corrected dome dimensions.

GENERAL LENS DATA:

Surfaces                :               14

Stop                    :               10

System Aperture         : Float By Stop Size = 1.4

Glass Catalogs          : OHARA MISC-KRC SCHOTT

Ray Aiming              : Real Reference, Cache On

X Pupil Shift           :               0

Y Pupil Shift           :               0

Z Pupil Shift           :               0

X Pupil Compress        :               0

Y Pupil Compress        :               0

Apodization             : Uniform, factor =   0.00000E+000

Temperature (C)         :    2.00000E+001

Pressure (ATM)          :    1.00000E+000

Adjust Index Data To Environment  : Off

Effective Focal Length  :        4.049874 (in air at system temperature and pressure)

Effective Focal Length  :        4.049874 (in image space)

Back Focal Length       :        19.98738

Total Track             :            74.8

Image Space F/#         :        1.781207

Paraxial Working F/#    :        1.783774

Working F/#             :        1.852051

Image Space NA          :       0.2699018

Object Space NA         :    0.0004975251

Stop Radius             :             1.4

Paraxial Image Height   :        5.493455

Paraxial Magnification  :    -0.001774945

Entrance Pupil Diameter :        2.273668

Entrance Pupil Position :        60.79482

Exit Pupil Diameter     :             2.8

Exit Pupil Position     :              -5

Field Type              : Object height in Millimeters

Maximum Radial Field    :            3095

Primary Wavelength      :       0.5875618 µm

Lens Units              :   Millimeters

Angular Magnification   :         1.08773

Fields          : 4

Field Type              : Object height in Millimeters

#        X-Value        Y-Value         Weight

1       0.000000       0.000000       6.000000

2       0.000000    1548.000000       1.000000

3       0.000000    2167.000000       1.000000

4       0.000000    3095.000000       1.000000

Vignetting Factors

#       VDX       VDY       VCX       VCY       VAN

1  0.000000  0.000000  0.000000  0.000000  0.000000

2  0.000000  0.000000  0.000000  0.000000  0.000000

3  0.000000  0.000000  0.000000  0.000000  0.000000

4  0.000000  0.000000  0.000000  0.000000  0.000000

Wavelengths     : 3

Units: µm

#          Value         Weight

1       0.486133       1.000000

2       0.587562       1.000000

3       0.656273       1.000000

SURFACE DATA SUMMARY:

Surf     Type         Radius      Thickness                Glass      Diameter          Conic   Comment

OBJ STANDARD       Infinity           3000             SEAWATER          6190              0

1 STANDARD       Infinity              0             SEAWATER      122.6588              0

2 STANDARD           41.5            9.5               S-BSL7            77              0 dome

3 STANDARD             32           27.3                              61.78              0 nominal focus

4 STANDARD        -226.18              4                  SF4            58              0 NEW ELEMENT A

5 STANDARD          906.9              1                                 60              0

6 STANDARD         23.773             12                  BK7            41              0 NEW ELEMENT B

7 STANDARD         55.142              1                                 37              0

8 STANDARD             15              0               S-BSL7            26              0 dummy front

9 STANDARD             15             15                                 26              0

STO STANDARD       Infinity              0                                2.8              0

11 PARAXIAL              -              0                                2.8              - perfect lens

12 STANDARD       Infinity              5                                2.8              0

13 STANDARD       Infinity              0                           11.03679              0

IMA STANDARD       Infinity                                          11.03679              0

SURFACE DATA DETAIL:

Surface OBJ STANDARD

Surface   1 STANDARD

Surface   2 STANDARD dome

Aperture                 : Floating Aperture

Maximum Radius           :             38.5

Surface   3 STANDARD nominal focus

Aperture                 : Floating Aperture

Maximum Radius           :            30.89

Surface   4 STANDARD NEW ELEMENT A

Aperture                 : Floating Aperture

Maximum Radius           :               29

Surface   5 STANDARD

Aperture                 : Floating Aperture

Maximum Radius           :               30

Surface   6 STANDARD NEW ELEMENT B

Aperture                 : Floating Aperture

Maximum Radius           :             20.5

Surface   7 STANDARD

Aperture                 : Floating Aperture

Maximum Radius           :             18.5

Surface   8 STANDARD dummy front

Aperture                 : Floating Aperture

Maximum Radius           :               13

Surface   9 STANDARD

Aperture                 : Floating Aperture

Maximum Radius           :               13

Surface STO STANDARD

Surface  11 PARAXIAL perfect lens

Focal length             :                5

OPD Mode                 :                1

Surface  12 STANDARD

Surface  13 STANDARD

Surface IMA STANDARD

COATING DEFINITIONS:

EDGE THICKNESS DATA:

Surf           Edge

OBJ    3000.000000

1      26.008067

2       7.136821

3       1.788269

4       6.363175

5      12.238997

6       3.460640

7       5.320716

8       0.000000

9       7.483315

STO       0.000000

11       0.000000

12       5.000000

13       0.000000

IMA       0.000000

INDEX OF REFRACTION DATA:

System Temperature:   20.0000 Celsius

System Pressure   :    1.0000 Atmospheres

Absolute air index:    1.000272 at wavelength 0.587562 µm

Index data is relative to air at the system temperature and pressure.

Wavelengths are measured in air at the system temperature and pressure.

Surf                  Glass    Temp    Pres     0.486133   0.587562   0.656273

0               SEAWATER   20.00    1.00   1.34356417 1.33952898 1.33770190 source: Laikin, Lens Design

1               SEAWATER   20.00    1.00   1.34356417 1.33952898 1.33770190 source: Laikin, Lens Design

2                 S-BSL7   25.00    1.00   1.52190494 1.51632992 1.51385515 Recommended

3                          20.00    1.00   1.00000000 1.00000000 1.00000000

4                    SF4   20.00    1.00   1.77468086 1.75520125 1.74729815 lead containing glass type

5                          20.00    1.00   1.00000000 1.00000000 1.00000000

6                    BK7   20.00    1.00   1.52237629 1.51680003 1.51432235

7                          20.00    1.00   1.00000000 1.00000000 1.00000000

8                 S-BSL7   25.00    1.00   1.52190494 1.51632992 1.51385515 Recommended

9                          20.00    1.00   1.00000000 1.00000000 1.00000000

10                          20.00    1.00   1.00000000 1.00000000 1.00000000

11                          20.00    1.00   1.00000000 1.00000000 1.00000000

12                          20.00    1.00   1.00000000 1.00000000 1.00000000

13                          20.00    1.00   1.00000000 1.00000000 1.00000000

14                          20.00    1.00   1.00000000 1.00000000 1.00000000

THERMAL COEFFICIENT OF EXPANSION DATA:

Surf                  Glass     TCE *10E-6

0                SEAWATER     0.00000000 source: Laikin, Lens Design

1                SEAWATER     0.00000000 source: Laikin, Lens Design

2                  S-BSL7     7.20000000 Recommended

3                             0.00000000

4                     SF4     8.00000000 lead containing glass type

5                             0.00000000

6                     BK7     7.10000000

7                             0.00000000

8                  S-BSL7     7.20000000 Recommended

9                             0.00000000

10                             0.00000000

11                             0.00000000

12                             0.00000000

13                             0.00000000

14                             0.00000000

ELEMENT VOLUME DATA:

For centered elements with plane or spherical circular faces, exact

volumes are computed by assuming edges are squared up to the larger

of the front and back radial aperture.

For all other elements, approximate volumes are numerically integrated

to 0.1% accuracy. Zero volume means the volume cannot be accurately computed.

Single elements that are duplicated in the Lens Data Editor

for ray tracing purposes may be listed more than once yielding

incorrect total mass estimates.

Volume cc   Density g/cc         Mass g

Element surf   1 to   2     237.557022       1.000000     237.557022

Element surf   2 to   3      60.638161       2.520000     152.808166

Element surf   4 to   5      14.820136       4.790000      70.988450

Element surf   6 to   7      11.426695       2.510000      28.681004

Element surf   8 to   9       0.000000       2.520000       0.000000

Total Mass:                                               490.034643