# WISSL-2G housing Review

Drawing communication date: April 17th 2024

Reviewer: Justin Tucker

Last update: 4/18/2024

Review Scope:

- Survivability at 6000m

  - Justin (FOS seems low)

    - Consider OD increase

- O-ring design

  - Justin (warning)

    - On the edge for recommended squeeze

    - Extrusion gap at 1500psi is a problem

- Validate we have the correct connector and they are OK with placement

  - Erik (OK’d)

- Do they have any issues with using SS 17-4 PH H1025 material for the optical endcap?    We are looking at changing the heat treat from the current product for strength.

  - Justin – Not ideal. do we know why they don’t want to use Ti? Methods exist for using a hardened steel backing plate to reinforce on the inside to support Ti in stiffness for the optic interface.

# Survivability

## Tube Review

Yield seems fine, buckling FOS seems low.

## End cap

# O-ring

Warning

- O-ring extrusion gap exists at 1500psi!

- Min tolerance stack slightly lower squeeze than recommended.