Requirements for a Low-Altitude Survey AUV

4 November 2010 – 10 am Harbor Conference Room

David Caress, Hans Thomas, Brett Hobson, Steve Rock, Charlie Paull

Functional Requirements:

- Sensors provide 5 cm scale topography and co-located optical imagery

  - High frequency 1°x1° or better multibeam

  - Stereo color cameras w/ lighting (grayscale for altitudes > 3 m)

  - Sonar and cameras physically tied

  - Attitude sensor physically tied to sonar & cameras

- Require 100% coverage in arbitrarily steep/complex terrain

- Need to be able to extricate itself from under an overhang

- Sensor package must be oriented normal to seafloor, even when surveying a vertical wall

- Operate from altitudes of 2-10 m

- Minimum depth requirement of 4000 m

- Minimum practical endurance 12 hours, aim for 18 hours or more

- Minimize ascent and descent

- Must fit in shipping volume of 4’x8’x18’

- Can be operated from future “coastal vessel”

- Ancilliary sensors

  - CTD

  - Eh

  - Optical backscatter

  - Magnetometer

  - Flourometer and other dissolved gas sensors

  - O2?

  - Mass spectrometer?

  - ESP?

Implications

- Four degrees of freedom in controls

- Great emphasis on obstacle avoidance

- Great emphasis on SLAM using both multibeam and imagery

- Have to choose between having an articulated sensor package or a vehicle that can tilt arbitrarily

- Enable use of dropweights for ascent and descent