Requirements for a Low-Altitude Survey AUV

12 January 2011 – 10 am Harbor Conference Room

David Caress, Hans Thomas, Brett Hobson, Steve Rock

Agenda:

  - Starting up Ocean Imaging Project

  - Finish discussing requirements for low altitude survey AUV

Starting Up Ocean Imaging

- Scheduled ROV dives

  - Western Flyer/Ricketts June 1,2,3

  - Point Lobos/Ventana  October 5,6,10

- Reson 400 kHz multibeam will arrive late this month

  - Need to decide terms of support agreement with Reson

- Need to identify and order other equipment

  - Attitude and Nav

    - Could use existing Kearfott from the Mapping AUV

    - Could use the existing Octans and DVL on the Ricketts

  - Cameras

- There is an opportunity for an early Ricketts dive in March (possible Rock dive – DMO looking to move shallow work early while the tether is only 1800 m long)

  - We will run the 400 kHz multibeam on this day

  - We will mount the Kearfott too and compare attitude and nav from the ROV sensors and the Kearfott

- Need to start up planning with ROV Pilots – Eric Martin has been identified as our primary contact

Functional Requirements:

- Sensors provide 5 cm scale topography and co-located optical imagery

  - High frequency 1°x1° or better multibeam

  - Stereo color cameras w/ lighting (grayscale for altitudes > 3 m)

  - Sonar and cameras physically tied

  - Attitude sensor physically tied to sonar & cameras

- Navigation good to 0.05% distance traveled without terrain or vision aiding

- Sensor attitude accurate to 0.05° at 10 Hz (required for multibeam)

- Require 100% coverage in arbitrarily steep/complex terrain

- Need to be able to extricate itself from under an overhang

- Sensor package must be oriented normal to seafloor, even when surveying a vertical wall

- Operate from altitudes of 2-10 m

- Minimum depth requirement of 4000 m

- Minimum practical endurance 12 hours, aim for 18 hours or more

- Maximum speed 1 m/s (2 knots) for near bottom imaging

- Stable to within 3 degrees with rate less than 12 degrees / second

  - This is with respect to the terrain – sensors pointed normal to seafloor

- Minimize ascent and descent times – 30 m/minute minimum ascent descent rates

- Must fit in shipping volume of 4’x8’x18’

- Can be operated from future “coastal vessel”

- Ancilliary sensors

  - CTD

  - Eh

  - Optical backscatter

  - Magnetometer

  - Flourometer and other dissolved gas sensors

  - O2?

  - Mass spectrometer?

  - ESP?

Implications

- Five independent degrees of control for the sensor package

  - Surge (fore-aft), heave (up-down), sway (lateral), yaw (heading), pitch

- Four degrees of freedom in vehicle controls

  - Surge (fore-aft), heave (up-down), yaw (heading), sway (lateral) (actuating sway is unusual)

  - Pitch and roll can be passive, but pitch control would improve ascent & descent efficiency

- Kearfott-class INS with DVL aiding

- Great emphasis on obstacle avoidance

- Great emphasis on SLAM using both multibeam and imagery

- Have to choose between having an articulated sensor package or a vehicle that can tilt arbitrarily

  - Preference for sensor package articulated with one rotational degree of freedom

- Enable use of dropweights for ascent and descent

What to do next?

- Think more about the stability requirements for these sensors

- Map these requirements and implications compared to existing systems

  - Dorado

  - Tethys

  - Sentry

  - ABE

  - SeaBed

  - DepthX

  - Otter

  - Odin