Ocean Imaging Project Meeting

9 February 2011

Hans Thomas, Brett Hobson, David Caress, Eric Martin, Larry Bird

Among the issues to discuss are:

1) What I saw at WHOI-DSL last week (Jon Howland's stereo camera rig).

2) Camera choices

3) Multibeam status

4) Multibeam test plans

5) Possible Postdoc joining the team later this year.

- Larry showed CAD drawings of the Ventana toolsled with multibeam arrays, cameras, INS, and housing.

  - Move the cameras back towards the center.

  - CBe able to place cameras either along or across the vehicle.

  - Strobes at all four corners.

  - Rough idea of the camera separation is 18 inches, but needs to be fully defined.

  - Put the INS as close as possible to the center to minimize lever arms to the sensors.

  - Will get multibeam arrays within a couple of weeks.

  - The Kearfott is still at Kearfott getting fixed

- Camera choices

  - Reg Wilson says best you can do is get 12 bits dynamic range out of current camera sensors

  - Can take multiple images with different settings (old style bracketing)

  - Prosilicas have higher noise than specs would show. Steven McPhail at Southhampton is going to test Point Gray Grasshopper II camera.

  - All cameras use either a Sony or  a Kodak sensor.

- Can we get GigE to the surface?

  - There are some media converters that should work on Ventana.

- What photo rate?

  - Figure ROV runs at 0.2 m/s.

  - Want to use a single trigger pulse, can have a trigger divisor on cameras and strobes.

  - Will need better than 50% overlap in pictures.