MBARI ROV Low Altitude Survey System (Seafloor Mapping Project)

Operations Checklist

R/V Rachel Carson & ROV Ventana

Pre-Dive

ROV

Verify camera covers are removed

Clean camera domes

Verify laser covers are removed

Final check of connectors and cable routing (add zip ties as needed)

Control Room

Power up Euler (located in dry lab)

IP: 192.168.168.91

User: ops

Pswd: rov96rov

Ask pilots to turn on AC power to LASS

Ask pilots to turn on hydraulics

Euler

Open terminal window

Change to directory: OI_Deployments

Turn on all eight low power relays via ‘startlowpower’ or individually

‘relay x 1’ : turns on relay #x

‘relay x 0’ : turns off relay #x

1 Right Camera

2 Left Camera

3 Rotator #1

4 Rotator #2

5 CTD

6 ParoScientific

7 Kearfott

8 VN100

Run Mike’s GUI

Run PC-DAS

Run LCM-Spy

Run Procman Sheriff

Mike’s GUI

Verify Power/Interface can temperature and humidity

Turn on Reson T-50

Turn on LIDAR

Which services to start on Procman Sheriff?

Camera Startup

General Info

IP Right (Master) Camera: 192.168.168.135

IP Left (Slave) Camera:      192.168.168.136

Camera Model: Prosilica GX1920

Lens 5mm with corrective optics and dome port

Camera aperture F4

Camera focus for 2-3 meters in water

Camera control is in: ~/oi_bin

Camera control configuration is set in: /home/ops/src/perls/config/master.cfg

Camera images are logged to: /home/ops/ *date* /images/

To verify camera connectivity, ping each camera or run SampleViewer

To run the camera control program, run ‘mbari-oi-run’ or

‘oi_startall’

‘oi_killall’

* Note that strobes are OFF at this point

To check the camera humidities:

‘logthr’

‘logthl’

Kearfott Setup

In PC-DAS

Set Kearfott to Com1

Kearfott should be in idle mode

If it comes up in Doppler Align Mode, cycle power to Kearfott

Turn on checkbox for GPS aid align

Turn on checkbox for GPS depth damp

Turn on checkbox for Doppler track enable

Turn on checkbox for Lat/Lon valid (this turns on USBL aiding)

GPS GGA string should be updating

Kearfott status will change to “GPS free align complete” when aligned

Kearfott is ready to go at this point

Turn off Lat/Lon valid checkbox when on bottom with good DVL

Get ParoSci output     (???)

Start WinFrog   IP:

Start _______   IP:

Start Giancarlo IP:

Reson Setup

Start up remote desktop on Reson

IP: 192.168.168.132

User: reson

Pswd: reppe

Start up 7kcenter (should come up automatically)

Run UI – seabat7k from desktop

Running FP 1.3.2   (???)

LIDAR Setup

Run 3d@Depth software (laptop)

Only start the CTD service (via Procman Sheriff) after the LIDAR PPS and ZDA lights are green

Lasing by each head is indicated by the waterfall screens

ParoSci Setup

Is powered up already

Data should be present in PC-DAS

Time Server

IP: 134. 89.20.5

Basic Pre-Dive Steps

Kearfott on and ok, processing GPS & DVL, aligned

Multibeam on and ok

Camera Functionality

Verify camera domes and LIDAR glass were cleaned

Ping Right Camera 192.168.168.135

Ping Left Camera 192.168.168.136

Right Camera humidity ‘logthr’

Left Camera humidity ‘logthl’

Strobe Functionality

Turn both strobes on with Mike’s GUI

Inform deck crew that strobes will be firing

Run ‘oi_startall’

Confirm strobes are firing from deck crew

Images on Euler

Run ‘oi_killall’

Strobe rotators

Strobe rotator oil comp

LIDAR Functionality

PPS

Receiving CTD data

Humidity on both heads

Verify safety covers on both laser heads are removed

Set optical power to an eye safe mode

Inform deck crew lasers are coming on

Turn on lasers

Confirm both heads are lasing from deck crew

Turn off lasers

CTD wetting loop removed

CTD and Kearfott data logging

VN100 logging

Risi control software receiving INS data

# Edgetech sub bottom on and ok       <- No longer used

Sensor frame articulation

Swing arm articulation   <- Do we still do this on deck?

-------------------------------------------------------------

If all good, Inform crew on deck and pilots we’re ready for launch

ROV Descent

* Start VARS recording once vehicle is in the water *

In water multibeam humidity ok

In water LIDAR humidity on both heads ok

In water left camera humidity ok

In water right camera humidity ok

Swing arm articulation  <- Do we do this on descent?

Check USBL aiding enabled on Kearfott

Kearfott INS

Kearfott serial number

Kearfott on

Kearfott getting and processing USBL

Kearfott getting and processing DVL

Kearfott aligned

Turn on "Lat/Lon Valid" on program PC-DAS

Turn off "GPS Depth Damp" on PC-DAS

Start Kearfott record transmission to Reson

Cameras and Strobes

Camera control computer Euler on

Camera logging CPU clock synchronized and in UTC

Check free disk space (on Euler)

Ping both cameras

Bring up camera drivers   <- ???

Start test survey logging

Both strobes firing

Both cameras logging at desired rate without dropouts

Stop test survey logging

Reson T50

Sonar logging off

Sonar Power zero

Service Mode Tab

Health status

I/O Module Tab

SVP streaming data

Getting PPS and GPS

Hardware

Normalize receive head

Confirm 1 PPS is set to rising edge

Confirm external triggering disabled

Confirm trigger output enabled

Cable length (transmit): 3 m

Detection menu

Depth Gate bounds 0.1 to 10.0, follow seafloor

Range Gate bounds: 1.0 to 10.0

Adaptive Gate bounds: 0.1 to 25.0, follow seafloor

Tracker range enabled

Tracker power, gain, absorption and spreading enabled

Tracker pulse length enabled

Coverage angle NOT enabled

Multi-detect not enabled

Main Section

Multibeam pinging at 5.0 Hz

Multibeam configured for 400 kHz

Multibeam range set as desired

Pulse length set as desired

Multibeam power set as desired

Receive gain set as desired

Adaptive gates enabled

Advanced

Beam mode equiangle

Coverage angle 170

Pulse type FM

Absorption set as desired

Spreading loss set as desired

Sound speed reasonable with "normal" filter

Recording

Path set    [D:\Data]

Verify Record Selection -> Snippets, Sidescan, Sensor Data, Other

-- and this time we ARE NOT recording compressed water column

1000, 1001, 1002, 1003, 1004, 1005, 1006, 1007, 1008,

1009, 1010, 1011, 1012, 1013, 1014, 1015, 1016, 1017,

1050, 2004, 7000, 7001, 7002, 7004, 7007, 7010, 7014,

7021, 7027, 7028, 7047, 7503, 7504, 7511, 7610, 7611,

7612, 7613

+ 7012 and 7042 for compressed water column

Confirm data storage has enough space for new data

LIDAR

Power Lidar

Temperature ok

Humidity ok

CTD updating

Scans set to desired number of across track pulses

Lidar points per pulse

Lidar power set as desired

Lidar min range

Lidar max range

Lidar bottom side IP address: 192.168.168.130 Head 1

Lidar top side IP address: 192.168.168.131 Head 2

Lidar receiving PPS

Lidar data disk space [C:\]

CTD

CTD data being logged on Euler

Delta T

DeltaT data being logged

Gemini

Gemini data being logged

VN-100

VN100 logging

General

Clocks synchronized and in proper time zone

USBL/Winfrog aiding of INS during descent

USBL/Winfrog INS aiding off when DVL lock achieved

Survey Logging

To Start

Press Record (on) in Reson Software

Begin imaging ‘oi_startall’

Turn on lasers

To Stop

Turn off lasers

End Imaging ‘oi_killall’

Press Record (off) in Reson Software

ROV Ascent

Stop survey logging if not already done

Retract strobe arms for ascent

Turn Reson power to zero

Start image conversions

Begin transferring Lidar and Sonar data

* Stop VARS recording once vehicle is back on the deck *

Network Info and Logins

Open ship IP addresses: 134.89.20.56 – 59

Digi Module

IP: 134.89.32.111

Sending updates for GPS to Kearfott on port 5050

User: root

Pswd: dvps

T50

IP: 192.168.168.132 (updated)

#192.168.168.40 (alternate IP address on LIDAR subnet)

#topside 134.89.32.40

User: reson

Pswd: reppe

LIDAR

IP: 192.168.168.130

#134.89.20.58

User: EC

Pswd: 3d

Euler

IP: 192.168.168.91

#134.89.20.75

User: ops

Pswd: rov96rov

Camera

Right: 192.168.168.135

Left:   192.168.168.136

MOB (Mobilization)

Gear to bring from lab

TCU

Laptop and power supply

Large monitor

Tote #1 (consumables + laser safety gear)

Tote #2 (extra Ethernet + serial + assorted computer cables)

Tote #3 (active gear to be used: keyboard, mouse, Digi, etc…)

Tote #4 (Kits 1-4)

Tote #5 (Kits 5-8)

Eric’s tool box

Chad’s tool box

Chad’s tool bag

ROV

Coordinate work on vehicle with DJ and Mike B

Install power adaptor can (Sled power on vehicle, AC power to LASS Pow/Int can)

Install fiber connection from LASS Pow/Int can to Ventana Science can

Install valve controller cable from LASS Valve can to Ventana Science can (Jxx ??)

DeltaT ?

Gemini ?

Aft Deck

Mount GPS antenna on top of Ventana van (near cable tray)

GPS cable is pre-routed outside in cable tray (the rest is stowed up in ceiling near the galley)

Dry Lab

* TCU will fit thru Dry Lab door with front and rear covers removed and all Ethernet cables disconnected.

TCU on deck on forward starboard area (under small shelf)

Run GPS antenna cable into Dry Lab

Cables to run to Control Room:

Serial for Octans

Serial for Articulation

Serial for

Cat6 Ethernet Cable for Euler Extender (Xmtr)

Control Room

Large monitor, keyboard, mouse, and laptop on desk on starboard side of control room (next to VARS)

Serial cable from WinFrog to port 4 of Digi    <- old method?

Euler Extender (Rcvr)

Ethernet Switch

Digi Quad Serial Server

Port 1: Octans

Port 2: Articulation

Port 3:

Port 4: