# GOB Board Stack

# Technical Manual, v0.1

Monterey Bay Aquarium Research Institute

December 18, 2024

Table of Contents

General Oceanographic Board (GOB) Stack Overview	4

GOB Computer Board	4

Internal Environmental Sensing	4

GOB Power Supply Board	4

References	5

Appendices	5

Figure 8: Revision C of the Medium Power Relay Board	7

Figure 9: Revision B of the Low Power Relay Board	8

Table 4: Medium Power Relay Board Load Channels	6

Table 5: Low Power Relay Board Load Channels	8

Appendix 1: appendix1	21

Appendix 2: appendix2	21

Appendix 3: appendix3	21

Appendix 4: appendix4	21

Appendix 5: appendix5	21

# General Oceanographic Board (GOB) Stack Overview

The core of the GOB stack consists of two boards: The Computer Board (CB) and Power Supply Board (PB). While necessary switching regulators could have been designed into the CB, maximum flexibility is gained by having a dedicated power supply board. And, if necessary, a new PB can be designed without replicating the microcontroller circuitry.

The backplane consists of two dual row headers, one 64-pin and the other, 44-pin. These carry a multitude of analog, digital, communication, and power signals. The large number of signals and power handling allows for nearly any type of add-on board to be designed and used on the GOB stack.

This document will cover the Computer Board and Power Supply Board. Each add-on board will have its own dedicated user’s guide.

# GOB Backplane

Each of the 48 GPIO pins from the RP2350B microcontroller on the CB are brought out to the backplane. Some have default pin-mappings assigned to them for functions such as serial communication and SPI chip selects. These can be reconfigured, however, depending on one’s use of the GOB stack. If an add-on board needs a GPIO pin, it is then unavailable to other boards. Care should be taken to verify GPIO pins are actually available before designing any future boards. A master list of backplane signals will be maintained in this document (appendix 1). Nine additional pins on the backplane are unassigned and have no connections to the RP2350B. These can be used for any direct board-to-board signals in the future.

| J1 |  |  |  |  | J2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bus Name |  |  | Bus Name |  | Bus Name |  |  | Bus Name |
| 3.3V | 1 | 2 | 5V |  | 5V | 1 | 2 | 3.3V |
| 3.3V | 3 | 4 | 5V |  | 5V | 3 | 4 | 3.3V |
| GND | 5 | 6 | 12V_INSTR |  | +POE_VC12 | 5 | 6 | +POE_VC45 |
| GND | 7 | 8 | 12V_INSTR |  | -POE_VC36 | 7 | 8 | -POE_VC78 |
| GND | 9 | 10 | 12V_INSTR |  | GND | 9 | 10 | GND |
| GND | 11 | 12 | 24V_INSTR |  | 12V_SYS | 11 | 12 | 24V_SYS |
| GND | 13 | 14 | 24V_INSTR |  | 12V_SYS | 13 | 14 | 24V_SYS |
| GND | 15 | 16 | 24V_INSTR |  | GPIO29 | 15 | 16 | GPIO40/ADC0 |
| GND | 17 | 18 | 24V_INSTR |  | GPIO30 | 17 | 18 | GPIO41_ADC1 |
| GND | 19 | 20 | GPIO0 |  | GPIO31 | 19 | 20 | GPIO42_ADC2 |
| GND | 21 | 22 | GPIO1 |  | GPIO32 | 21 | 22 | GPIO43_ADC3 |
| GND | 23 | 24 | GPIO2 |  | GPIO33 | 23 | 24 | GPIO44_ADC4 |
| GND | 25 | 26 | GPIO3 |  | GPIO34 | 25 | 26 | GPIO45_ADC5 |
| GND | 27 | 28 | GPIO4 |  | GPIO35 | 27 | 28 | GPIO46_ADC6 |
| GND | 29 | 30 | GPIO5 |  | GPIO36 | 29 | 30 | GPIO47_ADC7 |
| GND | 31 | 32 | GPIO6 |  | GPIO37 | 31 | 32 | Unassigned_3 |
| GND | 33 | 34 | GPIO7 |  | GPIO38 | 33 | 34 | Unassigned_4 |
| GND | 35 | 36 | GPIO8 |  | GPIO39 | 35 | 36 | Unassigned_5 |
| GND | 37 | 38 | GPIO9 |  | CB_USB_DET | 37 | 38 | Unassigned_6 |
| GND | 39 | 40 | GPIO10 |  | LB_USB_DET | 39 | 40 | Unassigned_7 |
| GND | 41 | 42 | GPIO11 |  | Unassigned_1 | 41 | 42 | Unassigned_8 |
| GND | 43 | 44 | GPIO12 |  | Unassigned_2 | 43 | 44 | Unassigned_9 |
| GND | 45 | 46 | GPIO13 |  |  |  |  |  |
| GND | 47 | 48 | GPIO14 |  |  |  |  |  |
| GND | 49 | 50 | GPIO15 |  |  |  |  |  |
| GND | 51 | 52 | GPIO22 |  |  |  |  |  |
| GND | 53 | 54 | GPIO23 |  |  |  |  |  |
| GND | 55 | 56 | GPIO24 |  |  |  |  |  |
| GND | 57 | 58 | GPIO25 |  |  |  |  |  |
| GND | 59 | 60 | GPIO26 |  |  |  |  |  |
| GND | 61 | 62 | GPIO27 |  |  |  |  |  |
| GND | 63 | 64 | GPIO28 |  |  |  |  |  |

# GOB Computer Board

### Internal Environmental Sensing

A board level environmental sensor, the BME280, is used to measure temperature, humidity, and pressure inside of the LASS housing. The TB utilizes the SPI bus to communicate with it, however, it also can be accessed via i2c.The environmental data is sent topside and is accessible through the LASS GUI. The BME280 has specific mounting and soldering instructions and should be followed diligently during the board layout and population phases.

# GOB Power Supply Board

# References

# Appendices

Appendix 1: Backplane Assignments

| J1 |  |  |  |  | J2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bus Name |  |  | Bus Name |  | Bus Name |  |  | Bus Name |
| 3.3V | 1 | 2 | 5V |  | 5V | 1 | 2 | 3.3V |
| 3.3V | 3 | 4 | 5V |  | 5V | 3 | 4 | 3.3V |
| GND | 5 | 6 | 12V_INSTR |  | +POE_VC12 | 5 | 6 | +POE_VC45 |
| GND | 7 | 8 | 12V_INSTR |  | -POE_VC36 | 7 | 8 | -POE_VC78 |
| GND | 9 | 10 | 12V_INSTR |  | GND | 9 | 10 | GND |
| GND | 11 | 12 | 24V_INSTR |  | 12V_SYS | 11 | 12 | 24V_SYS |
| GND | 13 | 14 | 24V_INSTR |  | 12V_SYS | 13 | 14 | 24V_SYS |
| GND | 15 | 16 | 24V_INSTR |  | GPIO29 | 15 | 16 | GPIO40/ADC0 |
| GND | 17 | 18 | 24V_INSTR |  | GPIO30 | 17 | 18 | GPIO41_ADC1 |
| GND | 19 | 20 | GPIO0 |  | GPIO31 | 19 | 20 | GPIO42_ADC2 |
| GND | 21 | 22 | GPIO1 |  | GPIO32 | 21 | 22 | GPIO43_ADC3 |
| GND | 23 | 24 | GPIO2 |  | GPIO33 | 23 | 24 | GPIO44_ADC4 |
| GND | 25 | 26 | GPIO3 |  | GPIO34 | 25 | 26 | GPIO45_ADC5 |
| GND | 27 | 28 | GPIO4 |  | GPIO35 | 27 | 28 | GPIO46_ADC6 |
| GND | 29 | 30 | GPIO5 |  | GPIO36 | 29 | 30 | GPIO47_ADC7 |
| GND | 31 | 32 | GPIO6 |  | GPIO37 | 31 | 32 | Unassigned_3 |
| GND | 33 | 34 | GPIO7 |  | GPIO38 | 33 | 34 | Unassigned_4 |
| GND | 35 | 36 | GPIO8 |  | GPIO39 | 35 | 36 | Unassigned_5 |
| GND | 37 | 38 | GPIO9 |  | CB_USB_DET | 37 | 38 | Unassigned_6 |
| GND | 39 | 40 | GPIO10 |  | LB_USB_DET | 39 | 40 | Unassigned_7 |
| GND | 41 | 42 | GPIO11 |  | Unassigned_1 | 41 | 42 | Unassigned_8 |
| GND | 43 | 44 | GPIO12 |  | Unassigned_2 | 43 | 44 | Unassigned_9 |
| GND | 45 | 46 | GPIO13 |  |  |  |  |  |
| GND | 47 | 48 | GPIO14 |  |  |  |  |  |
| GND | 49 | 50 | GPIO15 |  |  |  |  |  |
| GND | 51 | 52 | GPIO22 |  |  |  |  |  |
| GND | 53 | 54 | GPIO23 |  |  |  |  |  |
| GND | 55 | 56 | GPIO24 |  |  |  |  |  |
| GND | 57 | 58 | GPIO25 |  |  |  |  |  |
| GND | 59 | 60 | GPIO26 |  |  |  |  |  |
| GND | 61 | 62 | GPIO27 |  |  |  |  |  |
| GND | 63 | 64 | GPIO28 |  |  |  |  |  |

Computer Board (orange)

| J1-20 | GPIO0 | CB_UART0_TX | Spare/Debug serial port - sent to an Xcvr and connector |
| --- | --- | --- | --- |
| J1-22 | GPIO1 | CB_UART0_RX | Spare/Debug serial port - sent to an Xcvr and connector |
| J1-24 | GPIO2 | CB_I2C_SDA | i2c bus from Computer board to any add-on board |
| J1-26 | GPIO3 | CB_I2C_SCL | i2c bus from Computer board to any add-on board |
| J1-28 | GPIO4 | CB_UART1_TX | Used for inter-processor comms between Computer & LASS Boards |
| J1-30 | GPIO5 | CB_UART1_RX | Used for inter-processor comms between Computer & LASS Boards |
| J1-32 | GPIO6 | CB_RTC_CS | SPI1 *CS for Real Time Clock (RTC) on Computer Board |
| J1-60 | GPIO26 | CB_SPI1_SCK | SPI bus from Computer board to any add-on board |
| J1-62 | GPIO27 | CB_SPI1_MOSI | SPI bus from Computer board to any add-on board |
| J1-64 | GPIO28 | CB_SPI1_MISO | SPI bus from Computer board to any add-on board |
| J2-15 | GPIO29 | CB_ENVR_CS | SPI1 *CS signal for Environmental sensor on Computer Board |
| J2-17 | GPIO30 | CB_EEPROM_CS | SPI1 *CS signal for EEPROM/MAC Address chip on Computer Board |
| J2-19 | GPIO31 | CB_H2O_DET | Water Intrusion Detector Signal. L: Ok, H: H2O Present |
| J2-21 | GPIO32 | CB_HEARTBEAT | Heartbeat LED on Computer board |
| J2-27 | GPIO35 | CB_SD_DET | 3.3V level GPIO for detection of an SD Card (H: No Card, L: Card Inserted) |
| J2-29 | GPIO36 | CB_SD_CS | SPI1 *CS for SD Card Access |
| J2-37 |  | CB_USB_DET | USB detection signal from Computer board; turns off 3.3V on Power Supply Board |

Power Supply Board (red)

| J2-23 | GPIO33 | PB_ADC_MON1_CS | SPI1 *CS for octal ADC on Power Supply Board |
| --- | --- | --- | --- |
| J2-25 | GPIO34 | PB_ADC_MON2_CS | SPI1 *CS for quad ADC on Power Supply Board |

LASS System Board (yellow)

| J1-58 | GPIO25 | LIDAR_RELAY_CTRL | Coil activation (24V) for chassis mount power relay for LIDAR |
| --- | --- | --- | --- |
| J2-39 |  | LB_USB_DET | USB detection signal from LASS board; turns off 3.3V on Power Supply Board |

Medium Power Relay Board (green)

| J1-50 | GPIO15 | STROBE1_RELAY_CTRL | 3.3V level GPIO used for relay activation on Medium Power Relay Board |
| --- | --- | --- | --- |
| J1-52 | GPIO22 | STROBE2_RELAY_CTRL | 3.3V level GPIO used for relay activation on Medium Power Relay Board |
| J1-54 | GPIO23 | RESON_RELAY_CTRL | 3.3V level GPIO used for relay activation on Medium Power Relay Board |
| J1-56 | GPIO24 | NORBIT_RELAY_CTRL | 3.3V level GPIO used for relay activation on Medium Power Relay Board |

Low Power Relay Board (blue)

| J1-34 | GPIO7 | LCAM_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| --- | --- | --- | --- |
| J1-36 | GPIO8 | RCAM_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-38 | GPIO9 | ROT1_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-40 | GPIO10 | ROT2_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-42 | GPIO11 | CTD_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-44 | GPIO12 | PARO_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-46 | GPIO13 | KEAR_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |
| J1-48 | GPIO14 | VN100_RELAY_CTRL | 3.3V level GPIO used for relay activation on Low Power Relay Board |

Appendix 2: appendix2

Appendix 3: appendix3

Appendix 4: appendix4

Appendix 5: appendix5