| ###### Button control |  |
| --- | --- |
| t app button: shutter/power/wifi -P = Press, R = Release PR = Press and Release (click) | Example: t app button shutter PR |
| ###### Time between actions |  |
| sleep: time in seconds | Example: sleep 6 |
| ###### Camera Modes |  |
| t app appmode: video/photo/photo_burst (Burst) /photo_pes (Timelapse) /settings | Example: t app appmode photo_burst |
| ###### Custom text on LCD |  |
| t app fp_show 4 sleep 2 t app fp_string 'custom text here' t app fp_show 4 will show a blank screen before the text | Example: t app fp_show 4 sleep 2 t app fp_string 'i will kill you' |
| ###### LEDs Settings |  |
| t app led_count: 4/2/0 | Example: t app led_count 0 |
| ###### LED control |  |
| t app led: red_front/red_top_bottom/red_back/ {on/off} | Example: t app led red_front on |
| ###### Video Format |  |
| t app video_format: ntsc/pal | Example: t app video_format pal |
| ###### One Button Mode |  |
| t app obm: on/off | Example: t app obm on |
| ###### White Balance |  |
| t app white_balance: auto/3000/5500/6500/camraw | Example: t app white_balance 3000 |
| ###### Loop |  |
| d:\autoexec.ash REBOOT yes | Example: sleep 1 t app appmode photo sleep 120 t app button shutter PR d:\autoexec.ash REBOOT yes |
| ###### Protune |  |
| t app protune: on/off | Example: t app protune on |
| ###### Lock exposure |  |
| t app protune: on/off | Example: t app protune on |
| ###### Video Loop |  |
| t ia2 -exp lock: 1=on 0=off | Example: t ia2 -exp lock 1 |
| ###### Video+Photo |  |
| t app video_photo: off/5/10/30/60 | Example: t app video_photo 5 |
| ###### Spot Meter |  |
| t app spot_meter: on/off | Example: t app spot_meter on |
| ###### Mirror Flip |  |
| t app flip_mirror: enable=1/disable=0 | Example: t app flip_mirror 1 |
| ###### Delete a directory (such as DCIM) (works with all cameras!) |  |
| deletedir: SD card port d:\ + DCIM folder | Example: deletedir d:\DCIM |
| ###### Timelapse Settings |  |
| t app pes_settings: 0.5/1/2/5/10/20/60 | Example: t app pes_settings 5 |
| ###### Burst Settings |  |
| t app burst_settings: 3-1/5-1/10-1/10-2/30-1/30-2/30-3 | Example: t app burst_settings 30-2 |
| ###### Continuous Photo Settings |  |
| t app sps_settings: single=1/3/5/10 | Example: t app sps_settings 10 |
| ###### Photo Resoultion |  |
| t app photo_size: 12W/7M/7W/5M | Example: t app photo_size 7W |
| ###### Rotation /basic/ |  |
| t app test vin_rotate: 0/90/180/270 | Example: t app test vin_rotate 90 |
| ###### Rotation /advanced/ |  |
| t app test rec_fmt_rotate: Normal=0x01/Flip&Horizontal=0x02/ Rotate180=0x03/Flip&Vertical=0x04/Transpose=0x05/ Rotate90=0x06/Transverse=0x07/Rotate270=0x08 | Example: t app test rec_fmt_rotate 0x04 |
| ###### Key |  |
| t app key: up/up_rel/down/down_rel/left/left_rel/right/right_rel/zoomin/zoomin_rel/ zoomout/zoomout_rel/set/focus/focus_rel/shutter/shutter_rel/record/ mode/menu/power/del/format/copydir/switchosd | Example: t app key zoomin |
| ###### Default mode |  |
| t app default_mode: last/video/photo/photo_burst(Burst)/photo_pes (Timelapse) | Example: t app default_mode photo |
| ###### Auto-power off |  |
| t app shutdown_mode: manual/60/120/300 | Example: t app shutdown_mode 120 |
| ###### HERO3Plus Black Power 0ff |  |
| poweroff yes reboot yes | Example: poweroff yes reboot yes |
| ###### HERO3Plus Black Boss Boot |  |
| t app test boss_boot: enable=1/disable=0 | Example: t app test boss_boot 1 |
| ###### HERO3Plus Black SB sensor |  |
| t app sensor_sb: enable/disable | Example: t app sensor_sb enable |
| ###### HERO3Plus Black Photo Resolution |  |
| t app photo_size: 12W/10W/7M/7W/5M | Example: t app photo_size 10W |
| ###### HERO3Plus Black LCD Images |  |
| t app test fp_display: splash_screen.bmp/video.bmp/photo.bmp/one_button.bmp/ blank.bmp/background.bmp | Example: t app test fp_display video.bmp |
| ###### HERO3Plus Black Rotation |  |
| t app test vin_rotate: 0/90/180/270/flip_h/flip_v | Example: t app test vin_rotate 270 |
| ###### HERO3Plus Black HDMI Settings (PES) |  |
| t app hdmi_set_pes: none/10%/20%/30% | Example: t app hdmi_set_pes 10% |
| ###### HERO3Plus Black HDMI Settings (PIV) |  |
| t app hdmi_set_pib: 0mA/0.6mA/1.2mA/1.8mA | Example: t app hdmi_set_piv 1.3mA |
| ###### HERO3Plus button status v2.0 |  |
| t app btn_status: enable(1) / disable(0) | Example: t app btn_status 1 |
| ###### HERO3Plus 3D Smarty Master v2.0 |  |
| t app master_set_for_smarty: start(1) / stop(0) | Example: t app master_set_for_smarty 1 |
| ###### HERO3Plus 3D Smarty Slave v2.0 |  |
| t app slave_set_for_smarty: start(1) / stop(0) | Example: t app slave_set_for_smarty 1 |
| ###### HERO3Plus Heart beat for 3D v2.0 |  |
| t app heart_beat_3d: on(1) / off(0) | Example: t app heart_beat_3d 1 |
| ###### HERO3Plus 3D Video Start/Stop recording (?) v2.0 |  |
| t app test bacpac3d I2C SY: START_RECORD / STOP_RECORD | Example: t app test bacpac3d I2C SY START RECORD |
| ###### HERO3Plus WB Gain Set |  |
| t ia2 -awb: [set][][][] | Example: t ia2 -awb [788][][][] |
| ###### HERO3Plus ISO Still Set |  |
| t ia2 -ae still_iso: [iso] | Example: t ia2 -ae still_iso 1200 |
| ###### HERO3Plus Shutter Still Set |  |
| t ia2 -ae still_shutter: [shutter] | Example: t ia2 -ae still_shutter 126 |
| ###### HERO3Plus ISO Still Set |  |
| t ia2 -ae still_iris: [iris] | Example: t ia2 -ae still_iris 45 |
| The commands tagged with v2.0 ONLY work with GoPro HERO3 Plus Black cameras with the firmware 2.0 of 6 March 2014 t app, t ia2, deletedir work for all gopro models, t cal works only for hero3 black/plus black/plus silver |  |
| ###### Video/Photo Digital Filters For HERO3 Black / HERO3+ Black / HERO3+ Silver |  |
| t cal -de: 1/2/3/4/5/6/7/8/9/10 - 1: Art effect - 2: Sepia Effect - 3: Negative Effect - 4: Black White Effect - 5: Vivid Effect - 6: 70s Film Effect - 7: Punk Effect - 8: Popart Effect - 9: Wedding Effect - 10:Pinhole Effect | Example: t cal -de 4 |