SeeStar Arduino Pro-Mini Controller Shield revB Design Review Notes

Chad Kecy

07-16-2015

Overview

The controller shield resides in the camera module and interfaces the Arduino Pro Mini CPU board. It provides drive circuitry necessary for the GoPro camera as well as the external battery and LED modules. The controller shield functionality includes:

1. Providing power to the Arduino Pro-Mini

2. Applying power to and turning on the GoPro Hero 3/3+ camera

3. Applying power and trigger to the external LED module

4. Providing a real time clock for accurate time-keeping

5. Monitoring the voltage level and current usage of the external battery module

6. Providing an external serial connection for operation or debugging

This is an open source circuit board with simplicity, ease of build, and cost taken into consideration during the design process.

Signal Functions

All analog and digital signals are processed through the Arduino Pro-Mini. The analog signals have 10 bits of resolution and the digital signals can source or sink up to 40ma.

Analog Inputs

BATT_CURR

This analog voltage is generated by the current sense amplifier, U2. The MAX4372F has an internal gain of 50. With a maximum input current of 3A and a 20milli-ohm sense resistor, the current sense amplifier will output 0-3 volts over the expected current range of 0-3 amps. The signal is fed to analog input A1.

BATT_VOLT

This analog voltage is generated by a voltage divider/buffer, U3 and is a scaled representation of the battery voltage. The scaling is Actual Battery Voltage = 6.49 * BATT_VOLT. This signal is fed to analog input A0.

Digital Outputs

LED_ON

This digital output signal is used to trigger the LED module. The active high signal forward biases an NPN transistor (Q5) which provides a path to ground for an external switch. This signal is sourced from digital line D4.

UART_SHDN

This digital output signal is used to put the serial transceiver, U4, into a shutdown mode. UART_SHDN is an active low signal. While in shutdown mode, the MAX3311 draws only 1uA of current. This signal is sourced from digital line D6.

LED_POWER

This digital output signal is used to provide battery voltage to the LED module. The active high signal turns on an N-channel mosfet (Q3A) which in turn activates a P-channel mosfet (Q3B) which passes the battery voltage to the LED module. This signal is sourced from digital line D7.

CAM_ON

This digital output signal is used to trigger the shutter button on the GoPro Hero 3/3+ camera. The active high signal forward biases an NPN transistor (Q4) which provides a path to ground to the internal pull up resistor in the GoPro. The duration of this pulse needs to be 500msec to 1000msec. This signal is sourced by digital line D8.

CAM_POWER

This digital output signal is used to provide battery voltage to GoPro Battery Eliminator. The active high signal turns on an N-channel mosfet (Q2A) which in turn activates a P-channel mosfet (Q2B) which passes the battery voltage to the GoPro. This signal is sourced from digital line D9.

RTC_SS

This digital output signal is used to provide the SPI chip select signal for the DS3234 RTC as the Arduino Pro-Mini does not have a dedicated SPI chip select line. RTC_SS is an active low signal. This signal is sourced by digital line D10.

Digital Inputs

RTC_INT

This digital signal is a programmable active low interrupt output from the DS3234 Real Time Clock (RTC). The interrupt is enabled when either time-keeping alarm is activated. This signal is fed to digital line D2.

RTC_RESET

This active low digital signal indicates a power fail condition has occurred on the DS3234 RTC. The SPI signals are inhibited during a power fail but the TCXO and RTC continue to operate provide there is an external watch battery installed. This signal is fed to digital line D5.

Connectors/Jumpers

J1

This 4-pin Phoenix wire-to-board connector is used to bring battery voltage onto the shield, with each pin rated for 6A.

J2

This 2-pin female header is used to pass the CAM_ON signal to the GoPro 30-pin bus.

J3

This 2-pin Phoenix wire-to-board connector is used to pass battery voltage to the GoPro Battery Eliminator, with each pin rated for 6A.

J4

This 3-pin female header is used to interface an external serial connection.

J5

This 4-pin Phoenix wire-to-board connector is used to pass battery voltage to the LED module, with each pin rated for 6A.

J6  (Jumper)

This jumper enables the three status LEDs on the shield by providing a common ground to all three LEDs. By removing the jumper, the current usage of the board is reduced by 30ma.

J7

This 12-pin female header is used to interface the Arduino Pro-Mini.

J8

This 12-pin female header is used to interface the Arduino Pro-Mini.

J9

This 2-pin female header is used to pass the LED_ON (trigger) signal to the LED module.

J10  (Jumper)

This jumper disables the onboard serial transceiver, U4. This needs to be jumpered while the FTDI cable is installed during programming in order to prevent a transmit line contention.

Status LEDs

D2

This Blue LED indicates the GoPro camera has battery voltage applied to it.

D3

This Red LED indicates the LED module has battery voltage applied to it.

D4

This Green LED indicates the shield board has logic power turned on.

| SeeStar Arduino Pro-Mini Controller Shield revB Bill of Materials |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Ref | Value | Manufacturer | Manufacturer PN | Description | Cost |
| 1 | BATT |  | Panasonic-BCG | CR1220 | Battery Lithium Coin 3V 12.5mm | $0.98 |
| 2 | BT1 | S8411-45R | Harwin Inc | S8411-45R | BATTERY HOLDER 12.0MM COIN SMD | $1.08 |
| 3 | C1 | 0.1uF | TDK Corporation | C1608X7R1H104K | CAP CER 0.1UF 50V 10% X7R 0603 | $0.24 |
| 4 | C2 | 47uF | United Chemi-con | EMVE500ADA470MF80G | CAP ALUM 47UF 20% 50V SMD | $0.54 |
| 5 | C3 | 68uF | AVX Corporation | TAJD686K016RNJ | CAP TANTALUM 68UF 16V 10% SMD | $1.60 |
| 6 | C4 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 7 | C5 | 0.1uF | TDK Corporation | C1608X7R1H104K | CAP CER 0.1UF 50V 10% X7R 0603 | $0.24 |
| 8 | C6 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 9 | C7 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 10 | C8 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 11 | C9 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 12 | C10 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 | $0.11 |
| 13 | D1 | B340A-13-F | Diodes Inc | B340A-13-F | DIODE SCHOTTKY 3A 40V SMA | $0.47 |
| 14 | D2 | LTST-C150TBKT | Lite-On Inc | LTST-C150TBKT | LED 468NM BLUE CLEAR 1206 SMD | $0.50 |
| 15 | D3 | LTST-C150KRKT | Lite-On Inc | LTST-C150KRKT | LED SUPER RED CLEAR 1206 SMD | $0.39 |
| 16 | D4 | LTST-C150KGKT | Lite-On Inc | LTST-C150KGKT | LED GREEN CLEAR 1206 SMD | $0.39 |
| 17 | F1 | 0154004.DRT | Littelfuse Inc | 0154004.DRT | FUSE SLOW 125VAC, 125VDC 4A SMD | $3.14 |
| 18 | J1 | 1725672 | Phoenix Contact | 1725672 | CONN TERM BLOCK 2.54MM 4POS | $2.94 |
| 19 | J2 | SSW-102-01-T-S | Samtec Inc | SSW-102-01-T-S | CONN RCPT .100" 2POS SNGL TIN | $0.86 |
| 20 | J3 | 1725656 | Phoenix Contact | 1725656 | CONN TERM BLOCK 2.54MM 2POS | $1.46 |
| 21 | J4 | SSW-103-01-T-S | Samtec Inc | SSW-103-01-T-S | CONN RCPT .100" 3POS SNGL TIN | $0.93 |
| 22 | J5 | 1725672 | Phoenix Contact | 1725672 | CONN TERM BLOCK 2.54MM 4POS | $2.94 |
| 23 | J6 | TSW-102-17-T-S | Samtec Inc | TSW-102-17-T-S | CONN HEADER 2POS .100" SNGL TIN | $0.57 |
| 24 | J7 | SSW-112-01-T-S | Samtec Inc | SSW-112-01-T-S | CONN RCPT .100" 12POS SNGL TIN | $1.59 |
| 25 | J8 | SSW-112-01-T-S | Samtec Inc | SSW-112-01-T-S | CONN RCPT .100" 12POS SNGL TIN | $1.59 |
| 26 | J9 | SSW-102-01-T-S | Samtec Inc | SSW-102-01-T-S | CONN RCPT .100" 2POS SNGL TIN | $0.86 |
| 27 | J10 | TSW-102-17-T-S | Samtec Inc | TSW-102-17-T-S | CONN HEADER 2POS .100" SNGL TIN | $0.57 |
| 28 | L1 | 33uH | Bourns Inc. | SRN6045-330M | FIXED IND 33UH 1.4A 188 MOHM SMD | $0.43 |
| 29 | Q1 | SI2369DS-T1-GE3 | Vishay Siliconix | SI2369DS-T1-GE3 | MOSFET P-CH 30V 7.6A TO-236 | $0.44 |
| 30 | Q2 | SI6562CDQ-T1-GE3 | Vishay Siliconix | SI6562CDQ-T1-GE3 | MOSFET N/P-CH 20V 6.7A 8-TSSOP | $1.06 |
| 31 | Q3 | SI6562CDQ-T1-GE3 | Vishay Siliconix | SI6562CDQ-T1-GE3 | MOSFET N/P-CH 20V 6.7A 8-TSSOP | $1.06 |
| 32 | Q4 | MMBT3904 | Fairchild | MMBT3904 | TRANSISTOR GP NPN AMP SOT-23 | $0.16 |
| 33 | Q5 | MMBT3904 | Fairchild | MMBT3904 | TRANSISTOR GP NPN AMP SOT-23 | $0.16 |
| 34 | R1 | 0.02 | Vishay Dale | WSL1206R0200FEA | RES .02 OHM 1/4W 1% 1206 SMD | $0.92 |
| 35 | R2 | 1.00M | Vishay | CRCW06031M00FKEA | RES 1M OHM 1/10W 1% 0603 SMD | $0.81 |
| 36 | R3 | 121.0K | Vishay | CRCW0603121KFKEA | RES 121K OHM 1/10W 1% 0603 SMD | $0.81 |
| 37 | R4 | 22.1K | Vishay | CRCW060322K1FKEA | RES 22.1K OHM 1/10W 1% 0603 SMD | $0.81 |
| 38 | R5 | 383.0K | Vishay | CRCW0603383KFKEA | RES 383K OHM 1/10W 1% 0603 SMD | $0.81 |
| 39 | R6 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 40 | R7 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 41 | R8 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 42 | R9 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 43 | R10 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 44 | R11 | 1.69K | Vishay | CRCW06031K69FKEA | RES 1.69K OHM 1/10W 1% 0603 SMD | $0.81 |
| 45 | R12 | 1.69K | Vishay | CRCW06031K69FKEA | RES 1.69K OHM 1/10W 1% 0603 SMD | $0.81 |
| 46 | R13 | 294 | Vishay | CRCW0603294RFKEA | RES 294 OHM 1/10W 1% 0603 SMD | $0.81 |
| 47 | R14 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 48 | R15 | 10.0K | Vishay | CRCW060310K0FKEA | RES 10.0K OHM 1/10W 1% 0603 SMD | $0.81 |
| 49 | R16 | 0.0 | Vishay/Dale | CRCW06030000Z0EA | RES 0.0 OHM 1/10W 0603 SMD | $0.81 |
| 50 | R17 | 0.0 | Vishay/Dale | CRCW06030000Z0EA | RES 0.0 OHM 1/10W 0603 SMD | $0.81 |
| 51 | R18 | 10.0K | Vishay | CRCW060310K0FKEA | RES 10.0K OHM 1/10W 1% 0603 SMD | $0.81 |
| 52 | R19 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD | $0.81 |
| 53 | U1 | MAX5035BASA+ | Maxim Integrated | MAX5035BASA+ | IC REG BUCK 5V 1A 8SOIC | $4.41 |
| 54 | U2 | MAX4372FEUK+T | Maxim Integrated | MAX4372FEUK+T | IC AMP GAIN +50V/V PREC SOT23-5 | $2.41 |
| 55 | U3 | AD8603AUJZ | Analog Devices Inc | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 | $1.79 |
| 56 | U4 | MAX3311CUB+ | Maxim Integrated | MAX3311CUB+ | IC TXRX RS232 460KBPS 10-UMAX | $1.87 |
| 57 | U5 | DS3234S# | Maxim Integrated | DS3234S# | IC RTC W/TCXO 20-SOIC | $8.92 |
|  |  |  |  |  |  | $62.75 |