FEA of 9” acrylic window for SCPI light

By Francois Cazenave – January 11th 2013.

Using solidworks simulation, and “Under Pressure”, the deflection and failure depth of the window were calculated:

- Thickness=0.5”

- OD= 4.5”

- ID of supporting tube = 3.826”

- Tensile strength of acrylic = 8000 PSI

The window fails at about 450 PSI, or 300m.

So, a 0.5” window should not be used at depth exceeding 250 m.

Deflection at 250m = 0.065”

A 0.625” thick window is recommend for a depth of 300m (failure depth 450m).

At 300m, a 0.625” thick window deflects about 0.045”.

Analysis of aluminum housing:

The aluminum housing body fails at about 4500m.