Wired remote control of the Sony A6300 camera.

Chad Kecy

02-02-2017

Refer to the Sony A6300 Camera Interface Board schematic

- Set a DC power supply to 8.0Vdc, current limit of 1amp. (The input voltage range of the Sony A6300 is 7.2Vdc to 8.4Vdc). Set output to OFF.

- Plug the dummy battery into the A6300.

- Connect the red & black wires between the dummy battery and the DC power supply.

- Plug the test cable into the A6300 in the Multi connector.

- At the output end of the test cable there are 5 wires:

Orange: Power On and Off

Violet: Shutter Focus

White: Shutter Release

Grey: Video Record and Stop

Black: Camera Ground (Return)

- Connect the black wire to a PLC return

- Connect the orange, violet, white, and grey wires to low voltage DC, open collector outputs.

- Turn the On/Off knob on the A6300 to ON.

- Turn on the DC power supply.

- The camera should power up.

There are five functions available:

- Turn camera on

- Turn camera off

- Begin video recording

- Stop video recording

- Take a still image

All functions are initiated by pulling the appropriate line low for a specific amount of time. I have not accurately measured the exact time needed for each, but I have made the following observations:

- To power the camera off, pull Orange low for very short time, maybe 100msec.

- To power the camera on, pull Orange low for 500-1000msec.

- To begin video recording, pull Grey low for 500-1000msec.

- To stop video recording, pull Grey low for ~100msec.

- Taking still images is a two-step process:

  - Pull Violet low for 500-msec.

  - While Violet is still low, pull White low for ~100msec.