SeeStar III Sensor Module Notes

Sensor Module

This is an optional module which is used to interface various oceanographic instruments. It will receive power and serial communication from the master camera housing. The Sensor Module will be time synchronized with the master camera housing and will save the instrument data locally with a common time stamp. Most oceanographic instruments communicate via serial but some use analog outputs (voltage or current) or even a variable frequency signal. The interface board will need to have a variety of available inputs to accommodate a wide (but not infinite) set of instruments.

Idea – The temperature and depth sensors will be standardized parts and will always be attached to the Sensor Module (can easily find 5V analog versions of both). The four main instrument ports will be user settable to either serial or analog protocol (jumpers). Probably will be limited to a maximum of three serial instruments.

Interface Board – Potentially an Arduino Mega 2560 or Arduino DUE. Both have four serial UARTs, but the DUE is a more advanced, higher power board (may be overkill).

Arduino Mega 2560

| Microcontroller | ATmega2560 |
| --- | --- |
| Operating Voltage | 5V |
| Input Voltage (recommended) | 7-12V |
| Input Voltage (limit) | 6-20V |
| Digital I/O Pins | 54 (of which 15 provide PWM output) |
| Analog Input Pins | 16 |
| DC Current per I/O Pin | 20 mA |
| DC Current for 3.3V Pin | 50 mA |
| Flash Memory | 256 KB of which 8 KB used by bootloader |
| SRAM | 8 KB |
| EEPROM | 4 KB |
| Clock Speed | 16 MHz |
| Length | 101.52 mm |
| Width | 53.3 mm |
| Weight | 37 g |

Arduino DUE

| Microcontroller |  | AT91SAM3X8E |
| --- | --- | --- |
| Operating Voltage |  | 3.3V |
| Input Voltage (recommended) |  | 7-12V |
| Input Voltage (limits) |  | 6-16V |
| Digital I/O Pins |  | 54 (of which 12 provide PWM output) |
| Analog Input Pins |  | 12 |
| Analog Outputs Pins |  | 2 (DAC) |
| Total DC Output Current on all I/O lines |  | 130 mA |
| DC Current for 3.3V Pin |  | 800 mA |
| DC Current for 5V Pin |  | 800 mA |
| Flash Memory |  | 512 KB all available for the user applications |
| SRAM |  | 96 KB (two banks: 64KB and 32KB) |
| Clock Speed |  | 84 MHz |
| Length |  | 101.52 mm |
| Width |  | 53.3 mm |
| Weight |  | 36 g |

MicroSD Shield

Sparkfun sells a very inexpensive (and easy to interface) MicroSD shield.

Custom Shield

This will be primarily for power control to any/all of the attached instruments. This can be done with surface mount MOSFETs.

Do we need to monitor current consumption to the individual instruments?

DC-DC Converter

Will use battery volts as input

Will generate logic power (Watts TBD)

Will generate instrument power (Watts TBD) [Limit to 12Vdc/5Vdc instruments]