O-rings for SeeStar deep. Created 8/19/15 by Francois

All o-rings should be EPDM or buna-N, 70 durometer and should be very sparingly lubricated with o-ring grease

Camera housing:

1 endcap o-ring: 2-039

1 port o-ring: 2-023

2 connector o-rings: 2-014

Battery housing:

2 endcap o-rings: 2-147

1 connector o-rings: 2-014

Light housing

1 or two endcap or rings: 2-147

1 port o-ring: 2-033

1 connector o-ring: 2-014

o-ring list for SeeStar 1 (PVC housings):

Camera housing:

2 endcap o-rings: 2-151

2 connector o-rings: 2-014

Battery housing:

2 endcap o-rings: 2-151

1 connector o-ring: 2-014

Light housing:

2 endcap o-rings: 2-151

1 connector o-ring: 2-014