Under Pressure analysis of SeeStar 3 deep housing:

Housing Tube:

ID = 2.682”	
OD = 2.900”
Length = 8.5”

Thin wall buckling at 1462m
Shell failure at 2157m

Area of end face = 0.335 in2
Area of endcap = (2.9/2)2 x pi = 6.6 in2

Failure of end face occurs at P = at 40000PSI x 0.335 in2 / 6.6in2 = 2028 PSI (1360m)

Endcap with connector holes:

We ignore the connector hole, but include the center drain hole.

Tangential moment failure at 1328m.

Endcap with port hole:

We ignore the 1/8” thick area.

Tangential moment failure at 1278m

Acrylic port

OD = 1.25”. thickness = 0.323”

First, let’s assume the support ring is perfectly rigid, so free diameter = 0.76”

Radial stress failure at 3137m

Second, let’s assume the 1/8” section of the support ring provides not support, so free diameter = 1.25”

Radial stress failure = 1165m

We can safety rate this housing to 1000m.

Likely failure depth is about 1300m.