# SeeStar bill of Materials and assembly instructions. Draft

By Francois Cazenave, Created  09/12/2014

# General description of mechanical design and documentation

The mechanical design of SeeStar has been optimized for low cost, and ease of fabrication. The modules share similar or identical components.

The paragraphs below include a brief description and the bill a materials for each module.  The bills of materials below show all the components necessary for assembling the modules. Some components are commercial of the shelf (COTS), while others are custom. All custom component have drawings posted on the repository.

For electrical components, such as PCBs, connectors and buses, refer to the electrical design documentation.

Solidworks 3D models and mechanical drawings are posted on the repository. 3D models are also included in edrawing format and can be viewed using the free software Edrawing (downloadable at http://www.edrawingsviewer.com/)

A PDF version of the mechanical drawings is also included in the repository.

# Camera module

The camera module contains a Gopro Hero3 or Hero3+, the controller board and various connectors.

The mechanical design is very simple. The body of the housing is made of schedule 80 pipe, cut to length and with o-ring grooves machined on both ends. The clear port  is made of 5/8” cast acrylic sheet, and the end cap is made of 5/8” delrin sheet. The end cap and the clear port are held in position using stainless steel tie rods. Inside the housing, all the components are mounted on a sheet of aluminum, itself attached to the endcap. The endcap has two 8-pin micro series subconn connectors and an optional USB port. It also has three optional ¼-20 threaded holes for mounting the housing.

Assembly drawing and BOM:  SeeStar_Camera_Housing.PDF

| Item Number | Part Name | Description | Quantity | Custom or COTS |
| --- | --- | --- | --- | --- |
| 1 | SeeStar_Camera_Housing_Tube | Tube for battery housing, 3" sch 80 PVC pipe | 1 | Custom |
| 2 | SeeStar_Front_End_Cap | Acrylic window for Light, battery and camera modules | 1 | Custom |
| 3 | SeeStar_Camera_Rear_End_Cap | Rear end cap for camera housing | 1 | Custom |
| 4 | SeeStar_Wedge_Bracket | Optional bracket for mounting camera or light at 45 deg | 1 | Custom |
| 5 | Subconn_MCBH8M | SubConn micro series 8 pin male connector | 1 | COTS |
| 6 | Subconn_MCBH8F | SubConn Micro Series 8 pin Female Connector | 1 | COTS |
| 7 | SeeStar_Control_Board_V2 | Main control PCB | 1 | Custom |
| 8 | 10020898tbrb_scpiakinsadaptor_solid-1 | Camera adapter board | 1 | Custom |
| 9 | SAE_#5_Plug | Size 5, morb SAE plug, stainless steel | 1 | COTS |
| 10 | USB_Connector_Amphenol_MUSB-B551-04 | Optional USB connector, Amphenol Part #MUSB-B551-04 | 1 | COTS |
| 11 | Standoff .125 #4 | 1/8" nylon washer for #4 screw | 4 | COTS |
| 12 | Camera_Housing_Tie_Rods | 6.75" long 8-32 threaded rod, St. steel | 4 | Custom |
| 13 | GoPro_Hero_3 | Gopro Hero 3 camera | 1 | COTS |
| 14 | 0.25-20x1.75_screw | 1/4"-20 x 1.75" socket head cap screw | 3 | COTS |
| 15 | Pan_Head_Screw-6-32x0.375 | 6-32 x 3/8" pan head screw | 5 | COTS |
| 16 | Pan_Head_Screw-4-40x0.375 | Pan head screw, 4-40 x 3/8" | 4 | COTS |
| 17 | Standoff_1x0.25x6-32 | 1" length, 1/4" diameter, 6/32" thread standoff | 3 | COTS |
| 18 | Acorn_Nut_8-32 | 8-32 acorn nut, stainless steel | 4 | COTS |
| 19 | Flat_Washer_#8 | Size 8 flat washer, stainless steel | 4 | COTS |
| 20 | Camera_And_Board_Support | Camera and PCB support plate | 1 | Custom |
| 21 | o-ring-151 | 2-151 Buna-N o-ring | 2 | COTS |

# LED module

The LED housing is made of an aluminum or delrin body, a delrin endcap with one 8-pin subconn connector and an acrylic window.

The body of the LED can be made of anodized aluminum and delrin, depending of the application. If the housing is made of delrin, the light can only be left on for a few seconds (see design notes for more details). Therefore, for video application where the light must be left on for more than 10 seconds, use an aluminum body.

Assembly drawing and BOM: SeeStar_LED_Housing.PDF

| Item Number | Part Name | Description | Quantity | Custom or COTS |
| --- | --- | --- | --- | --- |
| 1 | CA12066_Emily-O_XM-L_LED_Lens | LED lens | 5 | COTS |
| 2 | SeeStar_Front_End_Cap | Acrylic window for Light, battery and camera modules | 1 | Custom |
| 3 | Subconn_MCBH8M | SubConn micro deries 8 pin male connector | 1 | COTS |
| 4 | SeeStar_LED_Housing_Tube | Housing barrel 3" nominal 5 LED size | 1 | Custom |
| 5 | SeeStar_LED_5_PCB | LED PCB board for 3" nominal housing with 5 Cree XML series LED | 1 | Custom |
| 6 | SeeStar_LED_Driver_PCB | LED driver electronics PCB | 1 | Custom |
| 7 | o-ring-151 | 2-151 Buna-N o-ring | 2 | COTS |
| 8 | SeeStar_LED_Rear_End_Cap | Rear end cap for LED and battery housings | 1 | Custom |
| 9 | Flat_Washer_#8 | Size 8 flat washer, stainless steel | 1 | COTS |
| 10 | 8-32x3inch_Socket_Head_Screw | #8-32 x 3 inch socket head cap screw 316 | 1 | COTS |
| 11 | 6-32x0.25inch_Binder_Head_Screw | #6-32x1/4" binder head machine screw 316 | 2 | COTS |
| 12 | 6-Belleville_Washer | #6 Belleville lock washer | 2 | COTS |

# Battery Module

The battery module is very similar to the camera module. The housing tube is longer (6” for the NiMH battery pack, and 10” for the lithium battery pack). The delrin end cap is the same as the one use on the LED housing and has one 8-pin subconn connector. The acrylic endcap is same as for the other modules.

Inside the housing the battery is held in place simply by a piece of soft foam wrapped around it and creating friction with the inside of the housing.

Assembly drawing and BOM: SeeStar_Battery_Housing.PDF

| Item Number | Part Name | Description | Quantity | Custom or COTS |
| --- | --- | --- | --- | --- |
| 1 | Subconn_MCBH8F | SubConn Micro Series 8 pin Female Connector | 1 | COTS |
| 2 | SeeStar_Front_End_Cap | Acrylic window for Light, battery and camera modules | 1 | Custom |
| 3 | Flat_Washer_#8 | Size 8 flat washer, stainless steel | 4 | COTS |
| 4 | Battery_Pack_Ni-MH | 19.2V-4.5Ah - Battery Specialties 10020893SCH-3-C | 1 | Custom |
| 5 | Battery_Housing_Tube | Tube for battery housing, 3" sch. 80 PVC Pipe | 1 | Custom |
| 6 | Battery_Housing_Tie_Rod | 7.5" long 8-32 all thread, stainless steel. | 4 | Custom |
| 7 | o-ring-151 | 2-151 Buna-N o-ring | 2 | COTS |
| 8 | Acorn_Nut_8-32 | 8-32 acorn nut, stainless steel | 4 | COTS |
| 9 | SeeStar_LED_Rear_End_Cap | Rear end cap for LED and battery housings | 1 | Custom |