SeeStar II Battery Module

Technical Manual, v0.1

Chad Kecy

Monterey Bay Aquarium Research Institute

August 14, 2015

Table of Contents

Overview	5

System Level Description	5

NiMH Cells	6

Polyfuse	6

Charging the Battery Module	7

Discharge Profile	7

Assembly Instructions	7

Appendices	8

Figure 1: Picture of the SeeStar II Imaging System	5

Figure 2: SeeStar II Battery Module Block Diagram	6

Figure 3: A single HHR450A cell	6

Table 1: Polyfuse Electrical Characteristics	8

Appendix 1: Electrical Specifications	8

Appendix 2: Mechanical Specifications	8

Appendix 3: Battery Module External Connector Pinout	8

Appendix 4: Battery Housing Bill of Material	10

# Overview

SeeStar II is an autonomous and modular imaging system (Fig 1). It is programmed prior to deployment and can be left unattended for months at a time. SeeStar II consists of three individual modules: the camera, battery, and LED. The battery module contains the 12-cell battery and polyfuse.

Figure 1: Picture of the SeeStar II Imaging System

# System Level Description

The battery module is the energy source for the SeeStar Imaging System. The SeeStar II battery module contains 12 rechargeable NiMH cells for a nominal bus voltage of 14.4Vdc.

Figure 2: SeeStar II Battery Module Block Diagram

[Include a picture of the battery + housing]

# NiMH Cells

The standard battery module contains 12 Panasonic HHR450A rechargeable Nickel Metal Hydride (NiMH) cells in series. The HHR450A is an L-Fat A size cell with a diameter of 18.2mm and height of 67mm. The battery has a capacity of 4500mAH and a nominal bus voltage of 14.4Vdc.

Figure 3: A single HHR450A cell

# Polyfuse

The battery module contains an inline resettable fuse in the event of a catastrophic failure. The part used is a TE Connectivity SRP350F PolySwitch PTC Device.

|  | IHOLD | ITRIP | VMAX | IMAX | PD MAX | Max Time-to-Trip |  | RMIN | RMAX | R1MAX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part # | (amps) | (amps) | (Vdc) | (amps) | (watts) | (amps) | (seconds) | (ohms) | (ohms) | (ohms) |
| SRP120F | 1.20 | 2.70 | 15 | 100 | 1.2 | 6.00 | 5.0 | 0.085 | 0.160 | 0.220 |
| SRP175F | 1.75 | 3.80 | 15 | 100 | 1.5 | 8.75 | 5.0 | 0.050 | 0.090 | 0.120 |
| SRP200F | 2.00 | 4.40 | 30 | 100 | 1.9 | 10.00 | 4.0 | 0.030 | 0.060 | 0.100 |
| SRP350F | 3.50 | 6.30 | 30 | 100 | 2.5 | 20.00 | 3.0 | 0.017 | 0.031 | 0.050 |
| SRP420F | 4.20 | 7.60 | 30 | 100 | 2.9 | 20.00 | 6.0 | 0.012 | 0.024 | 0.040 |

Table 1: Polyfuse Electrical Characteristocs

# Charging the Battery Module

The battery module can either be charged assembled or disassembled. In either case, it is best to utilize a trickle charging method to extend cell life and reduce excess heat generation. As the fully charged battery voltage can go as high as 16.8Vdc, the DC charging source should be set to 18.0Vdc with a current limit of 450mA – this level will take approximately 16 hours to fully charge the battery module.

# Discharge Profile

[Add in plot of discharge curve once batteries arrive]

# Assembly Instructions

[O-Ring care, installation of Subconn connector, attaching wires to TB1, wrapping battery, sealing the housing]

# Appendices

| Output Voltage | 9-15Vdc |
| --- | --- |
| Current Limit | 3.5 Amps |

Appendix 1: Electrical Specifications

| Length | Cm |
| --- | --- |
| Width | Cm |
| Height | Cm |
| Operating Temperature |  |
| Min | degC |
| Max | degC |
| Operating Depth | 1500m |

Appendix 2: Mechanical Specifications

| Battery Module J1 (To Camera Module) |  |  |  |
| --- | --- | --- | --- |
| Pin # | Signal Name | Description | Destination |
| 1 | BATT_POS | Power Output to Camera Module | TB1-1a |
| 2 | BATT_POS | Power Output to Camera Module | TB1-2a |
| 3 | BATT_NEG | Power Return to Camera Module | TB1-3a |
| 4 | BATT_NEG | Power Return to Camera Module | TB1-4a |
| 5 | COMM_A | Serial TXD Out from Camera Module | N/C |
| 6 | COMM_B | Serial RXD In to Camera Module | N/C |
| 7 | TRIG_OUT | Trigger Output from Camera Module | N/C |
| 8 | TRIG_IN | Trigger Input to Camera Module | N/C |

Appendix 3: Battery Module External Connector Pinout

| SeeStar Battery Housing Bill of Materials |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item # | Quan | Part | Manufacturer | Manufacturer PN# | Description | Distributor | Cost |
| 1 | 1 | Battery Pack | Battery Specialties | 11021184 | 12-Cell NiMH Battery Pack | Battery Specialties | $112 |
| 2 | 1 | Terminal Strip, 4-Pos | TE Connectivity | 1776293-4 | 4-Position Terminal Strip | Digikey | $3 |
| 3 | 1 | External Connector, Female | Subconn | MCBH8F | 8-Pin Female External Connector | Ocean Innovations | $110 |
| 4 | 1 | Underwater Housing | Groupbinc | TBD | Cylindrical Housing | Groupbinc | $400 |

Appendix 4: Battery Housing Bill of Material