SeeStar II Arduino Controller Shield

Hardware Interface Document, v0.2

Monterey Bay Aquarium Research Institute

December 31, 2015

Table of Contents

Overview	5

System Level Description	5

Basic Operation	8

Initial Board Power Up and Testing	8

1.	Selection of 3.3V or 5V Arduino	8

2.	Fuse Verification	8

3.	LED Functionality	8

4.	Real Time Clock Functionality	8

5.	Logic Power Verification	8

Functional Descriptions	9

Arduino Pro-Mini Interface	9

Battery Monitoring and Input Protection	10

Logic Power Generation	10

GoPro Hero Power and On/Off Control	11

LED Power and Trigger	11

Real Time Clock Control	11

Serial Interface	11

Board Jumper Options	12

DC Power LEDs	12

Serial Communication Source	12

Board Level Signal Descriptions	12

Analog Input Signals	12

SPI Bus Signals	12

Digital Input Signals	13

Digital Output Signals	13

Serial Signals	13

Appendices	14

Table 1: Arduino Pro-Mini JP7 Signals	9

Table 2: Arduino Pro-Mini JP6 Signals	9

Table 3: Board Level LEDs and Functions	12

Figure 1: Picture of the SeeStar II Imaging System	5

Figure 2: Block Diagram of SeeStar II Arduino Controller Shield	6

Figure 3: Top view of controller pcb	7

Figure 4: Rear view of controller pcb	7

Appendix 1: Electrical Specifications	14

Appendix 2: Mechanical Specifications	14

Appendix 3: Controller Shield Bill of Materials	15

# Overview

SeeStar II is an autonomous and modular imaging system (Fig 1). It is programmed prior to deployment and can be left unattended for months at a time. It is an open source project and is distributed under a Creative Commons Attribution-ShareAlike 4.0 License. SeeStar II consists of three individual modules: the camera, battery, and LED. The camera module contains the Arduino Controller Shield which is responsible for triggering the camera and activating the lights.

Figure 1: Picture of the SeeStar II Imaging System

# System Level Description

The Controller Shield acts as the main interface between the Arduino Pro-Mini CPU board and the rest of the electronics in the imaging system. It resides in the camera module and controls the timing of the camera and LED light module. It is programmed via the FTDI port on the Pro-Mini CPU and it contains a Real Time Clock (RTC) for accurate timing.

Figure 2: Block Diagram of SeeStar II Arduino Controller Shield

Figure 3: Top view of controller pcb

Figure 4: Rear view of controller pcb

# Basic Operation

This section is meant to assist the user with simple bench top testing. For details on installing it in the camera module refer to the Camera Technical Manual.

## Initial Board Power Up and Testing

Before any newly populated boards are powered up and used, a series of tests should be performed in order to ensure proper operation. This section is meant as a quick series of tests, for more details on individual circuits, refer to the Functional Descriptions section.

### Selection of 3.3V or 5V Arduino

Install a 2-pin shunt jumper on J2.

Pins 1-2 for 5V Arduinos

Pins 2-3 for 3.3V Arduinos

### Fuse Verification

F1 should have a 4 Amp slo-blo fuse installed

### LED Functionality

During testing it is useful to use the LEDs for quick visual feedback. Install a 2-pin shunt jumper on J9 to enable the LEDs.

### Real Time Clock Functionality

Install the required watch battery in BT1 on the rear side of the pcb.

### Logic Power Verification

Set an external DC power supply up for 12Vdc with a 3 amp current limit. Hook the output of the DC power supply up to J1 on the Controller Shield. Turn on the DC power supply and with a voltmeter, measure Vdd. This can be done at many locations; the positive end of C3 is convenient. Vdd should match the chosen value from step one.

# Functional Descriptions

## Arduino Pro-Mini Interface

The Arduino Pro-Mini plugs into the top of the Controller Shield; silk-screening on the shield indicates the position of the FTDI connector for proper orientation. The connectors on the shield were numbered to match those of the Pro-Mini (JP6 connects to J6 and JP7 connects to J7).

The usage of the Pro-Mini signals is displayed in Tables 1 and 2. Technical details of the signals are described in the board level signal section.

| JP7 Pin # | Arduino Name | Shield Name | Arduino Signal Type |
| --- | --- | --- | --- |
| 12 | TXO | TXD | TTL Level Serial |
| 11 | RXI | RXD | TTL Level Serial |
| 10 | RESET | N/A |  |
| 9 | GND | GND | Ground |
| 8 | D2 | RTC_INT | Digital Input |
| 7 | D3 | N/A |  |
| 6 | D4 | LED_ON | Digital Output |
| 5 | D5 | RTC_RESET | Digital Output |
| 4 | D6 | UART_SHDN | Digital Output |
| 3 | D7 | LED_POWER | Digital Output |
| 2 | D8 | CAM_ON | Digital Output |
| 1 | D9 | CAM_POWER | Digital Output |

Table 1: Arduino Pro-Mini JP7 Signals

| JP6 Pin # | Arduino Name | Shield Name | Arduino Signal Type |
| --- | --- | --- | --- |
| 1 | RAW | BATT | Input Power |
| 2 | GND | GND | Ground |
| 3 | RESET | N/A |  |
| 4 | VCC | N/A |  |
| 5 | A3 | N/A |  |
| 6 | A2 | N/A |  |
| 7 | A1 | BATT_CURR | Analog Input |
| 8 | A0 | BATT_VOLT | Analog Input |
| 9 | SCK | SCK | SPI |
| 10 | MISO | MISO | SPI |
| 11 | MOSI | MOSI | SPI |
| 12 | D10 | RTC_SS | Digital Output |

Table 2: Arduino Pro-Mini JP6 Signals

## Battery Monitoring and Input Protection

Battery voltage is applied to the Controller Shield at J1; J1 has four pins due to the positive and negative power lines being doubled up in the inter-module cable to reduce resistive losses. Q1 offers reverse polarity protection and F1 is a replaceable fuse. The maximum current draw should never exceed 3 amps, but the traces and fuse are rated for 4 amps.

The current draw and voltage level of the battery module is monitored by the Controller Shield. U2 is a current sense amplifier which amplifies a low level voltage developed across sense resistor R1. The built-in gain of the MAX4372F is 50. The output of U2 (signal BATT_CURR) is a 0-3V analog signal that is calculated by:

BATT_CURR (volts) = IBatt (amps) x Gain x Rsense (ohms)

= IBatt (amps) x 50 x 0.02 (ohms)

= IBatt (amps) x 1 (ohms)

or conversely

IBatt (curr) = BATT_CURR (volts) / 1 (ohms)

For example, measuring 1.2V at BATT_CURR corresponds to a battery current of 1.2 amps:

Battery current = 1.2V / 1 ohm

Battery current = 1.2 Amps

The battery voltage level is scaled via resistors R3 and R6. With a maximum expected battery voltage of 16Vdc, the scaling ratio is set to 0.188. The output of op amp U3 is BATT_VOLT. It is calculated by:

BATT_VOLT (volts) = (R6/(R3+R6)) x BATT (volts) = 0.188 x BATT (volts)

or conversely

BATT (volts) = 5.319 x BATT_VOLT (volts)

For example, measuring 2.35V at BATT_VOLTS corresponds to a battery bus voltage of 12.5Vdc

## Logic Power Generation

The Controller Shield has been designed to work with either the 3.3V or 5V version of the Arduino Pro-Mini. This is accomplished by using an adjustable DC switching regulator with two possible feedback resistors to set the output voltage. A 3-pin jumper, J2, allows the user to configure the controller shield. Jumpering pins 1-2 is used for 5V operation, while jumpering 2-3 is for 3.3V. U1 is a switching regulator (MAX5035D) which provides 1 amp at approximately 85% efficiency.

** Do not power up the Controller Shield without installing a jumper on J2. Doing so will result in damaging U1 **

## GoPro Hero Power and On/Off Control

The Controller Shield is responsible for supplying battery voltage and a control signal to the GoPro Hero camera. In order to save battery capacity, power is completely removed from GoPro between images. The factory GoPro battery is removed and a Battery Eliminator is used in its place. The Battery Eliminator accepts 9-16Vdc and generates the required voltage for the GoPro.

The active high CAM_POWER signal turns on an N-Channel MOSFET (Q2A) which then turns on a P-Channel MOSFET (Q2B) which passes battery voltage to the Battery Eliminator via J3. This applies power to the GoPro but does not turn it on (electrically equivalent to simply putting a battery in the camera).

The active high CAM_ON signal forward biases an NPN transistor (Q4). The collector of Q4 is connected to pin #12 of the exposed Hero bus on the rear of the camera. Pulling pin 12 low is equivalent to depressing the on/off button. CAM_ON needs to be held high for approximately 1000msec to turn the GoPro on.

## LED Power and Trigger

The Controller Shield is responsible for supplying battery voltage and a trigger signal to the LED module.

The active high LED_POWER signal turns on an N-Channel MOSFET (Q3A) which then turns on a P-Channel MOSFET (Q3B) which passes battery voltage to the LED module via J8. This applies power to the LED but does not turn it on.

The active high LED_ON signal forward biases an NPN transistor (Q5). The collector of Q5 is connected to the on/off button on the LED. Pulling this line low turns on the LED and changes the operating mode of the LED. It is a momentary low signal and needs to be held low for approximately 500msec.

## Real Time Clock Control

The Controller Shield is equipped with a 2ppm accuracy real time clock (U5). The DS3234 maintains seconds, minutes, hours, day, date, month, and year information. It also has 2 user programmable alarms via the INT open. The DS3234 communicates via the SPI bus. See the Programming Guide and the DS3234 datasheet for further details.

## Serial Interface

The Controller Shield has an optional serial port (via J5) for accessing the Arduino Sketch or for writing a user interface. The TTL level serial signals are shared with the FTDI port and cannot be used simultaneously. Jumper J4 disables the serial transceiver to ensure proper FTDI operation.

# Board Jumper Options

## DC Power LEDs

There are three board level LEDs which indicate the state of various power levels. Table three is a listing of the LEDs and levels monitored. Jumper J9 can be installed to enable all three LEDs simultaneously.

| LED | Color | Function |
| --- | --- | --- |
| D2 | Blue | Camera Power |
| D3 | Red | LED Power |
| D4 | Green | Logic Power |

Table 3: Board Level LEDs and Functions

## Serial Communication Source

The TTL level serial signals are shared by the FTDI connector and the external serial transceiver U4. (MAX3227E). Attempting to use the FTDI port while the serial transceiver is active will result in miscommunication. The serial transceiver may be disabled via the UART_SHDN signal, or easier still is to place a jumper on J4. The serial transceiver will be disabled as long as jumper J4 is installed.

# Board Level Signal Descriptions

All analog and digital signals originate or terminate with the Arduino Pro-Mini. Their detailed descriptions and locations are described in this section.

## Analog Input Signals

BATT_CURR [A1; JP6-7]

This is a 0-3V analog voltage signal which corresponds to the current draw from the battery module. The conversion factor is Y(amps) = 1.0 x BATT_CURR (volts).

BATT_VOLT [A0; JP6-8]

This is a 0-3V analog voltage signal which corresponds to the output voltage of the battery module. The conversion factor is Y(volts) = 5.32 x BATT_VOLT (volts).

## SPI Bus Signals

SCK [SCK; JP6-9]

The clock input for the SPI bus.

MISO [MISO; JP6-10]

The SPI data output of the DS3234.

MOSI [MOSI; JP6-11]

The SPI data input to the DS3234.

## Digital Input Signals

RTC_INT [D2; JP6-8]

This is the active low alarm output of the DS3234 real time clock.

## Digital Output Signals

RTC_SS [D10; JP6-12]

This is the active low chip select signal for the SPI bus controlling the DS3234 real time clock.

LED_ON [D4; JP7-6]

This is an active high signal which is used to turn on the LED light. This is a momentary signal and needs to stay high approximately 500msec in order for the LED to turn on.

RTC_RESET [D5; JP7-5]

This is active low reset line. It is continuously monitored as a pushbutton input for generating a reset.

UART_SHDN [D6; JP7-4]

This is an active low signal which disables the serial transceiver (U4).

LED_POWER [D7; JP7-3]

This is an active high signal which applies battery voltage to the LED module. This is a continuous signal and must remain high while the LED is in operation.

CAM_ON [D8; JP7-2]

This is an active high signal which is used to turn on the GoPro Hero camera. CAM_POWER must already have been asserted for CAM_ON to work. This is a momentary signal and needs to remain high for approximately 1000msec for the GoPro to turn on.

CAM_POWER [D9; JP7-1]

This is an active high signal which applies battery voltage to the GoPro Battery Eliminator. This is a continuous signal and must remain high while the GoPro is in operation.

## Serial Signals

TXD [TXO; JP7-12]

This is a TTL level transmit signal coming from the Arduino.

RXD [RXI; JP7-11]

This is a TTL level receive signal going to the Arduino.

# Appendices

| Input Voltage | 9-15Vdc |
| --- | --- |
| Current |  |
| Max | 3 amps |
| Nominal | ~50 ma |

Appendix 1: Electrical Specifications

| Length | 2 316 in |
| --- | --- |
| Width | 1 38 in |
| Height | 916 in |
| Operating Temperature |  |
| Min | 0° C |
| Max | 70° C |
| Operating Depth | 1500m |

Appendix 2: Mechanical Specifications

| Ref | Value | Manufacturer | Manufacturer PN | Description |
| --- | --- | --- | --- | --- |
| BATT | CR1220 | Panasonic | CS1220 | Battery Lithium 3V Coin 12.5mm |
| BT1 | S8411-45R | Harwin Inc | S8411-45R | BATTERY HOLDER 12.0MM COIN SMD |
| C1 | 0.1uF | TDK Corporation | C1608X7R1H104K | CAP CER 0.1UF 50V 10% X7R 0603 |
| C2 | 47uF | United Chemi-con | EMVE500ADA470MF80G | CAP ALUM 47UF 20% 50V SMD |
| C3 | 68uF | AVX Corporation | TAJD686K016RNJ | CAP TANTALUM 68UF 16V 10% SMD |
| C4 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 |
| C5 | 0.1uF | TDK Corporation | C1608X7R1H104K | CAP CER 0.1UF 50V 10% X7R 0603 |
| C6 | 0.22uF | Kemet | C0603C224K3RAC | CAP CER 0.22UF 25V X7R 0603 |
| C7 | 1.0uF | Kemet | C0603C105K4RAC | CAP CER 1UF 16V X7R 0603 |
| C8 | 0.22uF | Kemet | C0603C224K3RAC | CAP CER 0.22UF 25V X7R 0603 |
| C9 | 1.0uF | Kemet | C0603C105K4RAC | CAP CER 1UF 16V X7R 0603 |
| C10 | 1.0uF | Kemet | C0603C105K4RAC | CAP CER 1UF 16V X7R 0603 |
| C11 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 |
| C12 | 0.10uF | Kemet | C0603C104K4RAC | CAP, CER, 0.10uF, 16V, 10%, X7R, 0603 |
| D1 | B340A-13-F | Diodes Inc | B340A-13-F | DIODE SCHOTTKY 3A 40V SMA |
| D2 | LTST-C150TBKT | Lite-On Inc | LTST-C150TBKT | LED 468NM BLUE CLEAR 1206 SMD |
| D3 | LTST-C150KRKT | Lite-On Inc | LTST-C150KRKT | LED SUPER RED CLEAR 1206 SMD |
| D4 | LTST-C150KGKT | Lite-On Inc | LTST-C150KGKT | LED GREEN CLEAR 1206 SMD |
| F1 | 0154004.DRT | Littelfuse Inc | 0154004.DRT | FUSE SLOW 125VAC, 125VDC 4A SMD |
| J1 | 1725672 | Phoenix Contact | 1725672 | CONN TERM BLOCK 2.54MM 4POS |
| J2 | TSW-103-14-T-S | Samtec Inc. | TSW-103-14-T-S | CONN HEADER 3POS .100" SNGL TIN |
| J3 | 1725656 | Phoenix Contact | 1725656 | CONN TERM BLOCK 2.54MM 2POS |
| J4 | TSW-102-14-T-S | Samtec Inc. | TSW-102-14-T-S | CONN HEADER 2POS .100" SNGL TIN |
| J5 | SSW-103-01-T-S | Samtec Inc | SSW-103-01-T-S | CONN RCPT .100" 3POS SNGL TIN |
| J6 | SSW-112-01-T-S | Samtec Inc | SSW-112-01-T-S | CONN RCPT .100" 12POS SNGL TIN |
| J7 | SSW-112-01-T-S | Samtec Inc | SSW-112-01-T-S | CONN RCPT .100" 12POS SNGL TIN |
| J8 | 1725672 | Phoenix Contact | 1725672 | CONN TERM BLOCK 2.54MM 4POS |
| J9 | TSW-102-14-T-S | Samtec Inc. | TSW-102-14-T-S | CONN HEADER 2POS .100" SNGL TIN |
| J10 | SSW-102-01-T-S | Samtec Inc | SSW-102-01-T-S | CONN RCPT .100" 2POS SNGL TIN |
| J11 | SSW-102-01-T-S | Samtec Inc | SSW-102-01-T-S | CONN RCPT .100" 2POS SNGL TIN |
| L1 | 100uH | Bourns Inc. | SRN6045-101M | FIXED IND 100UH 700MA 494 MOHM |
| Q1 | SI2369DS-T1-GE3 | Vishay Siliconix | SI2369DS-T1-GE3 | MOSFET P-CH 30V 7.6A TO-236 |
| Q2 | SI6562CDQ-T1-GE3 | Vishay Siliconix | SI6562CDQ-T1-GE3 | MOSFET N/P-CH 20V 6.7A 8-TSSOP |
| Q3 | SI6562CDQ-T1-GE3 | Vishay Siliconix | SI6562CDQ-T1-GE3 | MOSFET N/P-CH 20V 6.7A 8-TSSOP |
| Q4 | MMBT3904 | Fairchild | MMBT3904 | TRANSISTOR GP NPN AMP SOT-23 |
| Q5 | MMBT3904 | Fairchild | MMBT3904 | TRANSISTOR GP NPN AMP SOT-23 |
| R1 | 0.02 | Stackpole | CSR1206FK20L0 | RES SMD 0.02 OHM 1% 1/2W 1206 |
| R2 | 1.00M | Vishay | CRCW06031M00FKEA | RES SMD 1M OHM 1% 1/10W 0603 |
| R3 | 121.0K | Vishay | CRCW0603121KFKEA | RES SMD 121K OHM 1% 1/10W 0603 |
| R4 | 22.6K | Vishay | CRCW060322K6FKEA | RES SMD 22.6K OHM 1% 1/10W 0603 |
| R5 | 41.2K | Vishay | CRCW060341K2FKEA | RES SMD 41.2K OHM 1% 1/10W 0603 |
| R6 | 28K | Vishay | CRCW060328K0FKEA | RES SMD 28.0K OHM 1% 1/10W 0603 |
| R7 | 383.0K | Vishay | CRCW0603383KFKEA | RES SMD 383K OHM 1% 1/10W 0603 |
| R8 | 13.3K | Vishay | CRCW060313K3FKEA | RES SMD 13.3K OHM 1% 1/10W 0603 |
| R9 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R10 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R11 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R12 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R13 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R14 | 1.69K | Vishay | CRCW06031K69FKEA | RES SMD 1.69K OHM 1% 1/10W 0603 |
| R15 | 1.69K | Vishay | CRCW06031K69FKEA | RES SMD 1.69K OHM 1% 1/10W 0603 |
| R16 | 130 | Vishay | CRCW0603130RFKEA | RES SMD 130 OHM 1% 1/10W 0603 |
| R17 | 10.0K | Vishay | CRCW060310K0FKEA | RES SMD 10K OHM 1% 1/10W 0603 |
| R18 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R19 | 100.0K | Vishay/Dale | CRCW0603100KFKEA | RES 100K OHM 1/10W 1% 0603 SMD |
| R20 | 0.0 | Vishay/Dale | CRCW06030000Z0EA | RES 0.0 OHM 1/10W 0603 SMD |
| R21 | 0.0 | Vishay/Dale | CRCW06030000Z0EA | RES 0.0 OHM 1/10W 0603 SMD |
| R22 | 10.0K | Vishay | CRCW060310K0FKEA | RES SMD 10K OHM 1% 1/10W 0603 |
| U1 | MAX5035DASA+ | Maxim Integrated | MAX5035DASA+ | IC REG BUCK ADJ 1A 8SOIC |
| U2 | MAX4372FEUK+T | Maxim Integrated | MAX4372FEUK+T | IC AMP GAIN +50V/V PREC SOT23-5 |
| U3 | AD8603AUJZ | Analog Devices Inc | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U4 | MAX3227EAAE+ | Maxim Integrated | MAX3227EAAE+ | IC TXRX RS-232 W/SHTDWN 16-SSOP |
| U5 | DS3234S# | Maxim Integrated | DS3234S# | IC RTC W/TCXO 20-SOIC |

Appendix 3: Controller Shield Bill of Materials