MBARI’S MINI ROV CONTROL/WORK VAN SPECIFICATIONS

Single trip 20’ ISO container

Insulate interior with R13 insulation and install ¾” plywood liner on walls and ceiling

Retain double doors

Install water tight personnel door (located on side of van)

Install Roxtec regular cable gland

Install recess for 2t AC unit and “Mains” power connection (1 x 230 3 phase @ 20A) and (1 x 230/220 1 phase @ 60A)

Install (inside) 100A service breaker panel for the 230/220 1 phase main supply

Paint exterior with industry standard marine epoxy paint