| Project Number: | 901300 |
| --- | --- |

| Project Start: Jan. 1, | 2013 | Project Duration (in years): | 1 |
| --- | --- | --- | --- |

Please indicate what type of project is being proposed (mark only one).

|  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Feasibility/Scoping [Less than 100 days effort] |  |  | Development |  |  | Research |  |  | Infrastructure1 |  |  | Education |  |  |
|  |  |  | Time Series |  |  | Institute Initiative |  |  |  |  |  |  |  |  |

Project Manager:		Alana Sherman

Principal Investigator:	Charlie Paull

Lead Scientist:			Charlie Paull

Lead Engineer:			Dale Graves

Project Title:		MiniROV Upgrades

- ABSTRACT (new projects only)

In the past year, we have built a fully functional mini-ROV that provides a fly-away system capable of operating in up to 1 km depths on ships of opportunity. Given the limited time and money, our primary focus has been on building an excellent core vehicle. To date, the only features that have been added are those that are necessary to complete its initial science mission (i.e. high-resolution imaging and gas sampling). However, the core vehicle was designed to be reconfigurable for a wide array of science operations in remote areas. We now propose to increase the capabilities of the mini-ROV. After discussions with several potential science users, we have developed the following list of payload additions that will enable new science missions to be conducted, enhance the functionality of the vehicle, and increase its reliability:

- Five-function manipulator arm – This will enable a wide spectrum of sampling (rock samples, push cores, etc.) and experimental deployments to be conducted.  The arm will also enhance the capability of the ROV as an AUV rescue vehicle.

- Improvements to the core vehicle and accessories – An add-on daughter board for the fiber multiplexer to increase the number of serial data channels is essential for most of the proposed additions. The addition of a level-wind for the winch would reduce the number of personnel required for launch and recovery. Finally, we have a number of minor refinements we would like to make to the vehicle to improve its performance and reliability.

- Developing a dedicated van for the ROV – Currently, the existing system needs to be disassembled and shipped in multiple crates. We are dependent on finding an appropriate interior space near the working deck that serves as a control room as well as a place where the ROV can be stored and serviced while at sea. Finding an appropriate space can be an issue on many vessels of opportunity. For instance, this will become an issue in 2013 if both the mapping AUV and the mini-ROV go to the Arctic as proposed. Thus, we would like to configure a small van that is dedicated to supporting this vehicle by serving as the systems shipping container, the ROV’s at sea garage, and control room.

- Neutral umbilical – The existing umbilical is a repurposed section of Ventana’s tether and is operated with a clump weight. This tether is not ideal for other proposed applications such as midwater work and operating under icebergs due to the tether’s negative buoyancy. We have specified a neutral tether for these missions but lacked the funds to purchase it.

- Midwater sampling system – We propose to develop a miniaturized suction sampler, equivalent in function to the suction samplers on the Doc Ricketts and Ventana. This will allow the collection of multiple biological samples during each deployment.

We will propose each of the upgrades as discrete items, which will enable them to be implemented in 2013 or subsequent years.

- PROPOSAL

  - New Projects

Statement of the problem, why it is important

In mid-2012, we completed building the MiniROV on schedule and on budget with all required core vehicle functions operational. This generic platform is designed to be used in a wide variety of science applications. In order to maximize the utility of this vehicle, we are now proposing to add features that were beyond the scope or resources of the original development project. These improvements are designed to enhance the mission capabilities as well as increase the robustness of the vehicle.

How does it relate/contribute to MBARI’s mission and strategic plan?

MiniROV is an enabling technology that fits into two themes in MBARI’s strategic plan:

Exploration and Discovery: The MiniROV extends MBARI’s reach into remote areas.

Ocean Visualization: The MiniROV enables imaging of polar phenomena (e.g., marine permafrost and icebergs).

How does it relate/contribute to the Packard Foundation’s Ocean Visions Grantmaking policies? (See http://mww.mbari.org/oed/OED_Documents3.htm#Packard for documents.)

The MiniROV is a multi-purpose platform that can be used to contribute to several focus areas of The Packard Foundation’s Ocean Vision policy. This enabling technology is most relevant to the Packard Foundation’s focal area of Ocean Science Research and Monitoring. In addition, the MiniROV would also be a useful tool for monitoring coastal MPA’s another Packard Foundation focus area.

Approach to the problem

We are proposing five upgrades to the MiniROV. Each upgrade is independent and adds different capabilities to the vehicle. The whole project can be completed in one year or staged over several years.

High priority is assigned to the core vehicle upgrades and the manipulator arm which are both necessary for conducting the proposed 2013 field program in the Arctic as outlined in the Continental Margins Proposal (901023). The manipulator arm is required to provide AUV rescue capabilities.

The proposed van provides a protected environment for servicing and storage of the vehicle during deployments and doubles as a control room. In addition, it makes transport, mobilization and operations much more efficient.

Having a neutral tether is essential for extended horizontal operations such as midwater transects and including deployments as a rescue vehicle for the Iceberg AUV.

Midwater sampler will expand the operational capabilities of the fly away system on ships of opportunity. It will also enable sampling in the upper water column (e.g., <200 m), which cannot be done from the Western Flyer.

While a DVL is not essential for the Arctic science, it would be very useful both at enabling the vehicle to hover while sampling gas vents and to make quantitative video-transects. In addition, having a DVL makes it possible to conduct mid-water transects which the Midwater Ecology group has expressed an interest in. With minor reconfiguring, the DVL may also prove invaluable for operating under icebergs.  Funds to purchase a Doppler Velocity Logger (DVL) for the MiniROV are being requested in the Precision Control Project (600034). They have expressed an interest in using the MiniROV configured with the DVL for development and testing in the test tank.

Publications (from other projects over the last 5 years)

  - Continuing Projects

- New and Continuing Projects: Plans for 2013

Research Activities

Research activities using the MiniROV are outlined in other proposals (Continental Margins and Submarine Canyon Processes - 901023; Precision Control – 600034; Midwater Ecology – 901220; ROV Video MDUC proposal -500055).

Engineering/technology development

A full breakdown of each upgrade’s cost by item, task breakdown, and labor hours by discipline are included in a spreadsheet at the end of this section.

- Core vehicle improvements

  - Specific Upgrades:

    - Fiber connectors: Subconn OptoLink bulkhead connector and whip

      - Enables quick disconnect of the vehicle from the tether, which in turn allows storage and work on the vehicle in a van or other enclosure.

      - Minimizes ship-time lost to fiber re-termination

    - Focal upgrade: Moog PC 104 daughter card p.n. 907-232

      - Adds 6 more serial data channels for additional sensors and instruments (we have only one unused port)

      - This upgrade is required for manipulator upgrade

    - Level wind for winch: MBARI custom design and fabrication

      - Reduces operation crew by one person

      - Increases safety by using machine rather than person to level wind

    - Update Electronics pod housing

      - Increase length of housing to create more space for future expansion

      - New housing will use more robust materials

    - Update umbilical junction box: This umbilical junction box will need to be upgraded to accommodate the new quick disconnect feature

    - Power upgrade: Design and implement system to increase thruster bus voltage and thus overall vehicle thrust

  - General Improvements:

    - Improve documentation to facilitate hand over to the operations group

    - Minor refinement of current systems

      - Continue to optimize power system

      - Address any issues or improvements that are discovered after first science missions

- Manipulator arm

  - Five function manipulator arm: ECA Robotics Arm 5E

    - Enables sample collection and manipulation

    - Enables AUV rescue

- Dedicated van

  - Easier transport of ROV system

  - Enables ROV ops on ships with no interior space available for control room

  - Provides protected area to service and store ROV during cruises

- Neutral tether: Falmat custom order cable, 600 m

  - Enables extended horizontal excursions (midwater transects, iceberg work)

  - Could be used for iceberg work (necessary if using MiniROV as rescue vehicle for Iceberg AUV)

- Midwater sampling system: custom unit designed and fabricated at MBARI

  - Enables collection of discrete biological samples during ROV dives from surface to 1 km depths

- DVL

  - Enables ROV control system software to be tested in the test tank

  - Provides “distance traveled through water”, required for midwater transects.

  - Provides an accurate navigation tool for benthic dives

  - Enables existing ROV precision control system software (Rock) to be used to assist ROV operations

- ROV data stream

  - The intent is to format all the data from the MiniROV so it can be efficiently incorporated into the MBARI archives.

  - We are requesting 5 days of Rich Schramm’s time Rich Schramm’s time to create end-user GUI application to upload processed CTD data to standalone or shared SQL database.

  - Digital video data collected in the Arctic in 2012 and 2013 is going to be used as a test case for the video-labs on-going efforts to evaluate how to handle digital video formats. The resources to do this in 2013 will be covered under the ROV Video MDUC proposal -500055.

- Annual maintenance and purchasing of spares: We have rolled the annual maintenance budget and a small amount of labor into this project. These funds were previously approved to buy spare parts, consumables, and perform routine maintenance each year.

Field programs (detail ship requirements/deployments/diver requirements)

The 2013 scheduled for MiniROV operations includes:

- January -May 2013 - If the DVL requested in Precision Controls (600034) is approved, we anticipate the Mini-ROV will be periodically used in the test tank. Also, testing of the vehicle upgrades will also occur in this time window.

- June 15-early November – Paull Arctic cruise

- December - Possibly shipping to Antarctica where it could serve as an AUV rescue tool if Ken Smith’s NSF funded cruise occurs in early 2014.

Potential future field programs:

- The MBARI-GSC project in the Canadian Arctic, for which this vehicle was initially built, is intended to be a multiyear program. Efforts to bring the MiniROV back to the Arctic are likely to continue past 2013. The possibility that the MiniROV will be used on the Korean icebreaker Aaron, as KOPRI scientists are interested in participating in this research program, is also being discussed.

- Paull already has an active collaboration with Peter Barnes, who is the Principle Scientist at NIWA in New Zealand. He is interested in trying to obtain funding from the New Zealand Government to bring MBARI’s MiniROV and the mapping AUV to NZ to investigate submarine canyon heads and young seafloor faulting features.

- The Midwater Ecology group has two projects in development that would employ the mini-ROV in operations outside Monterey Bay. 1) Australia's CSIRO will be acquiring a new ship, R/V Investigator, in 2014. We have been invited to consider bringing the mini-ROV aboard the Investigator in a cooperative research project that would also demonstrate to CSIRO the research potential of a permanently installed ROV. 2) In continuing discussions with Conservation International, we have been focused on the use of the mini-ROV from ships of opportunity, to justify the expansion of MPAs in the western Pacific.

Specific deliverables

The following are the potential deliverables for this project:

- Improved core MiniROV

- Integrated manipulator on MiniROV

- Shipping container/control room outfitted for MiniROV

- Neutral tether for midwater dives

- Completed midwater sampling system for MiniROV

Milestones (see last page for personnel classification list)

|  | Milestone 1 | Milestone 2 | Milestone 3 | Milestone 4 | Milestone 5 | Milestone 6 | Total Resources for each resource category for all milestones listed here |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Milestone Date | TBD | TBD | TBD | TBD | TBD | TBD |  |
| Milestone Description | Core Upgrades | Manipulator | Van | Midwater Sampler | Neutral Tether | Annual Maint. |  |
| EE | 388 | 64 | 16 | 80 | 40 | 80 | 668 |
| SE |  |  |  |  |  |  |  |
| ME | 448 | 260 | 40 | 310 | 40 | 80 | 1178 |
| EE Tech | 100 | 20 | 56 | 24 | 40 | 80 | 320 |
| SE Tech |  |  |  |  |  |  |  |
| ME Tech | 180 | 120 | 120 | 140 | 40 | 80 | 680 |
| Machine Shop | 240 | 160 |  | 160 |  | 80 | 640 |
| Divers |  |  |  |  |  |  |  |

Budget justification

Please note that the $15,000 annual maintenance budget has been added into the budget of this project. This is the money that was to be ear marked annually to buy spares and perform routine maintenance for the vehicle.

| Item/Detail | M Tech | E Tech | ME | EE | Mch Shop | Materials |
| --- | --- | --- | --- | --- | --- | --- |
| Manipulator | 120 | 20 | 260 | 64 | 160 | $43,000 |
| Spec and purchase |  |  | 20 |  |  | $40,000 |
| Arm and can brackets | 40 |  | 40 |  | 40 | $500 |
| Drawer motor |  |  | 80 |  | 80 | $1,500 |
| Install/integration/test | 40 |  | 40 | 40 |  |  |
| Tube core/sampling devices | 40 |  | 80 |  | 40 | $500 |
| Misc hardware/connectors |  |  |  | 24 |  | $500 |
| Control van | 120 | 56 | 40 | 16 | 0 | $24,250 |
| 20' x 8' bare container |  |  | 40 |  |  | $4,050 |
| Insulation & walls |  |  |  |  |  | $4,100 |
| Electrical/lighting/heating | 40 | 40 |  |  |  | $6,500 |
| Personnel doors |  |  |  |  |  | $1,500 |
| Electrical pass thru |  |  |  |  |  | $1,500 |
| Misc hardware |  |  |  |  |  | $300 |
| shipping |  |  |  |  |  | $800 |
| Work bench/tool cabs | 80 |  |  |  |  | $5,500 |
| ROV topside installation |  | 16 |  | 16 |  |  |
| Neutral Umbilical | 40 | 40 | 40 | 40 | 0 | $30,000 |
| Spec and purchase |  |  | 20 | 20 |  | $30,000 |
| Install/integration/test | 40 | 40 | 20 | 20 |  |  |
| Midwater Samplers | 140 | 24 | 310 | 80 | 160 | $15,000 |
| Sampler index motor |  | 24 | 80 | 80 | 40 | $2,000 |
| Suction pump motor |  |  | 40 |  | 40 | $2,000 |
| sampler mech | 80 |  | 110 |  | 80 | $5,000 |
| Detritus sampler | 40 |  | 40 |  |  |  |
| D-sampler door open/cls mech | 20 |  | 40 |  |  | $5,000 |
| Misc hardware/connectors |  |  |  |  |  | $1,000 |
| Vehicle Improvements | 180 | 100 | 448 | 388 | 240 | $43,800 |
| Focal upgrade *Required for manipulator | 20 | 40 | 24 | 24 | 20 | $3,300 |
| Update Epod housing | 20 |  | 80 | 40 | 60 | $2,000 |
| Update umbilical j-box | 20 |  | 40 | 20 | 40 | $500 |
| Fiber connectors upgrade |  |  | 24 | 24 |  | $18,000 |
| Power upgrade | 40 | 40 | 120 | 120 | 80 | $10,000 |
| Winch level wind | 80 | 20 | 160 | 40 | 40 | $10,000 |
| Improve documentation |  |  |  | 40 |  |  |
| Power system optimization |  |  |  | 80 |  |  |
| Annual Maint./spares/consumables budget | 80 | 80 | 80 | 80 | 80 | $15,000 |
|  | 80 | 80 | 80 | 80 | 80 | $15,000 |
| Totals | 680 | 320 | 1178 | 668 | 640 | $171,050 |

Project team

Dale Graves, Charlie Paull, Alana Sherman.

Project lab space

None needed

Data availability outside project team, uniqueness

We intend to have this data stream incorporated into MBARI’s data stream and archived so it will be available to others. The video data we collect this Arctic cruise will be used by the video lab as a test case for MDUC services.

Export Regulation
Does the project team anticipate any foreign collaborations or international travel? If so, please identify collaborators by name, institution, and country.

Scott Dallimore, Geological Survey of Canada, Sidney, BC Canada

Does the project team plan or anticipate export controlled purchases, shipments, or transfers of items (e.g., research instruments, proprietary data, hand-carried tools or samples) outside the U.S. temporarily or permanently? If yes, please identify country or countries and indicate whether these items will remain under MBARI control while abroad or controlled by others. Will the items be exported to more than one country? Please include foreign collaborator information as described above.

The DVL Explorer is export controlled by the Dept. of Commerce to NS2 countries and will need an export license if shipped to a NS2 country.  However, none of the potential MiniROV work sites are on the NS2 list.

Based on Don Fisher’s judgment, neither EAR or ITAR restrictions apply to the present vehicle. We do not believe that these upgrades will change its status. We do not believe that these upgrades will change its status.

Risk matrix (development and infrastructure projects only)

|  | Technical Risk | Organizational Risk | Tech Transfer Risk | Labor Estimate Risk |
| --- | --- | --- | --- | --- |
| High |  |  |  |  |
| Medium |  |  |  | X |
| Low | X | X |  |  |

Personnel Classifications

| Engineer - Mechanical Engineer | Engineer - Electrical Engineer | Engineer - Software Engineer | Engineer - Mechanical Tech | Engineer - Electrical Tech | Engineer - Software Tech | Observ Support Technician | Science – Software Engineer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (Reg) | (Reg) | (Reg) | (Reg) | (Reg) | (Reg) | (Reg) | (Reg) |
| Jim Birch (2/3) | Mark Chaffey | Dave Caress | John Ferreira | Paul Coenen | Karen S | Mike Kelley | Mike Godin |
| Larry Bird | Scott Jensen | Danelle Cline | Frank Flores | Tom Marion |  | Jared Figurski | Frederic Py |
| Francois Cazenave | Chad Kecy | Duane Edgington(2/3) | Mike Parker | Jim Montgomery |  |  |  |
| Jon Erickson | Denis Klimov | Kevin Gomes (2/3) | Jim Schofield | Jose Rosal |  |  |  |
| Dale Graves (2/3) | Paul McGill | Kent Headley (2/3) |  |  |  |  |  |
| Andy Hamilton (2/3) | Ed Mellinger | Rich Henthorn |  |  |  |  | Science – Mechanical Engineer |
| Brett Hobson | Craig Okuda | Bob Herlien |  |  |  |  | (Reg) |
| Bill Kirkwood | Alana Sherman | Brian Kieft |  |  |  |  | Gene Massion |
| Doug Pargett |  | Thom Maughan |  |  |  |  |  |
| Farley Shane |  | Mike McCann |  |  |  |  |  |
|  |  | Rob McEwen |  |  |  |  |  |
|  |  | Tom O’Reilly |  |  |  |  |  |
|  |  | Wayne Radochonski |  |  |  |  |  |
|  |  | Mike Risi |  |  |  |  |  |
|  |  | Brent Roman |  |  |  |  |  |
|  |  | Carlos Rueda-Velasquez |  |  |  |  |  |
|  |  | Brian Schlining |  |  |  |  |  |
|  |  | Rich Schramm |  |  |  |  |  |

Development and infrastructure projects must complete the risk matrix (see item #5 at the end of the proposal form).

Not all projects will require all four titles to be filled in. Please indicate individuals only for those titles that are appropriate for your project.

This number should be the same number you entered on the resource requests for this particular resource.