Rover 1K Electrical Concept Design Review (Alana Sherman)

04-13-2011

Notes

In attendance: Alana Sherman, Jose Rosal, Paul McGill, Ed Mellinger, Andy Hamilton, Dale Graves, Ken Smith, Brett Hobson, Charlie Paull, Doug Au

Vehicle to have LED lights - similar in lumens to current phantom.

Ground faulting OTS - agency safety rated for topside (tether).

Shanna Rae - 6kW, 110V, single phase available.

Overvoltage protection for Vicors needed

Tether: 14AWGx4 SouthBay specificed 8A max - under what conditions?

Heating on the drum - water cooled?

Possible power solutions:

Running Vicor modules in series (at input) and parallel the outputs.

Shunt regulation

Question about multiple busses for the thruster 48Vdc. 2 Horz + 1 Vert per bus

Combining busses in the event of a Vicor failure?

Utilize shutdown feature of Vicor modules to maximize bus efficiency?

Excess tether only 10% longer. Why? No consensus.

Ed - noted to specify DC voltage for new tether.