EPod Reconfigure Test plan

- High-voltage testing:

  - Verify +375V on proper pins on MINK-8-CCPL coming from umbilical jbox

  - Verify proper pins going to positive and negative from bulkhead connector to terminal block

  - Disconnect all +375V outputs except thrusters

  - Connect thruster +375V- verify power on correct pins

  - Connect 24V Vicor input and programmable load to 24V Vicor output

    - Turn on 375 input and verify +24V on output

    - Load test output to 300W or higher depending on programmable load

  - Reconnect +12V Vicor and attach programmable load to output

    - Turn on 375 and verify +12V output

    - Put load of 75 Watts on Vicor 12V

  - Repeat steps e with +5V vicor

  - Connect Aux +375V- verify power thru fuse

  - Need to wait on testing Crydom switch until Wago stack is up

- Test focal

  - Remove 5V fuse for Wago and connect 5v output from vicor to fuse block

  - This should turn on focal stack

  - Make sure fiber connected between top and bottom focal boxes

- Test Wago

  - Remove 24V fuses  except EPod loads, connect +24V from vicor to fuse block- fan should turn on, gfm modules and wago stack

  - Connect fuse (6A) for Wago stack- this will turn everything on except +5V

  - Check comms to stack if OK, connect fuse for +5V (wago)

  - Try to exercise Wago stack:

    - Try to select GFM channels

    - Test six relays functions- this can be done by checking connectivity on output or putting in fuses for other loads and measuring +24V

    - Read voltage bridge and humidity signal

    - Read rtd

    - Test Crydom SSR for +375 Aux

      - Measure +375V at bulkhead connectors:

        - Core j-box

        - Tool skid j-box

        - GigE/HV output

    - Test analog input to thrusters

    - Switch 5V loads

      - Disable thrusters

      - Other loads will have to be measured at bulkhead connector: depth, heading, lasers

  - Test video

    - Connect HD camera cable to EPod

    - Make sure rattler fuse insered, test rattle powers up

    - Check for video on top side