Smart Clump Test plan

- High-voltage testing:

  - Verify +375V on proper pins on MINK-8-CCPL coming from umbilical jbox

  - Verify proper pins going to positive and negative from bulkhead connector to terminal block

  - Disconnect all +375V outputs except thrusters

  - Confirm power on correct pins on thruster MINK16

  - Connect 24V Vicor input and programmable load to 24V Vicor output

    - Turn on 375 input and verify +24V on output

    - Load test output to 300W

  - Reconnect +5V Vicor and attach programmable load to output

    - Turn on 375 and verify +5V output

    - Test load of 50 Watts on Vicor 5V

  - Connect External loads +375V- verify power thru fuse

  - Need to wait on testing Crydom switch until Wago stack is up

- Test focal

  - Remove 5V fuse for Wago, disconnect rattler and GFMs and connect 5v output from Vicor to fuse block

  - This should turn on focal stack, verify focal lights up

  - Set up topside stack for testing…

- Test Wago

  - Remove 24V fuses  except EPod loads, connect +24V from vicor to fuse block- fan should turn on, gfm modules and wago stack

  - Connect fuse (6A) for Wago stack- this will turn everything on except +5V

  - Check comms to stack are OK, connect fuse for +5V (wago)

  - Try to exercise Wago stack:

    - Connect serial port to wago stack

    - Try to read humidity input and +375V bridge

    - Read rtd

    - Try to set Analog outputs to thrusters and light circuit

    - Test relays for use with tristate board, verify relays closing as expected

    - Try selecting and reading GFM channels

    - Test +24V is correctly being sent to:

      - Lights

      - Aux cam

      - HD cam

      - Tilt

      - Alt/depth

      - sonar

    - Test Crydom SSR for +375 Aux

      - Measure +375V at thruster connectors:

    - Switch 5V loads

      - Disable thrusters

      - Other loads will have to be measured at bulkhead connector: depth, heading, lasers

  - Test video

    - Connect HD camera cable to EPod

    - Make sure rattler fuse insered, test rattle powers up

    - Check for video on top side