65mm diameter

59mm length

10 Amps RMS CRITICAL Future application

48V Peak Rover by wire

48V Peak Battery for future application; Do not know minimum voltage

1828 RPM under load

1.89 Nm Nominal Torque (possibly peak torque)

K064150-5Y1 appears to be golden motor based on

- 10 Amps max

- 65mm Diameter

- 96.8mm Length exceeds 59mm

- Peak Torque at 10 amps is 1.9 Nm

- Rated Torque at 10 Amps (7 amps RMS) only 1.4 Nm

K089075-5Y1 possible golden motor based on Torque continuous

- 10 Amps Max

- 89 mm Diameter over exceeds 65mm

- 25mm Length below dimension requirement

- Peak Torque at 10 amps is 2 Nm

- Rated Torque at 10 amps at 2Nm