# Resolver-to-Motor Alignment Procedure

## Purpose

This procedure aligns the resolver position feedback system to the motor's electrical zero position, ensuring accurate commutation and optimal motor performance.

## Equipment Required

- Digital multimeter (DMM) or oscilloscope

- Resolver-to-digital converter PCB (for NM signal monitoring)

- Permanent marker or scribe

- Shaft rotation tool (or ability to rotate by hand)

- Resolver mounting hardware (with ability to rotate housing)

- Optional: Digital angle gauge or protractor for verification

## Safety Precautions

⚠️ WARNING: Do NOT apply power to motor phases during alignment process
⚠️ Ensure motor drive is disconnected and locked out
⚠️ Verify no stored energy in capacitors before handling
⚠️ Use proper hand protection when rotating shaft manually

## Background Information

### Key Concepts

Electrical Zero: The rotor position where phase A winding has maximum positive flux linkage with the rotor magnets. This occurs when a rotor north pole aligns with the phase A magnetic axis.

Mechanical vs Electrical Degrees:

- Electrical degrees = Mechanical degrees × Number of pole pairs

- Example: 8-pole motor (4 pole pairs): 90° mechanical = 360° electrical

What Needs Alignment:

- Resolver STATOR (stationary housing) aligns to motor STATOR

- Resolver ROTOR is mechanically fixed to motor shaft (rotates together)

- The resolver rotor's absolute position on the shaft doesn't matter electrically

- We adjust the resolver stator/housing to make the NM index pulse occur at motor electrical zero

### NM Signal (North Magnetic / Index Signal)

- Generated by your resolver-to-digital converter

- Pulses once per electrical revolution

- For motors with multiple pole pairs, pulses multiple times per mechanical revolution

- This signal indicates when the resolver "thinks" electrical zero has occurred

## PART 1: Finding Motor Electrical Zero

### Step 1: Prepare the Motor and Identify Phases

1.1. Verify power disconnection

- Confirm motor drive power is OFF and locked out

- Verify no voltage present on ABC phase terminals

1.2. Identify Phase A, B, and C

Method 1: Check Motor Documentation/Labeling

- Most motors have phases labeled on the terminal block or wiring diagram

- Look for labels: A, B, C or U, V, W or L1, L2, L3

- Standard: A=U=L1, B=V=L2, C=W=L3

Method 2: Check Wire Colors (if present)

- Common color codes vary by region and manufacturer

- North America: Black (A), Red (B), Blue (C) - NOT STANDARDIZED

- Europe: Brown (U), Black (V), Grey (W)

- Warning: Wire colors are NOT standardized - verify with documentation

Method 3: Determine Phase Sequence Using Back-EMF

If phases are not labeled, you can identify them electrically:

3a. Label the three motor wires temporarily as Wire 1, Wire 2, Wire 3

3b. CRITICAL - Correct Oscilloscope Connection:

Connect as follows:

- Channel 1 probe tip: Wire 1

- Channel 1 ground clip: Wire 2

- Channel 2 probe tip: Wire 3

- Channel 2 ground clip: Wire 2 (SAME wire as Ch1 ground)

⚠️ WARNING: Both ground clips MUST connect to the same wire (Wire 2)

- All oscilloscope grounds are connected together internally

- Connecting grounds to different wires creates a SHORT CIRCUIT

- This will damage the motor or oscilloscope

What you're measuring:

- Ch1: Voltage between Wire 1 and Wire 2

- Ch2: Voltage between Wire 3 and Wire 2

Set scope to AC coupling, 1V/div, 50ms/div

3c. Rotate shaft slowly and steadily in the CLOCKWISE direction (viewed from shaft end)

- Observe the two sine waves on the oscilloscope

- Both waveforms should be similar amplitude sine waves

- Note which waveform LEADS the other

Understanding "Which Waveform Leads":

A waveform "leads" when it reaches its peak (or zero crossing) EARLIER in time than the other waveform.

- Leading waveform: Peaks appear to the LEFT on the oscilloscope display

- Lagging waveform: Peaks appear to the RIGHT on the oscilloscope display

Visual Example:

Time flows left to right on scope →

Leading:  ___/‾‾‾\___      (peaks earlier, further LEFT)

Lagging:      ___/‾‾‾\___  (peaks later, further RIGHT)

The waveforms should be separated by approximately 120° (1/3 of a cycle).

3d. Determine phase relationships:

- If Channel 1 (Wire 1 to Wire 2) LEADS Channel 2 (Wire 3 to Wire 2) by ~120°:

  - Meaning: Ch1 peaks appear LEFT of Ch2 peaks

  - Wire 1 = Phase A

  - Wire 2 = Phase B

  - Wire 3 = Phase C

- If Channel 2 LEADS Channel 1:

  - Meaning: Ch2 peaks appear LEFT of Ch1 peaks

  - Wire 1 = Phase C

  - Wire 2 = Phase A

  - Wire 3 = Phase B

3e. Mark and label the wires clearly with Phase A, B, C

Verification Tip: Once labeled, you can verify by measuring A-B and B-C. When rotating clockwise, A should lead B, and B should lead C, each by 120° electrical.

Method 4: Using a Multimeter Only (Slower Method)

4a. Rotate shaft at constant slow speed

4b. Measure AC voltage between Wire 1 and Wire 2:

- DMM red probe: Wire 1

- DMM black probe: Wire 2

- Note peak voltage and shaft rotation angle

4c. Continue rotating, measure between Wire 2 and Wire 3:

- DMM red probe: Wire 2

- DMM black probe: Wire 3

- Note peak voltage and shaft rotation angle

4d. Continue rotating, measure between Wire 3 and Wire 1:

- DMM red probe: Wire 3

- DMM black probe: Wire 1

- Note peak voltage and shaft rotation angle

4e. The voltages should peak in sequence as you rotate - this reveals the phase order

- The order in which peaks occur (during clockwise rotation) determines phase sequence

- If peaks occur in order 1-2-3: Wire 1=A, Wire 2=B, Wire 3=C

Quality Check:

- [ ] Phase A positively identified

- [ ] Phase B positively identified

- [ ] Phase C positively identified

- [ ] Phase labels clearly marked on wires or terminal block

1.3. Access the shaft

- Ensure shaft can rotate freely

- Remove any mechanical loads if possible (for easier rotation)

- Verify resolver rotor is already mounted to shaft (if not, mount it now - orientation doesn't matter)

1.4. Identify motor specifications

- Record number of poles: ______

- Calculate pole pairs (poles ÷ 2): ______

- Note: You will see this many NM pulses per mechanical revolution

### Step 2: Locate Electrical Zero Using Back-EMF Method

Note: This is the most reliable method for finding electrical zero.

2.1. Configure measurement setup

Option A: Using Digital Multimeter (DMM)

- Set DMM to AC voltage mode

- Connect red probe to Phase A terminal

- Connect black probe to Phase B terminal

Option B: Using Oscilloscope

- Set scope to AC coupling, 1V/div, 100ms/div

- Channel 1 probe tip: Phase A terminal

- Channel 1 ground clip: Phase B terminal

- You only need one channel for this measurement

Alternative: Measure Phase A to Neutral (if accessible)

- If motor has accessible neutral/star point:

- Channel 1 probe tip: Phase A terminal

- Channel 1 ground clip: Neutral/star point

- This often gives cleaner waveforms

2.2. Rotate shaft slowly

- Rotate shaft BY HAND at approximately 1 revolution per 2-3 seconds

- Observe the sine wave voltage generated (back-EMF)

- The voltage magnitude depends on rotation speed

2.3. Identify the zero-crossing

- Locate where the A-B voltage crosses zero going POSITIVE

- Alternative method: Find the PEAK POSITIVE voltage (this may be easier to identify)

Important Note: Depending on motor construction:

- Some motors: Zero-crossing = electrical zero

- Some motors: Peak voltage = electrical zero

- If unsure, use the peak positive voltage method (more common)

2.4. Mark electrical zero position

- Stop rotating when you find the reference point

- Use permanent marker to mark shaft position relative to motor housing

- Make TWO marks:

  - One mark on the shaft

  - One mark on the motor housing aligned with shaft mark

- Label this mark "Electrical Zero #1" or "E0-1"

2.5. Verify multiple electrical zero positions (for multi-pole motors)

- Continue rotating and mark ALL electrical zero positions

- For an N pole-pair motor, you should find N equally-spaced positions

- Mark them as E0-1, E0-2, E0-3, etc.

- Verify they are evenly spaced: 360° ÷ (number of pole pairs) apart

Quality Check:

- [ ] All electrical zero positions identified

- [ ] Positions are equally spaced mechanically

- [ ] Marks are clear and visible

- [ ] Selected one primary reference position for alignment

## PART 2: Resolver Installation and Initial Setup

### Step 3: Mount Resolver Assembly

3.1. Verify resolver rotor installation

- Confirm resolver rotor is securely mounted to motor shaft

- Check for proper mechanical clearance (no rubbing)

- Verify rotor runs true (no wobble)

3.2. Prepare resolver stator for adjustment

- Install resolver stator/housing onto motor, but DO NOT fully tighten mounting bolts

- Leave bolts loose enough that you can rotate the resolver housing by hand

- Ensure resolver rotor/stator gap is correct per manufacturer specs

3.3. Connect resolver to converter PCB

- Wire resolver excitation and signal lines to your resolver-to-digital converter

- Verify all connections are correct per converter documentation

- Apply power to resolver-to-digital converter ONLY (NOT motor phases)

### Step 4: Setup Measurement Equipment

4.1. Configure oscilloscope for NM signal

- Connect scope probe to NM (North Magnetic / Index) output

- Set scope to edge trigger, rising edge

- Trigger level: Set to 50% of NM signal high level (typically 2.5V for 5V logic)

- Time base: 100ms/div (adjust as needed)

- Voltage: 2V/div

4.2. Optional: Monitor resolver A/B signals

- If available, connect second channel to resolver A or B signal

- This helps verify resolver is functioning correctly

- You should see sine/cosine waveforms as shaft rotates

4.3. Verify resolver system operation

- Slowly rotate motor shaft by hand

- Confirm NM pulses appear on oscilloscope

- Count pulses per mechanical revolution (should equal number of pole pairs)

- If no pulses: Check wiring, power, and converter configuration

Quality Check:

- [ ] Resolver powered and functioning

- [ ] NM signal visible on oscilloscope

- [ ] Number of NM pulses per revolution matches pole pair count

- [ ] Resolver housing is rotatable but secure enough not to move on its own

## PART 3: Resolver Alignment Procedure

### Step 5: Align Resolver Stator to Electrical Zero

5.1. Position shaft at electrical zero

- Rotate motor shaft to align with your "Electrical Zero #1" mark

- Verify alignment is precise (marks line up exactly)

- Hold shaft steady or use a fixture to prevent movement

5.2. Monitor NM signal

- Observe oscilloscope display

- You may or may not see an NM pulse at this position (depends on current resolver stator alignment)

5.3. Rotate resolver stator (housing) to find alignment

- While holding the motor SHAFT stationary at electrical zero

- Slowly rotate the resolver HOUSING/STATOR (not the shaft)

- Watch the oscilloscope for the NM pulse

Critical: You are rotating the resolver body, NOT the shaft

5.4. Identify NM pulse position

- Continue rotating resolver housing until NM signal transitions HIGH (rising edge)

- Fine-tune the resolver housing position to center the NM pulse trigger at the exact electrical zero position

- The goal: NM rising edge occurs exactly when shaft is at electrical zero mark

5.5. Lock resolver housing in position

- While maintaining resolver housing position, carefully tighten mounting bolts

- Tighten in star pattern to avoid shifting the housing

- Recommended: Tighten incrementally (25% → 50% → 75% → 100% torque)

- Verify housing did not rotate during tightening

Important Notes:

- Work carefully - small housing movements create large electrical errors

- If resolver shifts during tightening, loosen and repeat steps 5.3-5.5

- For multi-pole motors, it doesn't matter which E0 position you use, but document which one

## PART 4: Verification and Testing

### Step 6: Verify Alignment Accuracy

6.1. Test all electrical zero positions

- Slowly rotate shaft through one complete mechanical revolution

- Verify NM pulses occur at ALL marked electrical zero positions

- All NM pulses should align with their corresponding marks (E0-1, E0-2, etc.)

6.2. Check for alignment drift

- Rotate shaft multiple revolutions in both directions

- Verify NM pulses remain consistent with electrical zero marks

- Look for any mechanical play or looseness

6.3. Measure alignment accuracy (if possible)

- Using oscilloscope cursors or timing measurements

- Calculate position error in mechanical degrees

- Convert to electrical degrees: Error(electrical) = Error(mechanical) × pole pairs

Target accuracy:

- High-performance servo: ±1° electrical or better

- General motor control: ±5° electrical acceptable

- If error >10° electrical: Realign resolver

6.4. Document alignment

- Record which electrical zero position was used (E0-1, E0-2, etc.)

- Note measured alignment accuracy

- Photograph alignment marks and resolver mounting

- Record date and technician name

### Step 7: Final Checks

7.1. Mechanical verification

- [ ] All resolver mounting bolts torqued to specification: _____ N⋅m

- [ ] No mechanical interference between resolver rotor and stator

- [ ] Shaft rotates smoothly through full range

- [ ] Resolver cable secured and strain-relieved

7.2. Electrical verification

- [ ] NM signal clean and consistent (no noise or dropouts)

- [ ] Resolver A/B signals present and 90° phase shifted (if monitoring)

- [ ] All wiring connections secure

- [ ] No short circuits or ground faults

7.3. System integration preparation

- [ ] Alignment marks documented and photographed

- [ ] Alignment accuracy recorded

- [ ] Resolver configuration parameters recorded

- [ ] Ready for motor controller commissioning

## PART 5: Troubleshooting

### Issue: No NM Pulses Detected

Possible Causes:

- Resolver-to-digital converter not powered

- Incorrect wiring to converter

- Converter configuration incorrect

- Faulty resolver or converter

Solutions:

- Verify power supply to converter (check voltage levels)

- Check all resolver wiring against converter documentation

- Verify resolver excitation signal is present (typically 5-10kHz AC)

- Consult converter documentation for configuration requirements

- Test with known-good resolver if available

### Issue: Wrong Number of NM Pulses Per Revolution

Possible Causes:

- Converter configured for wrong pole count

- Resolver or converter malfunction

Solutions:

- Check converter pole pair configuration setting

- Verify motor pole count is correct

- Reconfigure converter for correct pole pairs

- Test resolver on bench if possible

### Issue: NM Pulses Don't Align with Electrical Zero Marks

Possible Causes:

- Resolver housing rotated during bolt tightening

- Incorrect electrical zero identification

- Resolver rotor slipping on shaft

Solutions:

- Loosen resolver housing and repeat alignment procedure

- Re-verify electrical zero using back-EMF method

- Check resolver rotor set screw or press-fit

- Use fixture or assistant to hold housing during tightening

### Issue: NM Alignment Varies After Multiple Rotations

Possible Causes:

- Mechanical play in resolver mounting

- Shaft or resolver rotor slipping

- Loose mounting bolts

Solutions:

- Check all mounting bolt torques

- Verify resolver rotor is secure on shaft

- Check for worn bearings or excessive shaft play

- Consider using threadlocker on mounting bolts

### Issue: Unable to Detect Back-EMF Voltage

Possible Causes:

- Rotating shaft too slowly

- Motor phases disconnected or open circuit

- Damaged motor windings

- Incorrect probe connections

Solutions:

- Increase shaft rotation speed (faster generates more voltage)

- Use more sensitive voltage range on meter/scope

- Verify probe connections:

  - Oscilloscope: Probe tip to Phase A, Ground clip to Phase B (or to Neutral if available)

  - DMM: Red probe to Phase A, Black probe to Phase B

- Check continuity of motor phases (with power OFF):

  - Measure resistance between Phase A and Phase B

  - Measure resistance between Phase B and Phase C

  - Measure resistance between Phase C and Phase A

  - All should read similar low resistance (typically 0.5-10Ω)

- Verify phase wires are properly connected to motor terminals

- Test motor winding resistance (should be low, typically <10Ω per phase)

## Appendix A: Alignment Accuracy Requirements

| Application Type | Target Accuracy | Acceptable Range | Impact of Poor Alignment |
| --- | --- | --- | --- |
| High-performance servo | ±1° electrical | ±0-2° electrical | Minimal - smooth operation |
| CNC machine tool | ±2° electrical | ±0-3° electrical | Slight torque ripple |
| General industrial | ±5° electrical | ±0-7° electrical | Noticeable at low speed |
| Pumps/fans | ±10° electrical | ±0-15° electrical | Reduced efficiency |

Note: Errors >15° electrical may cause:

- Significant torque ripple

- Wrong initial direction of rotation

- Overcurrent faults during startup

- Reduced motor efficiency

## Appendix B: Electrical vs Mechanical Degree Conversion

| Motor Poles | Pole Pairs | Mechanical Degrees | Electrical Degrees |
| --- | --- | --- | --- |
| 2 | 1 | 360° | 360° |
| 4 | 2 | 180° | 360° |
| 6 | 3 | 120° | 360° |
| 8 | 4 | 90° | 360° |
| 10 | 5 | 72° | 360° |
| 12 | 6 | 60° | 360° |

Formula: Electrical° = Mechanical° × Pole Pairs

Example: For an 8-pole motor (4 pole pairs):

- ±1° electrical = ±0.25° mechanical

- ±5° electrical = ±1.25° mechanical

## Appendix C: Reference Information

### Motor Information

- Motor model: ___________________________

- Number of poles: _______

- Number of pole pairs: _______

- Rated voltage: _______ V

- Rated current: _______ A

### Resolver Information

- Resolver model: ___________________________

- Transformation ratio: _______

- Excitation frequency: _______ kHz

- Excitation voltage: _______ V

### Resolver-to-Digital Converter Information

- Converter model: ___________________________

- Configured pole pairs: _______

- NM output logic level: _______ V

- Firmware version: _______

### Alignment Record

- Date of alignment: ___________________________

- Technician: ___________________________

- Electrical zero position used: _______ (E0-1, E0-2, etc.)

- Measured alignment accuracy: ±_______ electrical degrees

- Notes: ___________________________

## Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Name] | Initial release |

END OF PROCEDURE