Pressure test of modified camera port

Project: I2map

Engineer Francois Cazenave

12/21/21015

The modified camera port and modified titanium ring were assembled for pressure testing. A LVTD was placed inside the assembly. The purpose of the LVDT is to measure the displacement of the glass port during pressure testing. The range of the LVDT is -0.100” to +0.100”.  The LVDT was calibrated today in the cold room at 4oC.  0V output is -0.100” and 10V output is +0.100”.

An 8-pin, right angle, bulkhead connector is used to connect the LVDT. The pin out is as follows:

- Pin 1, black, Primary coil 1

- Pin 2, white, primary coil 2

- Pin 3, red, secondary coil 1

- Pin 4, green, secondary coil 2

The output of the signal conditioner is a 0 to 10VDC. (1/1000” = 0.05VDC)