Procedure for lens assembly:

All work to be done in clean, dry environment.

- Thoroughly clean all components.

- Place titanium ring on bench, dome side down.

- Insert o-ring in rear lens o-ring groove and insert rear lens.

- Secure rear lens with threaded retainer.

- Flip titanium ring

- Insert middle lens in titanium ring

- Insert lens spacer

- Insert double concave lens

- Secure lenses with threaded retainer

- Insert kapton ring in titanium ring

- Place back up ring around the dome port.

- Insert the dome and back up ring in the titanium ring. The back-up ring may need to be pushed to the bottom of the groove with a tool.

- Insert the o-ring in the groove, push it against the back up ring using a tool.

- Insert the Dome retainer. Screws scan be used to insert it evenly and slowly.

- Tighten screws

- Purge the inside of the lens assembly with clean dry air or nitrogen and plug the ports (TBD, see note below).

Some questions:

- What material do you recommend for the o-rings (and what hardness). We typically use buna-N, 70 durometer.

- What lubricant do you recommend for the o-rings (we typically use Dow Corning Molykote 55 o-ring grease)

- How would you insert the lens in the titanium ring? By hand or with a tool such as a vacuum lifter?

- Would you recommend pulling a slight vacuum on the lens assembly? It would hold the dome firmly in position and seat the o-rings. Also, it would prevent having a positive pressure differential, if there is an increase of temperature (for example during testing of the camera in the lab). If you recommend a