- Introduce ourselves 	2 minutes

- Present grand challenge (Kelly)	2 minutes

  - To better our understanding of the midwater ocean environment

  - Midwater time series continuation

    - Value of time series (O2 example)

    - Increases in efficiency are necessary to maintain unique dataset

    - Introduce vehicle (Investigations of imaging for midwater autonomous platforms)

- Must retain high quality visual imagery (Mark)	5 minutes

The challenge of working at great pressure

  - Solution in the ROV (full dome)

  - That doesn’t work in the AUV because of size. Solution: flat dome

    - Problem: Strength

    - Solutions

      - Redesigned dome support

        - Rigorous study of glass fracture mechanics

        - Material properties determined by destructive testing

        - The predicted stress and strain was calculated using a model, confirmed by pressure testing

        - Calculate expected rate of crack growth (glass ALWAYS has cracks!) over time. The results give a predicted failure rate over time for a given pressure.

        - Proof test

        - At the end of the service life the dome is either replaced OR it can be re-tested at the proof pressure and the predicted failure rate at the 100 hour service life are re-established.

        - The result of this work is a system with superior optical properties that has – to date – been reliable!

      - Show videos

- Another challenge in making the video time series more efficient comes with the data processing (Mark)	2 minutes

  - Currently, it takes 22 hours of human analyst time to work through the 11, 10 minute transects that make up the monthly time series

  - Danelle Cline has been working to develop a deep neural network to speed up and automate this process

  - This classifier is trained on a completely different domain (transfer learning) to learn to become a good feature extractor and doesn’t require pre-definition of specific features

  - Showing great promise – show example video

- Adding acoustics a new ‘sense’ to i2Map 	5 minutes

  - Matched acoustic and imaging data for ‘acoustic fingerprints’

  - ‘See’ much further from the vehicle

  - Extending the range of behavioral observations (tracking)

  - Observe behavioral responses to platform

    - Compare to ROV to bridge time series

- Wrap up	1 minute

  - The future - Together, these new approaches will be used to add nighttime surveys to the time series

  - Understanding the mesopelagic is critical and timely