/* Register CONFIG1 (0x2abfe)                               */

#define _CONFIG1(x) __attribute__((section("__CONFIG1.sec,code"))) int _CONFIG1 = (x);

/*

** Only one invocation of CONFIG1 should appear in a project,

** at the top of a C source file (outside of any function).

**

** The following constants can be used to set CONFIG1.

** Multiple options may be combined, as shown:

**

** _CONFIG1( OPT1_ON & OPT2_OFF & OPT3_PLL )

**

**   JTAG:

**     JTAGEN_OFF           Disabled

**     JTAGEN_ON            Enabled

**

**   Code Protect:

**     GCP_ON               Enabled

**     GCP_OFF              Disabled

**

**   Write Protect:

**     GWRP_ON              Enabled

**     GWRP_OFF             Disabled

**

**   Background Debugger:

**     BKBUG_ON             Enabled

**     BKBUG_OFF            Disabled

**

**   Clip-on Emulation mode:

**     COE_ON               Enabled

**     COE_OFF              Disabled

**

**   ICD pins select:

**     ICS_PGx3             EMUC/EMUD share PGC3/PGD3

**     ICS_PGx2             EMUC/EMUD share PGC2/PGD2

**     ICS_PGx1             EMUC/EMUD share PGC1/PGD1

**

**   Watchdog Timer:

**     FWDTEN_OFF           Disabled

**     FWDTEN_ON            Enabled

**

**   Windowed WDT:

**     WINDIS_ON            Enabled

**     WINDIS_OFF           Disabled

**

**   Watchdog prescaler:

**     FWPSA_PR32           1:32

**     FWPSA_PR128          1:128

**

**   Watchdog postscale:

**     WDTPS_PS1            1:1

**     WDTPS_PS2            1:2

**     WDTPS_PS4            1:4

**     WDTPS_PS8            1:8

**     WDTPS_PS16           1:16

**     WDTPS_PS32           1:32

**     WDTPS_PS64           1:64

**     WDTPS_PS128          1:128

**     WDTPS_PS256          1:256

**     WDTPS_PS512          1:512

**     WDTPS_PS1024         1:1,024

**     WDTPS_PS2048         1:2,048

**     WDTPS_PS4096         1:4,096

**     WDTPS_PS8192         1:8,192

**     WDTPS_PS16384        1:16,384

**     WDTPS_PS32768        1:32,768

**

*/

#define JTAGEN_OFF           0x3FFF

#define JTAGEN_ON            0x7FFF

#define GCP_ON               0x5FFF

#define GCP_OFF              0x7FFF

#define GWRP_ON              0x6FFF

#define GWRP_OFF             0x7FFF

#define BKBUG_ON             0x77FF

#define BKBUG_OFF            0x7FFF

#define COE_ON               0x7BFF

#define COE_OFF              0x7FFF

#define ICS_PGx3             0x7DFF

#define ICS_PGx2             0x7EFF

#define ICS_PGx1             0x7FFF

#define FWDTEN_OFF           0x7F7F

#define FWDTEN_ON            0x7FFF

#define WINDIS_ON            0x7FBF

#define WINDIS_OFF           0x7FFF

#define FWPSA_PR32           0x7FEF

#define FWPSA_PR128          0x7FFF

#define WDTPS_PS1            0x7FF0

#define WDTPS_PS2            0x7FF1

#define WDTPS_PS4            0x7FF2

#define WDTPS_PS8            0x7FF3

#define WDTPS_PS16           0x7FF4

#define WDTPS_PS32           0x7FF5

#define WDTPS_PS64           0x7FF6

#define WDTPS_PS128          0x7FF7

#define WDTPS_PS256          0x7FF8

#define WDTPS_PS512          0x7FF9

#define WDTPS_PS1024         0x7FFA

#define WDTPS_PS2048         0x7FFB

#define WDTPS_PS4096         0x7FFC

#define WDTPS_PS8192         0x7FFD

#define WDTPS_PS16384        0x7FFE

#define WDTPS_PS32768        0x7FFF

/* Register CONFIG2 (0x2abfc)                               */

#define _CONFIG2(x) __attribute__((section("__CONFIG2.sec,code"))) int _CONFIG2 = (x);

/*

** Only one invocation of CONFIG2 should appear in a project,

** at the top of a C source file (outside of any function).

**

** The following constants can be used to set CONFIG2.

** Multiple options may be combined, as shown:

**

** _CONFIG2( OPT1_ON & OPT2_OFF & OPT3_PLL )

**

**   Two Speed Start-up:

**     IESO_OFF             Disabled

**     IESO_ON              Enabled

**

**   Oscillator Selection:

**     FNOSC_FRC            Fast RC oscillator

**     FNOSC_FRCPLL         Fast RC oscillator w/ divide and PLL

**     FNOSC_PRI            Primary oscillator (XT, HS, EC)

**     FNOSC_PRIPLL         Primary oscillator (XT, HS, EC) w/ PLL

**     FNOSC_SOSC           Secondary oscillator

**     FNOSC_LPRC           Low power RC oscillator

**     FNOSC_FRCDIV         Fast RC oscillator with divide

**

**   Clock switching and clock monitor:

**     FCKSM_CSECME         Both enabled

**     FCKSM_CSECMD         Only clock switching enabled

**     FCKSM_CSDCMD         Both disabled

**

**   OSCO/RC15 function:

**     OSCIOFNC_ON          RC15

**     OSCIOFNC_OFF         OSCO or Fosc/2

**

**   RP Register Protection:

**     IOL1WAY_OFF          Unlimited Writes To RP Registers

**     IOL1WAY_ON           Write RP Registers Once

**

**   Oscillator Selection:

**     POSCMOD_EC           External clock

**     POSCMOD_XT           XT oscillator

**     POSCMOD_HS           HS oscillator

**     POSCMOD_NONE         Primary disabled

**

*/

#define IESO_OFF             0x7FFF

#define IESO_ON              0xFFFF

#define FNOSC_FRC            0xF8FF

#define FNOSC_FRCPLL         0xF9FF

#define FNOSC_PRI            0xFAFF

#define FNOSC_PRIPLL         0xFBFF

#define FNOSC_SOSC           0xFCFF

#define FNOSC_LPRC           0xFDFF

#define FNOSC_FRCDIV         0xFFFF

#define FCKSM_CSECME         0xFF3F

#define FCKSM_CSECMD         0xFF7F

#define FCKSM_CSDCMD         0xFFBF

#define OSCIOFNC_ON          0xFFDF

#define OSCIOFNC_OFF         0xFFFF

#define IOL1WAY_OFF          0xFFEF

#define IOL1WAY_ON           0xFFFF

#define POSCMOD_EC           0xFFFC

#define POSCMOD_XT           0xFFFD

#define POSCMOD_HS           0xFFFE

#define POSCMOD_NONE         0xFFFF

/* Register CONFIG3 (0x2abfa)                               */

#define _CONFIG3(x) __attribute__((section("__CONFIG3.sec,code"))) int _CONFIG3 = (x);

/*

** Only one invocation of CONFIG3 should appear in a project,

** at the top of a C source file (outside of any function).

**

** The following constants can be used to set CONFIG3.

** Multiple options may be combined, as shown:

**

** _CONFIG3( OPT1_ON & OPT2_OFF & OPT3_PLL )

**

**   Write Protection Location:

**     WPEND_WPSTARTMEM     Write Protect from page 0 to WPFP

**     WPEND_WPENDMEM       Write Protect from WPFP to the last page of memory

**

**   Write Protect Configuration Page:

**     WPCFG_WPCFGEN        Enabled

**     WPCFG_WPCFGDIS       Disabled

**

**   Write Protection Disable:

**     WPDIS_WPEN           Enabled

**     WPDIS_WPDIS          Disabled

**   Write Protection Flash Page:

**     WPFP_WPFP0           0

thru

**     WPFP_WPFP511         511

**

*/

#define WPEND_WPSTARTMEM     0x7FFF

#define WPEND_WPENDMEM       0xFFFF

#define WPCFG_WPCFGEN        0xBFFF

#define WPCFG_WPCFGDIS       0xFFFF

#define WPDIS_WPEN           0xDFFF

#define WPDIS_WPDIS          0xFFFF

#define WPFP_WPFP0           0xFE00

thru

#define WPFP_WPFP511         0xFFFF