# Medium Voltage Converter Board Engineering Change Orders

Chad Kecy

11-11-2013

# ECO A

Description

The heatsink temperature node is mislabeled.

Problem

Due to the heatsink temperature signal being named differently at two locations on the schematic resulted in an incorrect layout. The measured signal does not reach the ADC.

Figure One: ECO A, schematic page 1

Figure Two: ECO A, schematic page 2

Fix

Solder a blue wire from U20-1 to U4-6.

# ECO B

Description

The Matchport serial port interface is incorrect.

Problem

The Matchport uses TTL level serial signals. The serial transceiver, U54, is not needed.

Figure Three: ECO B, schematic page 6.

Fix

Remove U54, C99. C100, C101, C103, C104.

Add blues wires from: 	U54-12 to U54-13

U54-8 to U54-9

U54-14 to U54-11

U54-7 to U54-10

# ECO C

Description

Modification of 48V Delay Circuit

Problem

The timing of the 48V delay circuit was slightly off. The reference arm of the comparator U35 should be tied to the output of the mosfet U38, not 24V directly.

Figure Four: ECO C, schematic page 4

Fix

Remove the surface mount resistor R85 from the backside of the pcb. Add in a leaded resistor of the same value, 22.1kohms between U35-3 and C65+.

# ECO D

Description

The ground fault current measurement scaling is incorrect.

Problem

The final output of the ground fault circuit was scaled for a 3.0V ADC. The signal is actually being measured by a 5V ADC.

Figure Five: ECO D, schematic page 7

Fix

Remove R174, R175, U64, and C124

Place a zero ohm resistor across R174 pads

Jumper U64-1 to U64-3

# ECO E

Description

High Voltage Scaling Resistors Incorrect

Problem

The low values of R6 and R9 in parallel with RN3 were causing incorrect measurements to be made.

Figure Six: ECO E, schematic page 1

Fix

Change R6 to 100kohms and R9 to 475kohms.

# ECO F

Description

High Voltage Current Sense Circuit Incorrect

Problem

The HV current sense circuit as designed is not working as planned.

Fix

Remove R1, R4, R5, R18, R19, RN1, RN2, C3, and U3.

Solder a 20AWG shunt across R1 pads

Jumper U3-7 to U3-9

Figure Seven: ECO F, schematic page 1

# ECO G

Description

Red Panel Mount LED Circuit Incorrect

Problem

The low impedance of the integrated LED module is interfering with the the high voltage measurements from RN3.

Figure Eight: ECO G, schematic page 1

Fix

Remove R8 and R12.

# ECO H

Description

TO-3 Can, U51, Laid Out Incorrectly

Problem

The footprint for the LM317K STEEL part had Vadj and Vin swapped resulting in the incorrect output.

Figure Nine: ECO H, schematic page 5

Fix

Remove R121 from backside of pcb

Desolder U51.

Resolder U51 on back side of pcb (using a grey sili-pad underneath part).

Use a leaded 240 ohm resistor on top. Solder between Vout trace and R122 trace.

# ECO I

Description

The digital isolators (ADUM5401) are keeping the MISO line inactive.