| Name | Signal Type | Dir (w/r to BB) | BB Expan Signal | BB Expan Port | Active (Outputs) |
| --- | --- | --- | --- | --- | --- |
| NODE_1_OFF | Digital | Out | GPIO2_7 | P8-46 | Low Pulse |
| NODE_1_ON | Digital | Out | GPIO2_6 | P8-45 | Low Pulse |
| NODE_1_STATE | Digital | In | GPIO2_22 | P8-27 |  |
| NODE_1_CURR | Analog | In | AIN0 | P9-39 |  |
| NODE_2_OFF | Digital | Out | GPIO2_9 | P8-44 | Low Pulse |
| NODE_2_ON | Digital | Out | GPIO2_8 | P8-43 | Low Pulse |
| NODE_2_STATE | Digital | In | GPIO2_23 | P8-29 |  |
| NODE_2_CURR | Analog | In | AIN1 | P9-40 |  |
| NODE_3_OFF | Digital | Out | GPIO2_11 | P8-42 | High Pulse |
| NODE_3_ON | Digital | Out | GPIO2_10 | P8-41 | High Pulse |
| NODE_3_STATE | Digital | In | GPIO2_24 | P8-28 |  |
| NODE_3_CURR | Analog | In | AIN2 | P9-37 |  |
| NODE_4_OFF | Digital | Out | GPIO2_13 | P8-40 | High Pulse |
| NODE_4_ON | Digital | Out | GPIO2_12 | P8-39 | High Pulse |
| NODE_4_STATE | Digital | In | GPIO2_25 | P8-30 |  |
| NODE_4_CURR | Analog | In | AIN3 | P9-38 |  |
| UART_1_TTL_TXD | Digital | Out | UART1_TXD | P9-24 |  |
| UART_1_TTL_RXD | Digital | In | UART1_RXD | P9-26 |  |
| UART_2_TTL_TXD | Digital | Out | UART2_TXD | P9-21 |  |
| UART_2_TTL_RXD | Digital | In | UART2_RXD | P9-22 |  |
| UART_4_TTL_TXD | Digital | Out | UART4_TXD | P9-13 |  |
| UART_4_TTL_RXD | Digital | In | UART4_RXD | P9-11 |  |
| UART_5_TTL_TXD | Digital | Out | UART5_TXD | P8-37 |  |
| UART_5_TTL_RXD | Digital | In | UART5_RXD | P8-38 |  |
| 48V_RET_GF | Digital | Out | GPIO1_4 | P8-23 | High SS |
| 48V_GF | Digital | Out | GPIO1_0 | P8-25 | High SS |
| TEST_RET_GF | Digital | Out | GPIO1_1 | P8-24 | High SS |
| TEST_GF | Digital | Out | GPIO1_29 | P8-26 | High SS |
| GF_CURR | Analog | In | AIN4 | P9-33 |  |
| H2O_ALARM | Digital | In | GPIO1_5 | P8-22 |  |
| 48V_MON | Analog | In | AIN5 | P9-36 |  |
| ENVR_ALARM | Digital | In | GPIO1_31 | P8-20 |  |
| EEPROM_WP | Digital | Out | GPIO1_30 | P8-21 | High SS |

Gateway Node BeagleBone Signal Definitions