Configuration of P8

| P8 Pin | Proc | BB Signal | GN Signal | Type | BB Mode |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | GND | GND | Gnd | N/A |
| 2 | GND | GND | GND | Gnd | N/A |
| 3 | R9 | GPIO1_6 | NODE_4_OFF | Digital Out | 7 |
| 4 | T9 | GPIO1_7 | NODE_4_ON | Digital Out | 7 |
| 5 | R8 | GPIO1_2 | NODE_3_OFF | Digital Out | 7 |
| 6 | T8 | GPIO1_3 | ** Reserved ** | Digital Out | 7 |
| 7 | R7 | TIMER4 | BB_R7 | Unused | 2 |
| 8 | T7 | TIMER7 | BB_T7 | Unused | 2 |
| 9 | T6 | TIMER5 | BB_T6 | Unused | 2 |
| 10 | U6 | TIMER6 | BB_U6 | Unused | 2 |
| 11 | R12 | GPIO1_13 | NODE_2_OFF | Digital Out | 7 |
| 12 | T12 | GPIO1_12 | NODE_2_ON | Digital Out | 7 |
| 13 | T10 | EHRPWM2B | BB_T10 | Unused | 7 (gpio0_23) |
| 14 | T11 | GPIO0_26 | EEPROM_WP1 | Digital Out |  |
| 15 | U13 | GPIO1_15 | NODE_1_OFF | Digital Out | 7 |
| 16 | V13 | GPIO1_14 | NODE_1_ON | Digital Out | 7 |
| 17 | U12 | GPIO0_27 | EEPROM_WP2 | Digital Out | 7 |
| 18 | V12 | GPIO2_1 | EEPROM_WP3 | Digital Out | 7 |
| 19 | U10 | EHRPWM2A | BB_U10 | Unused | 7(gpio0_22) |
| 20 | V9 | GPIO1_31 | ENVR_ALARM | Digital In | 7 |
| 21 | U9 | GPIO1_30 | EEPROM_WP4 | Digital Out | 7 |
| 22 | V8 | GPIO1_5 | H2O_ALARM | Digital In | 7 |
| 23 | U8 | GPIO1_4 | 48V_RET_GF | Digital Out | 7 |
| 24 | V7 | GPIO1_1 | TEST_RET_GF | Digital Out | 7 |
| 25 | U7 | GPIO1_0 | 48V_GF | Digital Out | 7 |
| 26 | V6 | GPIO1_29 | TEST_GF | Digital Out | 7 |
| 27 | U5 | GPIO2_22 | NODE_1_STATE | Digital In | 7 |
| 28 | V5 | GPIO2_24 | NODE_3_STATE | Digital In | 7 |
| 29 | R5 | GPIO2_23 | NODE_2_STATE | Digital In | 7 |
| 30 | R6 | GPIO2_25 | NODE_4_STATE | Digital In | 7 |
| 31 | V4 | UART5_CTSN | BB_V4 | BT/Unused | 7 |
| 32 | T5 | UART5_RTSN | BB_T5 | BT/Unused | 7 |
| 33 | V3 | UART4_RTSN | BB_V3 | BT/Unused | 7 |
| 34 | U4 | UART3_RTSN | BB_U4 | BT/Unused | 7 |
| 35 | V2 | UART4_CTSN | BB_V2 | BT/Unused | 7 |
| 36 | U3 | UART3_CTSN | BB_U3 | BT/Unused | 7 |
| 37 | U1 | UART5_TXD | UART5_TTL_TXD | BT/Unused | 4 |
| 38 | U2 | UART5_RXD | UART5_TTL_RXD | BT/Unused | 4 |
| 39 | T3 | GPIO2_12 | BB_T3 | BT/Unused | 7 |
| 40 | T4 | GPIO2_13 | BB_T4 | BT/Unused | 7 |
| 41 | T1 | GPIO2_10 | BB_T1 | BT/Unused | 7 |
| 42 | T2 | GPIO2_11 | BB_T2 | BT/Unused | 7 |
| 43 | R3 | GPIO2_8 | BB_R3 | BT/Unused | 7 |
| 44 | R4 | GPIO2_9 | BB_R4 | BT/Unused | 7 |
| 45 | R1 | GPIO2_6 | BB_R1 | BT/Unused | 7 |
| 46 | R2 | GPIO2_7 | BB_R2 | BT/Unused | 7 |

Configuration of P9

| P9 Pin | Proc | BB Signal | GN Signal | Type | BB Mode |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | GND | GND | Gnd | N/A |
| 2 | GND | GND | GND | Gnd | N/A |
| 3 | 3.3VEXP | 3.3VEXP | 3.3V | Power | N/A |
| 4 | 3.3VEXP | 3.3VEXP | 3.3V | Power | N/A |
| 5 | VDD_5V | VDD_5V | 5V_BB | Power | N/A |
| 6 | VDD_5V | VDD_5V | 5V_BB | Power | N/A |
| 7 | SYS_5V | SYS_5V | 5V | Power | N/A |
| 8 | SYS_5V | SYS_5V | 5V | Power | N/A |
| 9 |  | PWR_BUT* | PWR_BUT* | Special | N/A |
| 10 | A10 | SYS_RESETn | SYS_RESETn | Special | 0 |
| 11 | T17 | UART4_RXD | UART4_TTL_RXD | Serial | 6 |
| 12 | U18 | GPIO1_28 | NODE_3_ON | Digital Out | 7 |
| 13 | U17 | UART4_TXD | UART4_TTL_TXD | Serial | 6 |
| 14 | U14 | EHRPWM1A | BB_U14 | Unused | 7(gpio1_18) |
| 15 | R13 | GPIO1_16 | BB_R13 | Unused | 7 |
| 16 | T14 | EHRPWM1B | BB_T14 | Unused | 7(gpio1_19) |
| 17 | A16 | I2C1_SCL | BB_A16 | Unused | 7(gpio0_5) |
| 18 | B16 | I2C1_SDA | BB_B16 | Unused | 7(gpio0_4) |
| 19 | D17 | I2C2_SCL | I2C2_SCL | I2C Bus | 3 |
| 20 | D18 | I2C2_SDA | I2C2_SDA | I2C Bus | 3 |
| 21 | B17 | UART2_TXD | UART2_TTL_TXD | Serial | 1 |
| 22 | A17 | UART2_RXD | UART2_TTL_RXD | Serial | 1 |
| 23 | V14 | GPIO1_17 | BB_V14 | Unused | 7 |
| 24 | D15 | UART1_TXD | UART1_TTL_TXD | Serial | 0 |
| 25 | A14 | GPIO3_21 | BB_A14 | Unused | 7 |
| 26 | D16 | UART1_RXD | UART1_TTL_RXD | Serial | 0 |
| 27 | C13 | GPIO3_19 | BB_C13 | Unused | 7 |
| 28 | C12 | SPI1_CS0 | SPI1_CS0 | SPI Bus | 3 |
| 29 | B13 | SPI1_DO | SPI1_MOSI | SPI Bus | 3 |
| 30 | D12 | SPI1_DI | SPI1_MISO | SPI Bus | 3 |
| 31 | A13 | SPI1_SCLK | SPI1_SCLK | SPI Bus | 3 |
| 32 | 1.8VREF | 1.8V_REF | 1.8V_REF | Volt Ref | N/A |
| 33 | C8 | AIN4 | GF_CURR | Analog In | N/A |
| 34 | GNDADC | GNDA_ADC | GNDA_ADC | Analog Gnd | N/A |
| 35 | A5 | AIN6 | 48V_CURR | Analog In | N/A |
| 36 | A5 | AIN5 | 48V_VOLT | Analog In | N/A |
| 37 | B7 | AIN2 | NODE_3_CURR | Analog In | N/A |
| 38 | A7 | AIN3 | NODE_4_CURR | Analog In | N/A |
| 39 | B6 | AIN0 | NODE_1_CURR | Analog In | N/A |
| 40 | C7 | AIN1 | NODE_2_CURR | Analog In | N/A |
| 41 | D14 | CLKOUT2 | BB_D14 | Unused | 3 |
| 42 | C18 | GPIO0_7 | SPI1_CS1 | SPI Bus | 2 |
| 43 | GND | GND | GND | Gnd | N/A |
| 44 | GND | GND | GND | Gnd | N/A |
| 45 | GND | GND | GND | Gnd | N/A |
| 46 | GND | GND | GND | Gnd | N/A |