| Chip | Pin | Addr | Dir | Act | Val | Signal |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 7 | 71 | Out | Low | High | NODE_1_OFF |
| 2 | 6 | 70 | Out | Low | High | NODE_1_ON |
| 2 | 22 | 86 | In | Low | N/A | NODE_1_STATE |
| 2 | 9 | 73 | Out | Low | High | NODE_2_OFF |
| 2 | 8 | 72 | Out | Low | High | NODE_2_ON |
| 2 | 23 | 87 | In | Low | N/A | NODE_2_STATE |
| 2 | 11 | 75 | Out | High | Low | NODE_3_OFF |
| 2 | 10 | 74 | Out | High | Low | NODE_3_ON |
| 2 | 24 | 88 | In | Low | N/A | NODE_3_STATE |
| 2 | 13 | 77 | Out | High | Low | NODE_4_OFF |
| 2 | 12 | 76 | Out | High | Low | NODE_4_ON |
| 2 | 25 | 89 | In | Low | N/A | NODE_4_STATE |
| 1 | 4 | 36 | Out | High | Low | 48V_RET_GF |
| 1 | 0 | 32 | Out | High | Low | 48_GF |
| 1 | 1 | 33 | Out | Low | Low | TEST_RET_GF |
| 1 | 28 | 60 | Out | Low | Low | GPIO1_28 |
| 1 | 29 | 61 | Out | Low | Low | TEST_GF |
| 1 | 5 | 37 | In | Low | N/A | H2O_ALARM |
| 1 | 31 | 63 | In | Low | N/A | ENVR_ALARM |
| 1 | 30 | 62 | Out | High | Low | EEPROM_WP |

Original chip/pin assignments

| Chip | Pin | Addr | Dir | Act | Val | Signal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 14 |  | Out | Low | High | NODE_1_OFF |
| 1 | 15 |  | Out | Low | High | NODE_1_ON |
| 2 | 22 | 86 | In | Low | N/A | NODE_1_STATE |
| 1 | 12 |  | Out | Low | High | NODE_2_OFF |
| 1 | 13 |  | Out | Low | High | NODE_2_ON |
| 2 | 23 | 87 | In | Low | N/A | NODE_2_STATE |
| 1 | 3 |  | Out | High | Low | NODE_3_OFF |
| 1 | 2 |  | Out | High | Low | NODE_3_ON |
| 2 | 24 | 88 | In | Low | N/A | NODE_3_STATE |
| 1 | 7 |  | Out | High | Low | NODE_4_OFF |
| 1 | 6 |  | Out | High | Low | NODE_4_ON |
| 2 | 25 | 89 | In | Low | N/A | NODE_4_STATE |
| 1 | 4 | 36 | Out | High | Low | 48V_RET_GF |
| 1 | 0 | 32 | Out | High | Low | 48_GF |
| 1 | 1 | 33 | Out | Low | Low | TEST_RET_GF |
| 1 | 28 | 60 | Out | Low | Low | GPIO1_28 |
| 1 | 29 | 61 | Out | Low | Low | TEST_GF |
| 1 | 5 | 37 | In | Low | N/A | H2O_ALARM |
| 1 | 31 | 63 | In | Low | N/A | ENVR_ALARM |
| 1 | 30 | 62 | Out | High | Low | EEPROM_WP |

Modified chip/pin assignments